3GPP TSG-RAN WG2 Meeting #123bis	R2-2310063

Xiamen, China, October 9th – 13th, 2023

Agenda item:	7.20.2

Source:	Samsung

Title:	Open issues on two TAs for multi-DCI multi-TRP

WID/SID:	NR_MIMO_evo_DL_UL-Core

Document for:	Discussion and Decision

# Introduction

In Rel-18 discussions on MIMO Evolution for Downlink and Uplink, RAN1 has agreed to support multi-DCI multi-TRP operation with two TAs. In this document, we continue discussion on open issues of two TAs for multi-DCI multi-TRP.

# Discussion

## TAG modelling and UE behaviour

In the last meeting, the following working assumption was made regarding the TAG modelling and UE behavior.

Working assumption:

We will use the 2-PTAG model, i.e., both TAGs of SpCell are PTAGs;

    - When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired,

    - when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired.

As specified in TS 38.321, the actions to be applied at TAT expiry are listed as follows in legacy operation.

- not perform any uplink transmission except the Random Access Preamble and MSGA transmission;

- flush all HARQ buffers;

- notify RRC to release PUCCH, if configured;

- notify RRC to release SRS, if configured;

- clear any configured downlink assignments and configured uplink grants;

- clear any PUSCH resource for semi-persistent CSI reporting;

- maintain NTA (defined in TS 38.211 [8]) of this TAG;

- consider all running timeAlignmentTimers as expired.

With 2 PTAGs in SpCell, only when both PTAGs are expired, legacy operation applies. If one PTAG is expired and the other PTAG is still valid, the DL and UL data transmission associated to the valid PTAG should not be impacted so that operation on serving cells are less impacted by the interruption due to TAT expiry. This is the advantage to be taken by supporting 2 TAs in multi-TRP operation.

One concern raised for 2 PTAGs is if there are 2PTAGs, whether UE is allowed to initiate RA towards both PTAGs or one PTAG is prioritized for UE initiated RA. Our understanding is that RA is initiated by UE towards the SpCell, but not triggered due to a specific PTAG expiry.

As in legacy operation, if UL timing is not aligned and there is UL data arrived, when there is no valid PUCCH resource for SR for UL data transmission, UE will initiate RA. With two TAs, if UL timing of one or both TAGs are not aligned and there is UL data arrived, UE will initiate RA when there is no valid PUCCH resource for SR for UL data transmission. Once RA is performed, if two TAGs are configured, NW can indicate one of the two TAGs to which the TA command in the RAR is applied.

Proposal 1: Confirm the Working Assumption as an agreement:

We will use the 2-PTAG model, i.e., both TAGs of SpCell are PTAGs;

- When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired,

- when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired.

Regarding the FFSs for the case that the TAT of one TAG expires while the TAT for the other TAG is still running, UL transmissions using TCI states of the TRP with TAT expired should be stopped, while DL receptions on the two TRPs and UL transmissions on the TRP with TAT running are not impacted. HARQ retransmissions can be scheduled using TCI states of the TRP with TAT running, thus, the HARQ buffer should not be flushed.

Proposal 2: With 2 TAGs, when the TAT for a TAG is expired, while the TAT for the other TAG is running, HARQ buffers are not flushed.

For actions 3-6, as in legacy operation, DL/UL resource in time and frequency domain is configured per BWP. For multi-DCI multi-TRP operation, the DL/UL resource for different channels and signals is used by applying the TCI state(s) specified to coreset pool index. The unified TCI state to be applied for different channels and signals for multi-DCI multi-TRP operation are shown in the tables below.

As RAN1 agreed, each joint/UL TCI state is configured with a TAG, thus, UL resource is associated to TAG via TCI state selection. When the TAT of a TAG is expired while the other TAT is running, DL reception is not impacted for either TAG, so configured downlink assignment should not be released; the UL resource scheduled with TCI state(s) that are associated to the TAG with TAT running should not be released. The remaining issue is whether the UL resource that is only associated to TCI states with TAT expired should be released or not. We think there is no need to release the configured UL resources for a serving cell if one TAT for the serving cell is still running. As the TCI state(s) assigned for a configured grant is dynamically indicated by MAC CE and DCI, it is possible that TCI state(s) associated to a TAG with the running TAT are reassigned to a configured UL resource. UE can stop UL transmission on the configured UL resource that is only associated to TCI states with TAT expired and perform UL transmission on the configured UL resource once TCI states with TAT running are indicated or once the expired TAT is recovered.

Proposal 3: With 2 TAGs, when one TAT for a TAG expired (PTAG in 2-PTAG model or STAG), while the other TAT for the other TAG running, do not release PUCCH, SRS, configured downlink assignment, configured UL grant, or PUSCH for SP CSI report, stop UL transmission on the configured UL resource for PUCCH and PUSCH that is only associated to TCI states with TAT expired.

## Impact on RACH

RAN1 has agreed to support CFRA by PDCCH order to obtain TAs for multi-DCI multi-TRP operation for both intra-cell and inter-cell cases.

### Intra-cell PDCCH order CFRA

For intra-cell PDCCH order CFRA, RAN1 has informed the following agreement on TAG indication in RAR in LS R1-2308674.

- For R18 MIMO WI, RAN1 made the following agreement in RAN1#114 related to MAC CE. RAN1 respectfully asks RAN2 to take this into account in their future work. Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement and PDCCH order CFRA, indicate a representation of the TAG ID with 1 bit (either the first TAG ID or the second TAG ID for the serving cell) as part of TA command in RAR Note: For intra-cell multi-DCI based Multi-TRP operation, only a single NTA,offset is configured.

Proposal 4: With 2 TAGs, for intra-cell PDCCH order CFRA, confirm RAN1 agreement that the R bit in RAR is used for TAG indication.

Since the R bit can only indicate the first or the second TAG of a serving cell, not the TAG ID value, we have to specify the legacy tag-Id (i.e., tag 0) is the first TAG ID, and the new tag-Id2 is the second TAG ID in RRC.

Proposal 5: For a SpCell configured with two TAGs, specify in RRC the legacy tag-Id (i.e., tag 0) is the first TAG ID, and the new tag-Id2 is the second TAG ID.

### Inter-cell PDCCH order CFRA

For inter-cell PDCCH order CFRA, RACH configuration for RACH towards additional PCIs is needed. Based on the following RAN1 agreements,

- one additional PRACH configuration for each additional PCI is supported for RACH triggered by PDCCH order;

- support indication of which PRACH configuration to be used in the RACH procedure in the PDCCH order

- PDCCH order can be received from one TRP triggering CFRA PRACH towards another TRP;

Agreement RAN1-112 Confirm the following working assumption: For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, one additional PRACH configuration is supported for each configured additional PCI - the additional PRACH configuration is used in a RACH procedure triggered by a PDCCH order for the corresponding configured additional PCI Agreement RAN1-112bis For intercell multi-DCI based Multi-TRP operation with two TA enhancement, support indication of which PRACH configuration to be used in the RACH procedure in the PDCCH order. - FFS: Whether additionalPCI or a generic identifier is indicated in PDCCH order - FFS: The detail of the indication in PDCCH order in terms of whether to support PRACH triggered for inactive additionalPCI. Agreement RAN1-112bis For multi-DCI based inter-cell multi-TRP and intra-cell multi-TRP operation with two TAGs configured in a CC, for a CFRA based PDCCH order from one TRP triggering PRACH towards another TRP, study whether and, if needed, how to determine the transmit power of the triggered PRACH preamble. Agreement RAN1-114 For inter-cell multi-DCI based Multi-TRP operation with two TA enhancement, support indication of additionalPCI in the PDCCH order • as baseline capability: support PRACH triggering towards servingCell PCI and or active additionalPCI.

The same issue here for inter-cell PDCCH order CFRA is that to which TAG the TA in RAR should be applied. One way is to impose the restriction that the second TAG is always applied for CFRA towards the additionalPCI. However, the simpler way is to use the R bit in RAR for TAG indication since this is used anyway for intra-cell case.

Proposal 6: With 2 TAGs, for inter-cell PDCCH order CFRA, the R bit in RAR is used for TAG indication.

Regarding the additional PRACH configuration for the additional PCI, we think the RACH-ConfigCommon can be configured since the CFRA resource (e.g., preamble index, SSB index, etc.) is indicated in PDCCH order. Furthermore, the additional RACH configuration is used only for inter-cell PDCCH order RACH and should not be used for UE initiated RACH. UE initiated RACH is performed as legacy towards the SpCell using legacy RACH configuration. This also aligns with our previous agreement that do not support per-TRP UE initiated RACH.

Proposal 7: Configure random access resource configuration (i.e. RACH-ConfigCommon) for each additional PCI to be used only for inter-cell PDCCH order CFRA.

### UE initiated RACH

For UE initiated CBRA, we have agreed in the last meeting that the baseline is to use R bit in RAR is used for TAG indication.

The following is taken as baseline (for intra-cell case): for CBRA, we reuse the mechanism agreed for CFRA case, i.e. use the RA RAR to indicate the TAG.

However, when UE entering RRC_CONNECTED by RRC setup/resume and performing CBRA, the network cannot distinguish legacy UE and Rel-18 UE supporting/configured with 2 TA before contention resolution, and the UE may have not been configured with 2 TAG before receiving RRC setup/resume message. In this case, for 4-step CBRA the network should indicate the legacy TAG ID in MAC RAR. For 2-step CBRA, the successRAR is included in MSGB in initial access. For the same reason, the TAG indication in successRAR may not be needed.

Proposal 8: In 4-step CBRA, network indicates the first TAG ID, i.e., the legacy TAG ID, by the R bit in RAR.

Proposal 9: Discuss whether TAG indication in successRAR in MSGB is needed or not.

In 2-step CFRA for Handover, C-RNTI MAC CE is included in MSGA and an absolute TAC MAC CE is included in MSGB. In this case, the network can identify UE by C-RNTI included in MSGA. If two TAGs are configured, TAG indication may be needed in absolute TAC MAC CE. Alternatively, we can specify the absolute TAC is always applied to the first TAG ID, i.e., the legacy TAG ID. Furthermore, in the UE feature list provided by RAN1 [4], TAG ID indication in Absolute TAC MAC CE is an optional feature with capability signaling. If UE does not support this capability, we need to specify to which TAG the absolute TA is applied.

Proposal 10: Discuss for absolute TAC MAC CE, if two TAGs are configured,

Option 1: TAG indication is included in the MAC CE

Option 2: The TA is applied to the first TAG ID, i.e., tag 0.

Option 3: Option 1 or Option 2 depending on UE capability (i.e., if UE does not support TAG indication in MAC CE the TA is applied to the first TAG ID, i.e., tag 0).

Proposal 11: If TAG indication is introduced in absolute TAC MAC CE, discuss

Option 1: use 2-bit TAG ID

Option 2: use 1-bit TAG indication to indicate 1st/2nd TAG ID.

Furthermore, TA is also maintained in RRC_INACTIVE for SDT and SRS in Positioning. We need to clarify that 2 TA in multi-DCI multi-TRP operation is only applicable in RRC_CONNECTED so that there is no impact to TA/TAG maintenance in RRC_INACTIVE.

Proposal 12: Rel-18 multi-DCI multi-TRP operation with 2 TA is only applied in RRC_CONNECTED.

## Maximum transmission time difference

In TS 38.133 clause 7.5, maximum transmission timing difference (MTTD) is defined for DC between PCell and PSCell, and for CA between different carriers.

A UE shall be capable of handling a relative transmission timing difference between subframe timing boundary of E-UTRA PCell and the closest slot timing boundary of PSCell to be aggregated for EN-DC operation. A UE shall be capable of handling a relative transmission timing difference among the closest slot timing boundaries of different carriers in FR1 and/or FR2-1 to be aggregated in NR carrier aggregation. A UE shall be capable of handling a relative transmission timing difference among the closest subframe timing boundaries of different carriers to be aggregated in FR1 and FR2-2 NR inter-band carrier aggregation. A UE shall be capable of handling a relative transmission timing difference between slot timing boundary of PCell and subframe timing boundary of E-UTRA PSCell to be aggregated for NE-DC operation. A UE shall be capable of handling a relative transmission timing difference between slot timing boundaries of PCell in FR1 or FR2-1 and the closest slot timing boundary of PSCell in FR1 or FR2-1 to be aggregated in NR DC operation. A UE shall be capable of handling a relative transmission timing difference between subframe timing boundaries of PCell in FR1 and the closest subframe timing boundary of PSCell in FR2-2 to be aggregated in NR DC operation.

In TS 38.321 clause 5.2, the relevant restriction on TAT expiry is specified as follows.

When the MAC entity stops uplink transmissions for an SCell due to the fact that the maximum uplink transmission timing difference between TAGs of the MAC entity or the maximum uplink transmission timing difference between TAGs of any MAC entity of the UE is exceeded, the MAC entity considers the timeAlignmentTimer associated with the SCell as expired.

This means if the MTTD between TAGs is exceeded, the TAT of the corresponding SCell is considered expired, the TAT of PTAG is not considered as expired.

For multi-DCI multi-TRP operation, RAN4 has defined new MTTD values between multiple TRPs that are applicable to both intra-cell and inter-cell cases [5]. Correspondingly, we need to discuss when/which TAG(s) is considered as expired if MTTD between TRPs is exceeded. The MTTD could be exceeded between TRPs associated to the same cell or associated to different cells/MAC entities. The following cases have to be considered.

Case 1: the MTTD between two STAGs is exceeded

Case 2: the MTTD between a PTAG and a STAG is exceeded

Case 3: the MTTD between two PTAGs is exceeded

For case 1, we can apply the principle in legacy operation that TAT of SCell is considered to be expired, so the TATs of both STAGs are considered as expired.

For case 2, similarly, the TAT of the STAG is considered as expired.

Proposal 13: If the MTTD between two STAGs or between a STAG and a PTAG is exceeded, the TAT of the STAG is considered as expired, UE stops UL transmissions using unified TCI state(s) associated to corresponding STAG.

For case 3, we can follow the legacy principle that if two PTAGs are from different MAC entities, the TAT of PTAG is not considered as expired. However, when the MTTD between two PTAGs from the same SpCell is exceeded, the question is whether any PTAG is considered as expired.

Proposal 14: If the MTTD between PTAGs from different SpCells is exceeded, do not consider TAT expired.

Proposal 15: If the MTTD between PTAGs from the same SpCell is exceeded, discuss whether any PTAG is considered as expired.

# Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: Confirm the Working Assumption as an agreement:

We will use the 2-PTAG model, i.e., both TAGs of SpCell are PTAGs;

- When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired,

- when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired.

Proposal 2: With 2 TAGs, when the TAT for a TAG is expired, while the TAT for the other TAG is running, HARQ buffers are not flushed.

Proposal 3: With 2 TAGs, when one TAT for a TAG expired (PTAG in 2-PTAG model or STAG), while the other TAT for the other TAG running, do not release PUCCH, SRS, configured downlink assignment, configured UL grant, or PUSCH for SP CSI report, stop UL transmission on the configured UL resource for PUCCH and PUSCH that is only associated to TCI states with TAT expired.

Proposal 4: With 2 TAGs, for intra-cell PDCCH order CFRA, confirm RAN1 agreement that the R bit in RAR is used for TAG indication.

Proposal 5: For a SpCell configured with two TAGs, specify in RRC the legacy tag-Id (i.e., tag 0) is the first TAG ID, and the new tag-Id2 is the second TAG ID.

Proposal 6: With 2 TAGs, for inter-cell PDCCH order CFRA, the R bit in RAR is used for TAG indication.

Proposal 7: Configure random access resource configuration (i.e. RACH-ConfigCommon) for each additional PCI to be used only for inter-cell PDCCH order CFRA.

Proposal 8: In 4-step CBRA, network indicates the first TAG ID, i.e., the legacy TAG ID, by the R bit in RAR.

Proposal 9: Discuss whether TAG indication in successRAR in MSGB is needed or not.

Proposal 10: Discuss for absolute TAC MAC CE, if two TAGs are configured,

Option 1: TAG indication is included in the MAC CE

Option 2: The TA is applied to the first TAG ID, i.e., tag 0.

Option 3: Option 1 or Option 2 depending on UE capability (i.e., if UE does not support TAG indication in MAC CE the TA is applied to the first TAG ID, i.e., tag 0).

Proposal 11: If TAG indication is introduced in absolute TAC MAC CE, discuss

Option 1: use 2-bit TAG ID

Option 2: use 1-bit TAG indication to indicate 1st/2nd TAG ID.

Proposal 12: Rel-18 multi-DCI multi-TRP operation with 2 TA is only applied in RRC_CONNECTED.

Proposal 13: If the MTTD between two STAGs or between a STAG and a PTAG is exceeded, the TAT of the STAG is considered as expired, UE stops UL transmissions using unified TCI state(s) associated to corresponding STAG.

Proposal 14: If the MTTD between PTAGs from different SpCells is exceeded, do not consider TAT expired.

Proposal 15: If the MTTD between PTAGs from the same SpCell is exceeded, discuss whether any PTAG is considered as expired.

# Reference

[1] RP-213598 WID: MIMO Evolution for Downlink and Uplink

[2] R1-2308674 LS on Rel-18 higher-layers parameter list

[3] R1-2308672 Consolidated higher layers parameter list for Rel-18

[4] R1-2308521 Updated RAN1 UE features list for Rel-18 NR after RAN1#114

[5] R4-2217278 WF on MRTD/MTTD requirement for multi-TRPs with 2 TAs