3GPP TSG RAN WG2 #123bis												 R2-2310070

Xiamen, China, October 9th – 13th, 2023

Agenda Item:	7.8.3

Source: 	NEC

Title:	Remaining Issues on Flight Path Reporting

Document for:	Discussion

- Introduction

In RAN2 123 the below agreements were made [1]:

- No delta signalling for updated flight path will be supported - Confirm that same indication is used for both initial and updated flightpath plan - If any waypoint/timestamp has changed more than the configured threshold the UE triggers flightpath update indication. - When both distance and time threshold are configured, the flightpath update can be triggered if either of those conditions are met. No prohibit timer will be introduced.

This contribution provides our considerations on flight path information and reporting.

- Discussion

Based on current agreements, the same indication is used for both initial and updated flightpath plan and under following three cases, the flightpath availability notification should be provided to the network.

- UE had not previously provided a flightpath information, can be considered as an initial flightpath

- At least one waypoint changed more than the configured threshold

- At least one timestamp changed more than the configured threshold

Considering a UAV UE performed handover from the first gNB to the second gNB, the UAV UE had not previously provided a flightpath information in the second gNB, so based on current agreements, UAV UE should provide flightpath availability if flightpath information is available. However, the second network may already have the flightpath of the UAV UE via Xn interface. And the initial flightpath plan indicated by the UAV UE may be the same as the one which the second gNB got from the first gNB.

The initial flightpath plan from the UAV UE may be the same as the one which gNB got from the source gNB.

RAN2 to clarify whether UE should provide initial flightpath availability after handover if the gNB has already got flightpath from the source gNB and the flightpath did not change since last reporting in source gNB.

A simple method can be used to reduce initial availability notifications: If UAV UE can be indicated by the network that a flightpath has already be provided by source gNB, UAV UE do not need to provide an initial flightpath availability notification after handover and can further provide update notification base on the latest flightpath reported in the source gNB.

If indicated by the network that a flightpath has already be provided by source gNB, UAV UE do not need to provide an initial flightpath availability notification after handover and can provide update notification base on the latest flightpath reported in the source gNB.

- Conclusion

Observation 1:	The initial flightpath plan from the UAV UE may be the same as the one which gNB got from the source gNB.

Proposal 1:	RAN2 to clarify whether UE should provide initial flightpath availability after handover if the gNB has already got flightpath from the source gNB and the flightpath did not change since last reporting in source gNB.

Proposal 2:	If indicated by the network that a flightpath has already be provided by source gNB, UAV UE do not need to provide an initial flightpath availability notification after handover and can provide update notification base on the latest flightpath reported in the source gNB.

- References

[1] R2-23xxxxx, Report of 3GPP TSG RAN WG2 meeting #123, ETSI MCC.