3GPP TSG-RAN WG2 Meeting #123bis                                                   R2-2310078

Xiamen, China, 9th – 13th October, 2023

Source:              CATT

Title:	             Enhancements on SSB-less SCell operation

Agenda Item:	7.3.3

Document for:	Discussion and Decision

Introduction

The NES WID [1] identifies the objectives for SSB-less operation as follows:

- Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2]

In RAN2#122 meeting, the following agreements are achieved:

If RAN4 conclude SSB-less SCell for inter-band CA for FR1 and co-located cells is feasible, the signaling of intra-band CA (including RRC change on timing of SSB-less SCell and capability signaling) can be considered as its baseline. Whether other new signaling is required depends on RAN4 input. If RAN4 concludes it is feasible, RAN2 can further work on at least the following specification impacts: - RRC configuration of the frequency of the SSB to be used for the UE to obtain the timing reference for the inter-band SCell. - UE capability reporting to indicate whether UE supports configuration of inter-band SCell that does not transmit SS/PBCH block.

In this contribution, we will give our views on the specification impact to UE capability and SSB-less SCell operation in the scenario 1.

Discussion

  - UE capability

| csi-RS-MeasSCellWithoutSSB Defines whether the UE can perform CSI-RSRP and CSI-RSRQ measurement as specified in TS 38.215 [13], where CSI-RS resource is configured for a cell that does not transmit SS/PBCH block. A UE that supports this feature shall also support scellWithoutSSB. | FS | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| scellWithoutSSB Defines whether the UE supports configuration of SCell that does not transmit SS/PBCH block. This is conditionally mandatory with capability signalling for intra-band CA but not supported for inter-band CA. | FS | CY | N/A | N/A |

Based on above UE capability description in TS 38.306, the legacy csi-RS-MeasSCellWithoutSSB and scellWithoutSSB are used to indicate that UE support the CSI-RS measurement on SSB-less SCell for intra-band CA but not for inter-band CA. For SSB-less SCell with inter-band CA on FR1 and co-located cells, as it is not sure whether extra UE capability to support measurement is needed, so it may be RAN4 scope to determine whether to extend legacy IE or introduce a new UE capability. Hence, it is suggested to wait for RAN4 progress before we make any decision on UE capability for TRS measurement.

Proposal 1: Wait for RAN4 progress on the UE capability for TRS measurement on SSB-less SCell with inter-band CA for FR1 and co-located cells.

  - SSB-less SCell operation

In RAN4#107meeting, the following agreements and conclusions on NES on RRM have been achieved:

- Agreements - Continue RAN4 work on the following SSB-less SCell scenarios - Scenario 1: SCell without SSB transmission and with TRS transmission - Scenario 2a: SCell without SSB transmission and without any other DL transmissions, but with UL reception at the NW side - Note: No RAN1 impacts are expected, and no RAN4 requirements will be defined if the scenario is not supported from RAN1 specification perspective. - Deprioritize RAN4 work on the following SSB-less SCell scenario - Scenario 2: SCell without SSB transmission and without TRS transmission

In RAN#101 meeting, the following agreements on scenarios are made:

conclusion: scenario 2a is not specified in REL-18.

Based on above agreements, the scenario 1, i.e., SCell without SSB transmission and with TRS transmission is agreed as the supported scenario.

On timing, L1/L3 measurement and TA

For scenario 1, some companies in RAN4 also discuss the issues on timing, TA and L1/L3 measurements, for example, some restriction on how to make UE use the reference timing as the SSB-less cell timing, and details whether UE could reuse the measurement result of reference cell are also under discussion. Therefore, it is better for RAN2 to wait for RAN4 further conclusions before starting any design of higher layer procedures on timing, L1/L3 measurement and TA.

Proposal 2: RAN2 waits for RAN4 further conclusions before starting any design of higher layer procedures specific on SSB-less SCell for inter-band CA for FR1, such as timing, L1/L3 measurement and TA.

On SCell activation

- 6.1.3.55 Enhanced SCell Activation/Deactivation MAC CEs The Enhanced SCell Activation/Deactivation MAC CE with one octet Ci field is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size and consists of seven C-fields, one R-field and zero or more TRS IDj fields in ascending order based on the ScellIndex for SCells indicated by the Ci field(s) to be activated. The Enhanced SCell Activation/Deactivation MAC CE of with one octet Ci field is defined as follows (Figure 6.1.3.55-1). The Enhanced SCell Activation/Deactivation MAC CE with four octet Ci field is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size and consists of 31 C-fields, one R-field and zero or more TRS IDj fields in ascending order based on the ScellIndex for SCells indicated by the Ci field(s) to be activated. The Enhanced SCell Activation/Deactivation MAC CE with four octet Ci field is defined as follows (Figure 6.1.3.55-2). - Ci: If there is an SCell configured for the MAC entity with SCellIndex i as specified in TS 38.331 [5], this field indicates the activation/deactivation status of the SCell with SCellIndex i, else the MAC entity shall ignore the Ci field. The Ci field is set to 1 to indicate that the SCell with SCellIndex i shall be activated and that a TRS IDj field is included for the SCell. The Ci field is set to 0 to indicate that the SCell with SCellIndex i shall be deactivated and that no TRS ID field is included for this SCell; - TRS IDj: If TRS IDj is set to a non-zero value, it indicates the corresponding TRS address by scellActivationRS-Id as specified in TS 38.331 [5] is activated. If TRS IDj is set to zero, it indicates that no TRS is used for the corresponding SCell; - R: Reserved bit, set to 0.

Based on above procedure, the specification TS 38.323 already supports the TRS-level SCell activation/deactivation. Since SSB-less SCell in scenario 1 is with TRS, so the SSB-less SCell activation/deactivation could be at TRS level. Then, the legacy Enhanced SCell Activation/Deactivation MAC CEs could be considered as the baseline for the Activation/Deactivation of SCell without SSB transmission and with TRS transmission.

Proposal 3: The legacy Enhanced SCell Activation/Deactivation MAC CEs could be considered as the baseline for the Activation/Deactivation of SCell without SSB transmission and with TRS transmission.

Conclusion

This contribution discusses topics associated with the WID objective for SSB-less operation, resulting in the following proposals:

Proposal 1: Wait for RAN4 progress on the UE capability for TRS measurement on SSB-less SCell with inter-band CA for FR1 and co-located cells.

Proposal 2: RAN2 waits for RAN4 further conclusions before starting any design of higher layer procedures specific on SSB-less SCell for inter-band CA for FR1, such as timing, L1/L3 measurement and TA.

Proposal 3: The legacy Enhanced SCell Activation/Deactivation MAC CEs could be considered as the baseline for the Activation/Deactivation of SCell without SSB transmission and with TRS transmission.

References

- RP-223540, New WID: Network energy savings for NR