3GPP TSG-RAN WG2 Meeting #123bis	R2-2310096

Xiamen, China, 9th – 13th October, 2023

Agenda item:		7.24.2

Source:		Lenovo

Title:		 Enhanced IF measurement report information

Document for:		Discussion and Decision

- Introduction

In the previous meeting there was discussion on the issue of unpredictable measurement sequence for inter-frequency measurement reporting and specification impact. Following agreements were made:

- Goal to Enhance measurement behaviour: Define a sequence or priority etc for intra/inter-RAT inter-frequency measurement. - UE shall follow the sequence, with R4 req (R4), or best effort (no req). - Higher priority items in the sequence, condition for reporting (R2) - Enhance measurement report information, so that the network can know if the interesting freq has been evaluated or not. RAN2 introduces recommended sequence for intra/inter-RAT inter-frequency measurement (not intend to request R4 work in Rel-18). FFS how this is captured in the TS.

This paper presents a simple solution to enhance measurement report information.

- Discussion

In the last meeting it was agreed for the network to indicate a priority order of the measurement identities and/ or measurement objects, which the UE strictly follows and measures and reports accordingly. As part of enhance measurement report information, UE can also include an indication in the Measurement Report indicating if there are any ongoing measurement in the UE and the measurement sample(s) obtained so far suggest the preliminary measurement value(s) of one or more cells meets certain criterion (e.g., exceeding a certain prelim-threshold), configured by the network. This kind of reporting and the corresponding configuration for preliminary measurements can be configured by the network.

Can the same mechanism be achieved using legacy way e.g., for each configured measurement object, using another reporting configuration with lower threshold Threshold/ TTT etc.? The proponent thinks ‘no’, since first of all, a UE will not be able to distinguish and therefore prioritize these special measurement identities and in addition, what is required here is neither periodic nor even triggered measurements, rather a snapshot of ongoing measurement at the time of otherwise legacy based measurement reporting and checking if the snapshot measurement value meet certain criterion. So, we propose RAN2 evaluate this solution and proceed accordingly.

Measurement Report can include an indication for any ongoing measurement in the UE for the next measurement identity.

- Having received promising preliminary measurement from the UE, the network may wait for a certain time and after this time, based on measurement reports received so far, a handover decision can be made. Since this is anyway network behaviour no proposals for this is required.

# Conclusion

In the previous meeting there was discussion on the issue of unpredictable measurement sequence for inter-frequency measurement reporting – this paper provides one solution direction:

Proposal 1: Measurement Report can include an indication for any ongoing measurement in the UE for the next measurement identity.

# References

[1]: R2-2305774 Discussion on the issue of unpredictable measurement sequence for inter-frequency measurement reporting and specification impact	CMCC, Ericsson, CATT

[2]: R2-2306762 [AT122][003][TEI18] Inter-frequency Measurements (CMCC)	CMCC