3GPP TSG-RAN WG2 Meeting #123bis	R2-2310122

Xiamen, China, 9 – 13 October 2023

Agenda item:	7.12.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Connected mode issues for mobile IAB

WID/SID:	NR_mobile_IAB-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we discuss several connected mode issues for Rel-18 mobile IAB: issues related to RACH-less handover beam indication for on-board UEs, whether to support conditional handover of on-board UEs, and avoiding unnecessary handover measurements at mobile IAB nodes.

# 2	Discussion

## 2.1	RACH-less handover for UEs on board mobile IAB

Below we list the relevant agreements RAN2 has made on RACH-less handover for mobile IAB.

- Feasibility of beam handling during RACH-less HO in the mIAB WI is FFS (and this need to be addressed for RACH-less to be supported for mIAB).

- RAN2 discuss further the following options to support beam operation for the first UL transmission/DL reception towards the target logical DU in RACH-less HO during DU migration:

Option 1: (Explicit approach) Explicit beam information is included in HO command. FFS the details.

Option 2: (Implicit approach) UE re-uses the same beam status as in the source cell (the beam information is not carried explicitly in HO command).

- RACH-less HO with same TA with security key change is in scope for served UEs during mIAB DU migration. FFS UL grant and HO completion procedure in mIAB RACH-less HO.

RAN2#121bis:

- RAN2 think that to have a fast handover from UE point of view for legacy UEs it is important that the target cell is known to the UE (detected and measured).

- For RACH-less, if supported, there would need to be a beam indication (in RRC HO command), which seems feasible in this release from R2 perspective. R2 assumes that the network can know/select the beam, either from network impl specific knowledge or from UE measurement report (legacy report).

- for the UL grant and HO completion in RACH-less HO:

- 1. Both type-1 configured grant and dynamic grant are supported

- 2. FFS handling of supervision timer and when HO is considered successfully complete (expect to align with other WI).

- Send LS to RAN3 to check whether there are issues / feasibility concerns

RAN2#122:

- RACH-less HO to be supported for UEs connected to a mIAB node (intended case: DU migration)

- RACH-less HO for mIAB is expected to reuse most parts from other WI, such as NTN.

- R2 assumes that RACH-less HO for mIAB can largely adopt the steps of the agreed NTN RACH-less HO procedure:

- 1. Receive a RACH-less HO command which can include pre-allocated grant optionally

- 2. Start time T304 for the target cell (RRC)

- 3. Perform DL and UL synchronization.

- 4. Start time alignment timer (MAC)

- 5. Monitor target cell PDCCH for dynamic grant if pre-allocated grant is not configured in RACH-less HO command (MAC, PHY)

- 6. Send initial UL transmission including RRCReconfigurationComplete message using the available UL grant (RRC, MAC, PHY)

- 7. Consider RACH-less HO is completed upon receiving NW configuration.

- 8. Stop timer T304 for the target cell (RRC).

RAN2#123:

As discussed in our previous contribution (R2-2308603) and in the online discussion at RAN2#123, we think that RAN2 should try to align the mobile IAB RACH-less handover procedure as much as possible with the RACH-less procedure being discussed under Rel-18 NTN WI.

However, one aspect where it is not clear whether the two WIs can be fully aligned is on the beam indication, in particular, how the target CU should determine the target DU beam to be indicated in the handover command, e.g. based on UE measurements or on blind indication.

In NTN, the cell size can be very large and support a massive number of UEs; therefore, a major incentive for supporting RACH-less handover is to avoid RACH resource overload due to many UEs simultaneously triggering handover. In this situation, allowing UEs to take measurements to assist with the beam indication could be seen as an acceptable trade-off for mitigating RACH congestion.

In mobile IAB, however, the main incentive for supporting RACH-less handover was to minimize the interruption experienced by connected UEs during the full migration procedure. In this case, requiring UEs to measure and report the beams of the target DU cell would introduce additional delays to the procedure, potentially offsetting the benefits of adopting a RACH-less approach. Furthermore, the appropriate measurement gaps would also need to be specified, which would require further coordination with RAN4 and possibly with RAN1 (who does not have any TUs allocated to the Rel-18 IAB WI).

Observation 1: Relying on UE measurements to determine the beam indications introduces additional delays to the RACH-less handover procedure, which could offset the benefits of RACH-less. Furthermore, RAN4 and RAN1 may need to be further involved to specify the procedure in that case.

Therefore, for mobile IAB, there seems to be some incentive to adopt an approach based on blind beam indications to minimize interruptions and delays for UEs. In this case, there are additional issues that will need to be considered.

If the blind indication is completely blind/random and if the indication is too granular, it may result in suboptimal beam selection and cause failed RACH-less HO; alternatively, if the indication is not granular enough, it could require the UE to spend time taking more precise measurements to refine the beam selection, as in the measurement-based approach discussed above; in either case, the benefits of a totally blind approach seem weak compared to just using a normal RACH-based handover procedure.

A “smarter” blind handover procedure could possibly be implemented if there is a known one-to-one beam mapping between the source and target DU cell beams, i.e. so that the target beam could be known already based on the beam the UE was using in the source cell. In this case, there are not any UE issues foreseen. However, to support blind handover based on beam mapping, it would need to be considered how the mapping is determined between DUs, and also how the mapping is informed to the source and target donor CUs (i.e. in order to select/indicate the beam). How this is handled could also depend on whether the IAB-MT is handed over before or after the DU migration procedure, and also whether the IAB-MT is handed over to the same or a different CU than the one acting as a donor for the target DU. RAN3 would need to be involved, considering the F1AP and XnAP enhancements that would need to be specified.

Observation 2: For blind beam indications to be effective, there would need to be a known one-to-one beam mapping between the source and target DU cells, which is also informed to the source and target CUs. This would require RAN3 to specify F1AP and XnAP enhancements.

Therefore, considering RAN2 has now agreed to support RACH-less handover in mobile IAB, it should be discussed further whether RAN2 thinks it is acceptable for the target cell beam to be determined based on UE measurements or if blind indications need to be supported. In the latter case, RAN2 will need to inform RAN3 as there are clear RAN3 impacts. It is also noted that RAN3 has still not replied to RAN2’s previous LS regarding feasibility of supporting RACH-less handover; thus, it is not clear what their views are on this matter based on our previous agreements, and whether that could change if RAN2 decides that blind beam indications should be supported.

Proposal 1: For RACH-less handover in mobile IAB, RAN2 should discuss whether to adopt measurement-based beam indications (which impacts RAN1/RAN4) or blind beam indications (which impacts RAN3) and inform the impacted work group(s) of the decision.

## 2.2	Conditional handover of on-board UEs

Earlier there were concerns that the full migration scenario could cause problems in the way of unwanted signalling storms or handover failures due to many on-board UEs being handed over from source to target logical DU in parallel with the IAB-MT handover.

RAN2 considered several options for handover of on-board UEs in this scenario. One way to reduce too many handovers being triggered simultaneously is through the use of CHO (supported by Rel-16 UEs and later). Furthermore, in RAN2#121bis, it was discussed whether CondEvent T1 (defined in Rel-17 for NTN-capable UEs) could be extended to Rel-18 TN UEs on-board a mIAB node to “spread out” the handover based on a timer.

FFS: May support CHO with CondT1 if it is “for free”, i.e. if TS impact is just to slightly modify the description to make it also applicable to TN.

CondEvent T1 is triggered when a configured threshold t1-threshold (defined as a specific UTC time in 10ms) has elapsed. CondEvent T1 is currently only applicable to NTN-capable UEs supporting the capability timeBasedCondHandover-r17. However, considering that legacy UEs can already determine the current UTC time from the timeInfoUTC field broadcasted in SIB9, it seems likely that there are no inherent technical issues that would prevent TN UEs from supporting this capability. Indeed, for Rel-18 TN UEs to support CondEvent T1, our understanding is that the required specification work within RAN2 would only be to remove certain restrictions (e.g. in 38.306, 38.331) that currently limit this feature/capability to NTN UEs.

Observation 3: Rel-18 TN UEs can likely support CondEvent T1 with small specification effort within RAN2.

On the other hand, in the mIAB context, the overall benefit from Rel-18 TN UEs supporting CondEvent T1 could be quite small. Recalling why CHO was originally discussed in this context, the purpose was to minimize signalling storms and/or connection failures associated with simultaneous handovers of connected UEs during full migration.

However, in RAN2#119bis, it was agreed (R2-2211101):

RAN2 focuses on the scenario where, during full migration, the UE sees the two logical DU cells as different physical cells (e.g. with different PCI if same carrier), and where the two logical DU cells use separate physical resources (i.e., different carriers, or orthogonal time and frequency resources of the same carrier, as supported by legacy L1).

And, in RAN3#119 it was agreed (R3-231101):

Agreements: The HO of UEs from the source logical mIAB-DU´s CU to the target logical mIAB-DU´s CU should happen after the completion of the F1 setup. When to trigger the HO is up to source logical mIAB-DU´s CU implementation.

Additionally, an earlier agreement from RAN3#117bis states (R3-226101):

mIAB-DU migration and mIAB-MT handover can be executed independently from each other.

These agreements imply that the cells of the source and target logical DUs could be active simultaneously and will be perceived as distinct cells by connected UEs, and further that there is no coupling of the IAB-MT HO with the handover of on-board UEs. Together, these already mitigate the risk of handover failures (at least there is no more risk than inter-CU handover in a stationary network). Furthermore, based on the above agreement from RAN3#119, the source logical DU’s donor CU could “spread out” the handover triggers for different UEs (i.e. to hand UEs over sequentially) in order to minimize signalling storms.

Observation 4: Based on existing agreements in RAN2 and RAN3, signalling storms can already be mitigated during full migration.

Considering the above, the motivation for supporting CondEvent T1 in Rel-18 TN UEs is still not very clear. Even if it is supported, the overall benefit could be quite small anyways since a relatively small number of UEs connected to the mobile IAB node would support it.

Observation 5: The overall motivation for and benefit from Rel-18 TN UEs supporting CondEvent T1 is still not very clear.

Proposal 2: RAN2 does not agree to support CondEvent T1 for Rel-18 TN UEs until benefits are better understood.

## 2.3	Preventing unnecessary measurements of mobile IAB cells

Earlier, in RAN2#119, it was agreed:

- The method of not broadcasting “iab-Support” indication, is sufficient to prevent other IAB-node from accessing mobile IAB (without further spec impact).

We assume that this agreement applies to IAB node integration only. In the context of handover – and particularly with inter-donor handover in mind – this agreement could suggest that before measurement reporting, an IAB node is required to first decode the system information of neighbour cells in order to refrain from reporting cells that are not broadcasting the “iab-Support” or “mobileIAB-Support” indications. Such a requirement could introduce extra measurement gaps, which could unnecessarily disrupt the backhaul of the IAB node.

Since there could be other ways to know whether a target cell belongs to a mobile IAB, e.g. based on XnAP procedures (see discussion in R3-234010 Section 4, for example), we raised this issue at RAN2#121bis and proposed that a mobile IAB-MT should not be required to decode the system information of cells so it can refrain from reporting measurements of other mobile IAB cells (see R2-2302929); however, RAN2 made the following assumption:

R2 assumes that a mobile IAB node is not required to receive the system information of neighbour cells for reporting of measurements (i.e. it will not refrain from reporting measurements of cells that are not broadcasting the “mobile iab Support” indication, and this is acc to current R2 TS).

In our view, the latter part of this assumption, i.e. that a mobile IAB-MT will not refrain from reporting measurements of cells that are not broadcasting the “mobile iab Support” indication, is too strong. For example, RAN2 should not preclude the scenario where a donor CU could configure an IAB-MT to avoid taking measurements of PCIs associated with other mobile IAB cells, e.g. based on the parameter MeasObjectNR > excludedCellsToAddModList. Therefore, RAN2 should clarify the following proposal.

Proposal 3: An IAB node may receive a measurement configuration indicating mobile IAB cells to be excluded from measurements.

# 3	Conclusion

This document has made the following observations:

Observation 1: Relying on UE measurements to determine the beam indications introduces additional delays to the RACH-less handover procedure, which could offset the benefits of RACH-less. Furthermore, RAN4 and RAN1 may need to be further involved to specify the procedure in that case.

Observation 2: For blind beam indications to be effective, there would need to be a known one-to-one beam mapping between the source and target DU cells, which is also informed to the source and target CUs. This would require RAN3 to specify F1AP and XnAP enhancements.

Observation 3: Rel-18 TN UEs can likely support CondEvent T1 with small specification effort within RAN2.

Observation 4: Based on existing agreements in RAN2 and RAN3, signalling storms can already be mitigated during full migration.

Observation 5: The overall motivation for and benefit from Rel-18 TN UEs supporting CondEvent T1 is still not very clear.

And proposed the following:

Proposal 1: For RACH-less handover in mobile IAB, RAN2 should discuss whether to adopt measurement-based beam indications (which impacts RAN1/RAN4) or blind beam indications (which impacts RAN3) and inform the impacted work group(s) of the decision.

Proposal 2: RAN2 does not agree to support CondEvent T1 for Rel-18 TN UEs until benefits are better understood.

Proposal 3: An IAB node may receive a measurement configuration indicating mobile IAB cells to be excluded from measurements.