3GPP TSG-RAN WG2 Meeting #123bis meeting	                      R2-2310141

Xiamen, China, 09th – 13th October, 2023

Agenda item:		7.5.4.2

Source:		Lenovo

Title:		Remaining details on discarding operation for XR

Document for:		Discussion and Decision

# Introduction

- RAN2#122 has agreed the following about the PDU discarding:

Agreement PDU-set discard indication for UL is configured using RRC to handle the PDU Set based discard functionality (i.e. whether UE discards all packets in PDU set when one PDU is discarded). The configuration is per PDCP entity. - Network indicates UE to apply PSI-based XR discard mechanism via dedicated signalling. - FFS how/whether to minimize additional UL signalling after this indication. - FFS if the NW indication is a one-shot or also subsequent packets.

In this contribution we further discuss the remaining details on PDCP discarding for XR communication.

# Discussion

PSI-based Discarding 
In RAN2#121 meeting it was agreed that PSI can be considered for PDU set discarding in the presence of UL congestion. Therefore, in addition to the timer-based discard mechanism within a given PDCP entity, a PDCP discarding mechanism based on PDU set importance level (PSI) is introduced for XR communications.

The detailed discarding mechanism based on the importance level (PSI) of a PDU set in the case of congestion has been initially discussed in RAN2#122 without reaching a consensus for all details yet. However, one general agreement could be achieved which is important for the further design of the discarding procedure. As shown in the agreement below, NW controls the PSI-based discarding at the UE at the presence of congestion. To be more specific, NW explicitly orders the UE to enable/disable PSI-based PDCP discarding, e.g. NW enables/disables PSI-based discarding, based on a detected congestion. Essentially the NW is responsible for the detection of congestion and UE just follows the NW signalling. Details of the NW indication, i.e. enabling/disabling, are FFS.

- Network indicates UE to apply PSI-based XR discard mechanism via dedicated signalling. - FFS how/whether to minimize additional UL signalling after this indication. - FFS if the NW indication is a one-shot or also subsequent packets.

Based on RAN2#123 meeting and further offline discussions the proposed solutions for PSI based discarding that have the strongest support can be basically categorized into the two options: timer-based (Option A) or threshold-based (Option B) discarding solution. When network determines there is congestion and PSI based discarding should be used it indicates to UE to apply PSI based discarding via dedicated signalling. The two options will generally behave as follows:

- Option A: timer-based PDU set discarding

  - UE classifies the PDU set into two groups (i.e., higher and lower importance) based on implementation, NW configures an additional PDCP discard timer value for the lower importance PDU sets which is to be applied when PSI-based discarding is enabled.

- Option B: threshold-based PDU set discarding

  - NW indicates a PSI threshold according to the congestion status and UE discards immediately all PDU sets which have PSIs above the PSI threshold

There are in our view several disadvantages of the threshold-based discarding scheme, which are in detail described in another contribution. Therefore, we want to highlight here only briefly the main problems we see.

The main concern we see with Option B is the inability of the gNB to be aware of the exact PSI level setting/mapping of an application and therefore we think that the gNB is not able to properly set a sensible PSI threshold. Section 4.4.2.6.2 of TS 26.522 describes a whole set of guidelines for setting the PSI levels, e.g. depending on codec, whether the PDU set contains frame, slice, tile etc. In general, the chosen PSI level will very much depend on the particular stream (codec, application, modality (video/audio), level of dependency between PDU sets etc.). Essentially PSI distribution can be very different across applications, but gNB does not have the knowledge about the PSI values of the application. However, it is a prerequisite that RAN is aware of how PSIs are set by an application in order to set the PSI threshold properly.

Furthermore, in option B, when PSI based discarding is enabled, UE discards immediately all low importance packets which may lead to a service quality deterioration. With option A, the UE still has a possibility to deliver at least part of low-importance PDU sets based on the PDCP discard timer configuration. For example, in Option B UE would discard all remaining pending PDUs of a low importance PDU set even though UE might have already transmitted 90% of the low importance PDU set successfully before PSI based discarding was enabled.

We also think that Option A is simpler from specification/implementation perspective. It should be noted that Option B requires the introduction of a new discard mechanism in PDCP while Option A relies on the legacy discard mechanism based on PDCP discard timer.

Proposal 1: RAN2 to agree on the timer-based PDU set discarding mechanism

  - UE classifies the PDU set into two groups (i.e., higher and lower importance) based on implementation, NW configures an additional PDCP discard timer value for the lower importance PDU sets which is to be applied when PSI-based discarding is enabled.

It was mentioned during RAN2#123 discussion that the timer-based solution (Option A above), where different PDCP discard timer values are configured/applied during congestion, may have some problem with the reporting of the remaining time in the DSR. If certain PSI levels (low importance) got a new timer value (higher than 0 but shorter than the normal timer value) those packets would likely be reported as more urgent data since they have less time left until the discarding point. Essentially UE triggers DSR reporting for the low importance PDUs/PDU sets earlier and more often due to the shorter PDCP discard timer values.

We think that DSR reporting is in general not very useful for the case the PSI discarding is enabled, e.g. during congestion.

At least for the unimportant PDU sets/PSI level, we see no additional benefit in providing DSR information to the scheduler. Therefore, disabling the DSR functionality when PSI-based discarding is applied seems like a simple way to address the concern for the timer-based discarding scheme.

Proposal 2: RAN2 to discuss whether to disable DSR reporting for time periods when PSI-based discarding is applied.

NW explicitly indicates the mode of operation and consequently the PDCP discard timer configuration to apply in the UE for a PDCP entity/LCH. UE should apply the indicated PDCP discarding behaviour until NW changes the mode of operation, e.g. NW explicitly switches the mode from “congestion” to normal. The Indication itself should be signalled by means of a MAC CE according to our understanding. Similar to the PDCP duplication enabling/disabling MAC CE, the indication should be done per DRB/PDCP entity.

Proposal 3: UE applies the PDCP discard timer configuration associated with the indicated mode of operation (congestion/normal mode) until NW explicitly changes the mode of operation, e.g. indicating a different PDCP discard timer configuration.

Proposal 4: MAC CE indicates on a DRB level which discarding mode to apply, e.g. PDCP discarding timer configuration associated with the mode (congestion/normal mode).

Impacts of discarding to L2 protocols (RLC/PDCP)

Discarding a packet at the transmitter side – e.g., due to exceeding the corresponding PSDB or due to a lost packet- may depending on what stage the discarding is done require informing the corresponding receiving entity about the discarded packets. If the to be discarded packets have not been yet submitted to lower layers, the associated L2 (PDCP/RLC) SN could be reused for new packets. However, if packets have been already provided to lower layers for a transmission attempt reassignment of Sequence numbers is not possible, e.g. discarding a PDCP SDU already associated with a PDCP SN causes a SN gap in the transmitted PDCP.

We think that the packet dropping mechanism should be enhanced for XR services. Different to the current specified mechanism, where only those RLC PDUs/SDUs can be discarded which have not yet been submitted to the lower layers for transmission, it may be needed to discard packets even if they have already been submitted to lower layers (MAC layer) for transmission. Discarding will be more often invoked for XR services compared to the legacy discard mechanism – e.g. due to the PDU Set Integrated handling Indication (PSIHI). Therefore, it is beneficial from capacity and transmission power point of view to also allow discarding of packets which have already an associated SN. It should be noted that RAN1 agreed to introduce Dynamic indication of unused CG PUSCH occasion(s) based on UCI for cases when UE doesn’t use the allocated CG PUSCH resources. This unused indication could be also used for cases when PDCP PDUs/SDUs and hence the corresponding TBs are discarded for cases when the TB contains data which is of no use for the receiving entity (due to an exceeded PSDB). Assumption here is that the PDUs of a PDU set, e.g. I/P frame are transmitted on CG PUSCH resources. 
As mentioned before this may though require informing the corresponding receiving entity about the discarded packets. The receiving entity may update its receiving window respectively corresponding timers in RLC/PDCP and for example not requesting RLC retransmissions (when applying RLC AM) based on the provided information on discarded packets.

Proposal 5: RAN2 to discuss enhancements to the discarding mechanism, e.g. informing receiving entity about discarded packets at the transmitter side, which may impact PDCP/RLC window operation.

For UL resource allocation, it would be beneficial if UE provides some assistance information regarding the remaining delay budget of the data pending in its buffer to the gNB. In RAN2#119bis meeting it was agreed that providing delay knowledge of buffered data, consisting of at least remaining time, and distinguishing how much data is buffered for which delay is considered is an useful improvement in order to allow for an enhanced uplink scheduling of XR services.

Similarly, gNB would also benefit from a notification by UE about UL data - which were reported in a BSR previously – which was discarded at the transmitter side before transmission, e.g. for which the delay budget is exceeded. gNB can take such knowledge into account for an efficient resource allocation, e.g. avoiding to issue further UL grants for data pending in UE’s buffer which is of no use for the user.

Proposal 6: RAN2 should discuss UE reporting enhancements to inform gNB about discarded PDU/SDUs at the transmitter side, e.g. when the delay budget is exceeded for data which has been previously reported in a BSR.

# Conclusion

In this contribution, we discuss the discarding of PDUs of a PDU set. We have the following proposals:

Proposal 1: RAN2 to agree on the timer-based PDU set discarding mechanism

  - UE classifies the PDU set into two groups (i.e., higher and lower importance) based on implementation, NW configures an additional PDCP discard timer value for the lower importance PDU sets which is to be applied when PSI-based discarding is enabled.

Proposal 2: RAN2 to discuss whether to disable DSR reporting for time periods when PSI-based discarding is applied, e.g. for lower importance PDUs/PDU sets.

Proposal 3: UE applies the PDCP discard timer configuration associated with the indicated mode of operation (congestion/normal mode) until NW explicitly changes the mode of operation, e.g. indicating a different PDCP discard timer configuration.

Proposal 4: MAC CE indicates on a DRB level which discarding mode to apply, e.g. PDCP discarding timer configuration associated with the mode (congestion/normal mode).

Proposal 5: RAN2 to discuss enhancements to the discarding mechanism, e.g. informing receiving entity about discarded packets at the transmitter side, which may impact PDCP/RLC window operation.

Proposal 6: RAN2 should discuss UE reporting enhancements to inform gNB about discarded PDU/SDUs at the transmitter side, e.g. when the delay budget is exceeded for data which has been previously reported in a BSR.

# References

S2-2301378, Reply LS on PDU Set Handling

[2] RAN2#120-e chairman note.