3GPP TSG-RAN WG2 Meeting #123bis	R2-2310144

Xiamen, China, 9th Oct – 13th Oct, 2023

Agenda item:		7.18.2

Source:		Lenovo

Title:		Discussion on remaining CP issues for MT-SDT

Document for:		Discussion and Decision

# Introduction

In RAN2#123 meeting, the following agreements were achieved [1].

Agreements

- Confirm that the condition for data volume threshold is not applicable for Small Data Transmission procedure triggered for MT-SDT

- Sdt-RSRP threshold is included in MT-SDT configuration.  Optional IE, but mandatorily present if only MT-SDT is configured.   The network can configure the MT and MO SDT threshold differently if it wants.

- Specify the setting of mt-SDT indication in the resume cause in section 5.3.2 (i.e. at the time of paging reception).

- Discuss per LCH vs Pre UE configuration of cg-SDT-MaxDurationToNext-CG-Ocacassion-r18 as part of the TEI18 discussion together with the extended CG period discussion. Note regardless of the final outcome of extended CG period, this IE will be specified in some form eventually as originally agreed (only details are FFS)

- When a UE receives a paging message indicating MT-SDT and upon first transmission of the CCCH message (i.e., when conditions for initiating MT-SDT are fulfilled), the UE should start timer T319a.

- As already captured in running spec, only MT-SDT cannot be configured per UE, if the cell supports both MO-SDT and MT-SDT

UE capabilities

- A new optional radio capability signaling (e.g., MT-SDT-r18) is defined to indicate UE’s support of Rel-18 MT-RA-SDT-r18.  MT-RA-SDT-r18 indicates whether the UE supports initiation of MT-SDT procedure and transmission/reception in RRC_INACTIVE state via Random Access procedure (i.e., RACH).

- Separate UE capability is used to indicate support MT-CG-SDT.  If the UE support MT-CG-SDT, it has to support MT-RA-SDT.

- SRB2 capability will be captured by adding “or MT-SDT” Rel-17 srb-sdt-r-17 capability.  (i.e. no separate capability)

- FFS how/whether we define UE capability to indicate UE’s support to select RACH resources instead of configured grant type 1 resource when triggering resume for SDT or MT-SDT and next CG-SDT resource is too far as specified in TS 38.331.

In this contribution, we discuss remaining CP issues related with the MT-SDT procedure.

# Discussion

CG-SDT triggered by MT-SDT

Figure 1: CG-SDT triggered by NW for DL (MT) data

As shown in the figure 1 which displays some exemplary MT CG-SDT procedure, UE transmits on the selected CG resources, the RRCResumeRequest message (CCCH). Since UE may not have any uplink data available for transmission, it may be beneficial if UE transmits in addition to the RRCResumeRequest message some DL related control information which supports link adaptation/scheduling functionality in the NW, e.g. channel state information (CSI) and/or RSRP information, e.g. RSRP of pathloss reference, and/or beam related information on the selected CG resources. The CSI information and/or RSRP info and/or beam information can be for example transmitted within a new MAC CE.

Proposal 1: The RRCResumeRequest message and some DL related control information can be multiplexed and transmitted on the selected CG resources in response to the MT-SDT if there is no UL data available for transmission.

Proposal 2: The downlink related control information may be transmitted within a new MAC CE.

SDT procedure selection

After the email discussion [2], a note is added to the CR that “NOTE 0: If both conditions for initiating MT-SDT and MO-SDT according to 5.3.13.1b are fulfilled, UE may initiate RRC connection resumption procedure for MT-SDT or MO-SDT based on implementation.”.

There could be a case that a paging for MT-SDT message is received at T1; The RRC layer may determine that the RRC connection resume procedure is be initiated since the RRC layer may not identity when the UL data arrives.  Then the UL data arrives after the UE (e.g. the lower layer or RRC layer) determines that conditions for initiating MT-SDT are fulfilled. Is the UE allowed to initiate a MO-SDT instead of the ongoing MT-SDT before the first transmission (CCCH message) of the MT-SDT? Referring to the agreement, we think this procedure can be supported. Otherwise, we can consider only one ongoing SDT is supported besides the agreements on “Assumption is that if the UE has UL data the UE can still check and trigger MO-SDT”.

Proposal 3: UE is allowed to initiate a MO-SDT instead of an already initiated MT-SDT before the first transmission (CCCH message) of the MT-SDT.

To support the MO-SDT which replaces the MT-SDT, UE would need to make sure that CCCH is ready before transmission, i.e. UE needs to update the resume cause value accordingly.

Proposal 4: UE needs to update the content in CCCH message according to the latest triggered SDT procedure.

# Conclusion

In this contribution, we discuss the procedure related with MT-SDT and provide the proposals:

Proposal 1: The RRCResumeRequest message and some DL related control information are multiplexed and transmitted on the selected CG resources for MT-SDT if no UL data available.

Proposal 2: The downlink related control information can be transmitted within a new MAC CE.

Proposal 3: UE is allowed to initiate a MO-SDT instead of an already initiated MT-SDT before the first transmission (CCCH message) of the MT-SDT.

Proposal 4: UE needs to update the content in CCCH message according to the latest triggered SDT procedure.

# References

RAN2#123 chairman notes.

[POST123][301][MT-SDT] CR to 38.331 (ZTE))