3GPP TSG-RAN WG2 Meeting #123bis	R2-2310192

Xiamen, China, October 9th – 13th, 2023

Agenda Item		:	7.6.3.2

Source		:	Inmarsat, Viasat, Sateliot, Novamint, ESA, Thales

Title		:	NB-IoT NTN Coarse UE location reporting

Document for		:	Approval

# 1. Introduction

In this contribution, we want to highlight the need to address a significant gap in the lack of an early and efficient mechanism for NB-IoT NTN UE to report their coarse location to the network, which is a crucial functionality for NTN systems.

NB-IoT NTN networks are becoming operational as we speak, and the lack of a mechanism for UE to report their location to the network, at least to allow the network to know the general location of the UE within the coverage area to the granularity roughly equivalent to a terrestrial cell is emerging as a significant gap that can severely impact network efficiency, mobility and even basic operation in regulatory compliance.

This topic had been discussed in Release-17, and solutions for all other NTN RAT have been identified and specified, including for eMTC, but NB-IoT has been left behind.  Whilst all NTN UE are expected to have GNSS capability, NB-IoT NTN UE presently don’t have a guaranteed mechanism to report their location to the network.

In this paper we urge RAN2 to urgently introduce an effective solution to this problem, within the time frame of Release-18.

# 2. Discussion

The urgent need to specify a mechanism for early and efficient reporting of at least “coarse-grained” location by NB-IoT NTN UE to the network is driven by three main scenarios:

Basic Regulatory compliance – When an NTN cell coverage overlaps two regulatory areas (e.g. cross-country borders), without knowledge of the UE location it may be impossible to determine in which jurisdiction the UE is located.  In the worst case, in case of ambiguity the network may not be able to provide service to avoid breaching regulations.

Efficient Radio Resource Management and cell mapping – Logical cell mapping and frequency management in dynamic satellite systems heavily relies on knowledge of UE distribution.  Without this information, the systems may have to operate at significantly lower efficiency.

Mobility Management – To support efficient mobility management in NTN, the size of TAI+CGI should ideally be comparable to TN deployments.  However, this resolution might be higher than the typical satellite beam footprint, making mobility far less efficient than optimal.

Observation 1:  UE location reporting to the network is a fundamental requirement for NTN to support:

- Basic Regulatory Compliance

- Efficient Radio Resource Management and Cell Mapping

- Efficient Mobility Management

Mobility management in IoT NTN is currently based on the definition of Tracking Areas (TAs) and Mapped Cell IDs, which correspond to fixed geographical areas on ground. Assumption is that E-UTRAN would provide its best estimate of the UE’s location as a TAI+CGI to the MME. To support efficient mobility management (as least as efficient as supported in TN networks), the size of these TAI+CGI related areas should, ideally, be comparable to the ones used in terrestrial deployments (e.g., within 5-10 km for the mapped cell ID, similar to typical terrestrial network cell size).

However, achieving a resolution within 5-10 km is challenging, especially with NGSO satellites that rely on large and earth-moving beams (e.g., beams of hundreds of kms in diameter) or GSO satellites. For these deployment cases, and given that NTN-capable devices are expected to support GNSS positioning, enabling UE reported location information (e.g., for example UE GNSS coordinates) would allow the MME to count with a more accurate UE location estimate that could be exploited for, e.g., improved mobility management.

Furthermore, with current flexible satellites, UE location is important also for resource management and dynamic cell mapping.  For this purpose, however, a lower resolution, down to 50-100 km (roughly equivalent to large terrestrial macro cell) could be sufficient.

To our best knowledge, in current specifications, there is no mandatory feature for UE location reporting for NB-IoT NTN devices. In more detail:

•	The RRC CoarseLocationInfo mechanism introduced in Rel-17 is not supported for NB-IoT. Such mechanism was only defined for eMTC. The reason is that sending location information over RRC needs AS security to be in place and AS security is not mandatory for NB-IoT devices with support for CP CIoT EPS Optimization only.

o	In practice, for any kind of device that would only support CP CIoT EPS Optimization with Data over NAS, the same assumption will hold, thus AS security cannot be assumed to be in place.

•	The LPP protocol, which actually would allow for reporting of GNSS coordinates from the UE to a location server on the network side, cannot be assumed to be supported by the device, as it is not mandatory for NB-IoT devices.  And in fact is NOT supported by the vast majority of NB-IoT devices.

Observation 2: RRC CoarseLocationInfo mechanism was introduced in Release-17 IoT NTN for eMTC but not for NB-IoT NTN, due to lack of AS security and no alternative solution was provided.

Observation 3: In practice, we have to recognize that majority of NB-IoT NTN devices will focus on CP CIoT Optimizations with Data over NAS, thus AS security cannot be assumed to be in place.

Observation 4:  Methods relying on LPP cannot be relied upon, if anything because LPP is not supported by most NB-IoT UE implementations.  This is already affecting Release-17 UE and it’s clear that we must identify a different solution.

Previous discussion on the support of UE location reporting for NB-IoT NTN, including an early report of coarse UE location, was held in the context of Rel-17 (R2-2204458, R2-2204437) and some proposals assuming UE location knowledge on the CN side were considered when studying discontinuous coverage in SA2 in Rel-18 (TR 23.700-28, S2-2207184).  Moreover, the implicit requirement for UE location information in the context of satellite coverage crossing various regulatory areas was already assumed in the early phases of NTN system architecture studies (TR 23.737)

Figure 1 - NTN cell coverage may span multiple regulatory areas, some of which may have restrictions which do not allow service to be provided.  Without UE location information, network may not be able to determine whether UE is allowed service or not.

However, due to a misjudgement on the required accuracy of the location reporting and the privacy and security implications related to its transmission early on during the access procedure, this functionality was not specified for NB-IoT.  Furthermore, the implementation of coarse UE location over NAS was discounted, but should be re-examined, in case the RRC solution is not feasible or not sufficient.

Lastly, whilst currently the network has the ability to request UE for Location Information after explicit or implicit user consent through coarseLocationReq-r17, this mechanism is not available for NB-IoT, and moreover is implemented in UE-dedicated signalling, which is not scalable at all in a satellite network, where all UE are expected to report their location.

During the Release-17 analysis, concerns were raised to implications related to user privacy when reporting coarse UE location to the network. However, the existence of concrete user privacy concerns when reporting of coarse UE location at a level of granularity equivalent to a terrestrial cell (5 km or larger) is highly questionable, given that such concerns do not exist in terrestrial networks (the network knows the UE is at least within the area of the serving cell or within an area in which it’s capable of measuring and possibly attaching to a number of neighbor cells, which can be even smaller than 5 km).

Observation 5: It is difficult to justify privacy concerns when reporting Coarse UE location at a level of granularity equivalent to the size of a terrestrial cell (e.g. 5 km or larger), given that such concerns with knowledge of the UE location by an external observer within the granularity of a cell do not exist in terrestrial networks.

We therefore propose the following:

Proposal 1: Specify a mechanism for UE location reporting as early as possible during the initial access procedure for NB-IoT NTN devices, including those devices with support for CP CIoT EPS Optimization only.

- At least coarse UE location on the order of 50-100 km shall be supported;

- Coarse UE location on the order of 5-10 km is desirable.

- Specify a method to broadcast the request for Coarse UE Location to a group of UE or to all UE within a cell.

- AS security should not be assumed.

- Consider specifying support for full UE location reporting over NAS with NAS security activated.

# 3. Conclusion

Observation 1:  UE location reporting to the network is a fundamental requirement for NTN to support:

- Basic Regulatory Compliance

- Efficient Radio Resource Management and Cell Mapping

- Efficient Mobility Management

Observation 2: RRC CoarseLocationInfo mechanism was introduced in Release-17 IoT NTN for eMTC but not for NB-IoT NTN, due to lack of AS security and no alternative solution was provided.

Observation 3: In practice, we have to recognize that majority of NB-IoT NTN devices will focus on CP CIoT Optimizations with Data over NAS, thus AS security cannot be assumed to be in place.

Observation 4:  Methods relying on LPP cannot be relied upon, if anything because LPP is not supported by most NB-IoT UE implementations.  This is already affecting Release-17 UE and it’s clear that we must identify a different solution.

Observation 5: It is difficult to justify privacy concerns when reporting Coarse UE location at a level of granularity equivalent to the size of a terrestrial cell (e.g. 5 km or larger), given that such concerns with knowledge of the UE location by an external observer within the granularity of a cell do not exist in terrestrial networks.

Proposal 1: Specify a Release-18 mechanism for UE location reporting as early as possible during the initial access procedure for NB-IoT NTN devices, including those devices with support for CP CIoT EPS Optimization only.

- At least coarse UE location on the order of 50-100 km shall be supported;

- Coarse UE location on the order of 5-10 km is desirable.

- Specify a method to broadcast the request for Coarse UE Location to a group of UE or to all UE within a cell.

- AS security should not be assumed.  SA3 should consider an exception, if necessary.

- Consider specifying support for full UE location reporting over NAS with NAS security activated.

# 4. References

- R2-2204458

- R2-2204437

- 3GPP TR 23.700-28

- S2-2207184

- 3GPP TR 23.737