3GPP TSG-RAN WG2#123bis	R2-2310194

Xiamen, China, 9-13 October 2023

Agenda Item:	7.2.2

Source: 	MediaTek Inc.

Title:  	SLPP signalling and procedures

Document for:	Discussion, decision

# 1	Introduction

This document examines the procedures described by SA2 in [1] for sidelink positioning and extends on the procedures proposed in [2], with a focus on establishing the SLPP functionality that needs to be specified at stage 3 level.  The primary focus is on UE-only operation, but with SLPP supporting the LMF as an endpoint, we provide a set of procedures that can also be used in network-involved cases.

# 2	Discussion

## 2.1	Overview of SL positioning procedure

In [1], a general procedure for UE-only ranging/SL positioning is defined as shown in figure 1.

Figure 1: Ranging/SL positioning procedure (figure 6.8.1-1 of [1])

We consider that there are some concerns with this procedure, as identified in [2], but SA2 have left enough flexibility that RAN2 should be able to specify valid behaviour.  In short, the following changes and elaborations to the procedure in figure 1 appear to be necessary:

- It needs to be clear that UE1..UEn, except for the target UE, are candidate anchor UEs, and there should be some opportunity for downselection to comport with the needs of the positioning operation:

  - The server UE will select a positioning method, and some of UE1..UEn may not support the selected method; such UEs should be dropped from the eventual set of anchor UEs.

  - If forwarding of SLPP messages is not supported in Rel-18, the anchor UEs should be visible over PC5 to both the target UE and the server UE.  If some of UE1..UEn lack such visibility, they should be dropped from the eventual set of anchor UEs (this downselection can be realised as part of steps 2 and 6, without modification to the figure).

  - The server UE is responsible for ensuring that the positioning operation meets QoS requirements, so it needs to have discretion about which anchor UEs would be used.  A good operation would provide a large set of candidate anchor UEs, allowing the server UE to downselect to what it considers a “sufficiently useful” set.

  - The description in [1] of step 1a implies that the client UE (not shown) selects the initial pool of (candidate) anchor UEs.  We are not clear on how this works, given that the client UE does not know in advance where the target UE is located (nor what positioning method will be used, nor the capabilities of any of the involved UEs, nor the identity of the server UE, etc.), and it may be useful to clarify this point with SA2.

- The capability exchange in step 4 does not appear to be useful unless UE1 forwards all the capabilities of the candidate anchor UEs to the server UE.  Such forwarding appears to have been the intention of SA2, but as noted in [2], it may be signalling-inefficient to distribute the capabilities of all candidate anchor UEs in this way.

  - At step 4, the involved UEs do not yet know what positioning method will be selected (even at the level of what methods can be supported by the eventual server UE), so it seems impossible to retrieve capabilities in an intelligent way.  Step 4 would need to comprise a complete dump of all capabilities of UE2..UEn, which could be a large amount of signalling (many candidates whose capabilities need to be forwarded en masse to the server UE).

  - In fact, the description of step 4 indicates “Step 4 may be performed during step 5 and step 6 with coordination of SL Positioning Server UE.”  We consider this to be a more promising approach to gathering capabilities.

  - We do consider that the server UE needs the capabilities of the target and anchor UEs.  Otherwise there is a risk that the method determination at the server UE settles on a method that the participating UEs cannot support.

  - In any case, provided capability exchange is supported in SLPP between a pair of communicating UEs, the stage 3 functionality should be able to support a variety of overall sequences of operations.

- Step 5 conceals significant complication.  As analysed in [2], we consider that the following steps are necessary within SA2’s step 5:

  - Discovery between UE1 and server UE (here we suggest that UE1 should solicit discovery, as discussed in [2])

  - Delivery of server capabilities to UE1 (target/anchor)

  - UE1 selects server and delivers list of candidate anchor UEs

    - New SLPP transaction type could be used for this step

    - RAN2 agreed not to specify how the server UE is selected, but it seems necessary to notify it that it has been selected, and give it a chance to accept or refuse

    - Notification of selection could be provided in upper layers (e.g., with an SL-MO-LR request), but it is not clearly specified in [1]

  - Delivery of UE1 capabilities to server

  - Discovery between server UE and UE2..UEn (can be omitted if SLPP forwarding is used for all communication between the server and these UEs)

  - Delivery of UE2..UEn capabilities to server (to server directly, and/or from UE1 to server if SLPP forwarding is used)

Proposal 1: Send an LS to SA2 to clarify RAN2 understanding that UE1..UEn in figure 6.8.1-1 of TS 23.586 are candidate anchor UEs (except for one of them, which is the target UE), and the server UE is expected to downselect based on which anchors are useful (considering anchor UE capabilities, geometry, QoS requirements, etc.).  SA2 can be asked to take this interpretation into account and feed back only if a problem is detected.

Proposal 2: Specify in SLPP a procedure for exchange of capabilities between two peer UEs, agnostic to their roles (with the exception of the server UE, as in proposal 13).

Proposal 3: Specify in SLPP a procedure for announcing selection of a server UE and delivering a list of candidate anchor UEs (item 3c in the list above).  If SLPP forwarding is supported for the delivery of anchor UE capabilities to the server UE, the list of candidate anchor UEs can also include their capabilities.

Proposal 2 could be realised by including an SLPP PDU in the discovery signalling (step 5) or by separate SLPP signalling after discovery, but from the SLPP perspective, only the capability procedure needs to be specified.  Proposal 3 requires a new SLPP transaction type and a corresponding new section in [3].

(For a view of the overall procedure implied by the “expanded” step 5, refer to figure 2 of [2]).

## 2.2	Assistance data transfer

### 2.2.1	Models for assistance data

The description in [1] of step 6 of figure 1 simply says that assistance data are transferred, without indicating who transfers them, how the transfer is initiated, or what purpose the assistance data serve.  There are several possible alternatives:

- The server UE negotiates SL-PRS configurations with the SL-PRS Tx UE(s) (target and/or anchors, depending on the positioning method selected), then distributes those configurations as assistance data to the SL-PRS Rx UE(s) (anchors and/or target, depending on the positioning method selected).  The assistance data implicitly identify which anchor UEs the server UE has selected for use.

- Each SL-PRS Tx UE announces its own SL-PRS configuration to each SL-PRS Rx UE, with the transferred configurations being modelled as assistance data sent between the UEs.  No assistance data are transferred to or from the server.

- As #2, but additional assistance data in the form of known anchor UE locations are transferred from the anchor UEs to the server UE to assist with calculating the location estimate.

Throughout this list, “SL-PRS configurations” refers to the semistatic parameters affecting the SL-PRS (e.g., sequence ID, comb, any other parameters to be determined by RAN1), but not to the sidelink radio resource allocation, which we understand will be managed in lower layers.

All these models can be realised by two transaction types: requested assistance data transfer and unsolicited assistance data transfer, both of which already have analogues in LPP.  The framework of [3] includes space for Request Assistance Data and Provide Assistance Data messages.

Proposal 4: Confirm that SLPP includes Request Assistance Data and Provide Assistance Data messages, and the assistance data transfer procedure can take place in solicited or unsolicited form.

### 2.2.2	Alternative #1

For alternative #1, the server UE needs to learn the assistance data for the involved UEs.  There could be cases where this information can be semistatically configured to the server UE (e.g., if the anchor UEs are RSUs whose location is known to the system), but it seems useful in the general case to allow UE1..UEn to provide some information (SL-PRS configuration, locations of the anchor UEs, etc.) to the server UE.

On Uu, the LMF requests configurations for DL-PRS and SRSp from the gNBs, who have the final word; the exactly analogous behaviour would be for the server UE to request SL-PRS configurations from the anchors (only), have the anchors take the final decision on what configurations will be used in both directions (target-to-anchor and anchor-to-target), and have the anchors (collectively, somehow) dictate that information to the target UE.  This does not seem entirely realistic on the sidelink, where the target UE normally has more autonomy than a UE operating on Uu and where there is no single point of control analogous to the gNB; it also requires complex coordination between the anchor UEs to determine a configuration for the target UE.

On the other hand, the server UE will ultimately perform the position calculation, and as on Uu, the server could be expected to know what characteristics the measurements need to meet the required accuracy.  Thus, it does seem reasonable for the server UE to request configurations from the other UEs.  We therefore suggest that the server UE could make an initial request for a configuration from each SL-PRS Tx UE, receive from the corresponding UE a final configuration that the UE can support, and distribute the resulting set of configurations to each SL-PRS Rx UE.  Such a flow is shown in figure 2.

Figure 2: Assistance data distribution (alternative #1)

Proposal 5: Define an SLPP procedure that can be used by the server UE to retrieve configurations and/or locations from the anchor UEs, either by reusing the Request/Provide Assistance Data messages or by defining new Request/Provide UE Configuration messages.

### 2.2.3	Alternative #2 and alternative #3

Alternative #2 can be seen as a peer-to-peer configuration method, in which each SL-PRS Tx UE autonomously determines a configuration and delivers the configuration to each SL-PRS Rx UE.  It is not fully clear how the configuration will be determined in this decentralised way, however, since the target and anchor UEs do not know how the location estimate will be computed and how the configuration parameters influence the calculation.  (The UEs except for the distinguished UE1 also do not know the QoS requirement, and it cannot in general be assumed that the target and anchor UEs have extensive positioning-algorithm knowledge that would allow them to evaluate what configurations are required to meet the QoS.)

If we stipulate that the Tx UE(s) can determine suitable SL-PRS configurations, the resulting flow is trivial, as shown in figure 3.  No Request Assistance Data message is shown, but there could be one as an initial trigger; the Rx UE(s) could incite the Tx UE(s) to determine their SL-PRS configurations by sending an SLPP Request Assistance Data.

Figure 3: Assistance data distribution (alternative #2)

Alternative #3 is the same, with the addition of assistance data to the server UE for the anchor UE locations, as shown in figure 4.  The example in the figure assumes downlink-like positioning (SL-PRS from anchors to target); for the uplink-like case, the arrow in step 1 would be reversed since the target is the SL-PRS Tx UE.

Figure 4: Assistance data distribution (alternative #3) for downlink-like positioning

In general, we consider these alternatives a bit impractical, given the issues described above.  However, there could be specific cases in which the involved UEs have additional positioning knowledge, and assistance data from UE1..UEn could be considered for such cases.

From the stage 3 signalling perspective, as long as the Provide Assistance Data message is specified in a role-agnostic way, all alternatives can be supported.  However, there may be interoperability problems if different UEs support different alternatives (e.g., if the server expects a Tx UE to configure itself while the Tx UE expects a configuration from the server), and this should be considered at the procedure level.

## 2.3	Measurement data transfer and location estimate delivery

The measurement data transfer aspect is naturally similar to Uu positioning: One or several UEs take measurements, and the measurements are delivered to the server UE as inputs to the position calculation.  In SLPP, it should be straightforward to realise this model with the Request/Provide Location Information messages, as on Uu.  Measurements may be requested from the target, the anchors, or both, but in terms of SLPP modelling, we only need to capture the location information transaction.

Proposal 6: Confirm that SLPP includes Request Location Information and Provide Location Information messages.

If SLPP forwarding is to be supported, it would allow the server UE to retrieve measurement data from SL-PRS Rx UEs to which it is not directly connected.  In this case, two transactions would be necessary, from the server to a Tx UE and from the Tx UE to the Rx UE, as shown in figure 5.  If the location information messages are role-agnostic, there is no stage 3 impact (except for the forwarding support).

Figure 5: Location information transactions with SLPP forwarding

Separate from the forwarding issue, the UE that communicated with the client (UE1 in the SA2 flows; we understand that this would typically, but not always, be the target UE) needs to respond to the client with the location estimate—step 7 of figure 6.7.1-1 in [1].  In Uu positioning, the server would deliver the result as a response to the original LCS request; in sidelink positioning, however, the equivalent of the LCS request comes to UE1 (target or anchor), so the computed location estimate needs to be delivered to UE1.  This delivery could take place with a new SLPP transaction type, or it could reuse the Provide Location Information; in either case, from the SLPP perspective, it is a single unsolicited message, as shown in figure 6.

Figure 6: Delivery of location estimate to UE1

Proposal 7: SLPP supports delivery of the computed location estimate from the server UE to the target or an anchor UE in an unsolicited message.  It can be discussed if the message is a Provide Location Information or a new transaction type.

## 2.4	Integrating everything

Combining the foregoing descriptions suggests an end-to-end behaviour for a typical downlink-like positioning case as shown in figure 7.  This flow uses alternative #1 for assistance data delivery, but if alternatives #2 and/or #3 are supported, steps 13-15 could be replaced with the equivalent steps from those alternatives.  It may not be necessary to capture this flow in a spec; from the stage 3 perspective, the exact order of operations is up to the participants as usual.  However, since all participants are UEs, it may be useful to specify the expected overall behaviour as a guideline to implementation.

The figure is shown in terms of the target and anchor roles, with implied assumptions about which method is in use and which UE is UE1.  This may result in a more readable figure than a generic method-agnostic and role-agnostic flow, but we are open to discuss exactly what should be captured.

Figure 7: SLPP portions of positioning procedure (downlink-like positioning using alternative #1)

Proposal 8: Consider the flow of figure 7 as a baseline operation for downlink-like positioning.  FFS whether to capture it (and similar flows for other positioning methods) in a spec.

## 2.5	Incorporating the LMF

The examples to this point in this paper have focussed on the server UE, but there is no large difference in most of the procedures when the server is an LMF.  In accordance with the agreement to identify the participants in SLPP as “Endpoint A” and “Endpoint B”, we suggest that the LMF can be incorporated into the foregoing proposals simply by interpreting the corresponding endpoint to be either a server UE or an LMF.

The following restrictions apply to LMF participation in SLPP procedures:

- In LPP, the capability exchange procedure is not used to expose LMF capability to the UE.  We do not identify a use case for such exposure in SLPP either.

- The only apparent use for a Provide Assistance Data going to the LMF is if the assistance data transaction is reused for negotiation of SL-PRS configurations (analogous to PRS/SRS configuration negotiation in NRPPa).

- We do not identify a use case for sending a Request Location Information to the LMF.

- The only apparent use case for the LMF to transmit a Provide Location Information is if the unsolicited form of the location information transaction is reused to deliver the location estimate to the target UE at the end of a sidelink positioning operation (no analogue in LPP).

- The server UE selection procedure obviously does not apply to an LMF.

Proposal 9: For the SLPP capability transfer procedures, the endpoint that receives the capability may be an LMF; the endpoint that transmits the capability may not be an LMF.

Proposal 10: For the SLPP assistance data transfer procedures, the endpoint that provides the assistance data may be an LMF; FFS if the endpoint that receives the assistance data may be an LMF.  (This FFS point relates to the possibility of using the assistance data transfer procedures to inform the server of the anchor or target UE configurations.)

Proposal 11: For the SLPP location information transfer procedures, the endpoint that receives the location information may be an LMF; FFS if the endpoint that provides the location information may be an LMF (applicable to the unsolicited case only).  (This FFS point relates to the possibility of using the location information transfer procedures to deliver the location estimate at the end of a sidelink positioning operation.)

Proposal 12: (Obvious) For the server UE selection procedure, neither endpoint can be an LMF.

Finalisation of the LMF roles depends on the possible support of SLPP forwarding (i.e., delivering data through SLPP over two hops, such as measurements from anchor to target to LMF).  We discuss this issue separately in [4]; the TP in this document is written agnostically to these conclusions.

## 2.6	Endpoint roles

From a pure protocol-definition point of view, it may not be necessary to specify which endpoint can take which roles (e.g., “Endpoint A may be a target UE”).  Assuming SLPP forwarding is supported (see [4]), it appears that a UE in any role can receive or transmit any message type, with the exception of the capability procedures (where it is not clear if the server UE can be interrogated for its capability) and the server UE selection procedure (which can obviously take place only between the target and server UEs).

On interrogation of the server UE for capabilities, the main question is whether the capabilities would influence the selection of the server UE.  The ability to function as a server UE will already be advertised in the discovery messages; it is not clear if there would be additional capabilities that should be taken into account by the target/anchor UE selecting the server.  We suggest that this point can be captured as FFS for now.

Proposal 13: In the procedure description in section 5.1.1 of TS 38.355, capture an FFS point on whether the server UE can be interrogated for its capability.

Proposal 14: In the procedure description for the server UE selection indication procedure, capture the restriction that it can take place only between a target or anchor UE and a server UE.

On the other hand, as described above, there are restrictions on which SLPP messages can involve an LMF, and these restrictions should be indicated in the spec (so that a UE has clear guidance not to send an unsupported message to an LMF, and so that a UE implementation can know which message types it does not need to expect from an LMF).

Proposal 15: In the procedure descriptions in section 5 of TS 38.355, capture the restrictions on which endpoint can be an LMF in accordance with RAN2 agreements.

# 3	Conclusion

This document promulgated the following proposals:

Proposal 1: Send an LS to SA2 to clarify RAN2 understanding that UE1..UEn in figure 6.8.1-1 of TS 23.586 are candidate anchor UEs (except for one of them, which is the target UE), and the server UE is expected to downselect based on which anchors are useful (considering anchor UE capabilities, geometry, QoS requirements, etc.).  SA2 can be asked to take this interpretation into account and feed back only if a problem is detected.

Proposal 2: Specify in SLPP a procedure for exchange of capabilities between two peer UEs, agnostic to their roles (with the exception of the server UE, as in proposal 13).

Proposal 3: Specify in SLPP a procedure for announcing selection of a server UE and delivering a list of candidate anchor UEs (item 3c in the list above).  If SLPP forwarding is supported for the delivery of anchor UE capabilities to the server UE, the list of candidate anchor UEs can also include their capabilities.

Proposal 4: Confirm that SLPP includes Request Assistance Data and Provide Assistance Data messages, and the assistance data transfer procedure can take place in solicited or unsolicited form.

Proposal 5: Define an SLPP procedure that can be used by the server UE to retrieve configurations and/or locations from the anchor UEs, either by reusing the Request/Provide Assistance Data messages or by defining new Request/Provide UE Configuration messages.

Proposal 6: Confirm that SLPP includes Request Location Information and Provide Location Information messages.

Proposal 7: SLPP supports delivery of the computed location estimate from the server UE to the target or an anchor UE in an unsolicited message.  It can be discussed if the message is a Provide Location Information or a new transaction type.

Proposal 8: Consider the flow of figure 7 as a baseline operation for downlink-like positioning.  FFS whether to capture it (and similar flows for other positioning methods) in a spec.

Proposal 9: For the SLPP capability transfer procedures, the endpoint that receives the capability may be an LMF; the endpoint that transmits the capability may not be an LMF.

Proposal 10: For the SLPP assistance data transfer procedures, the endpoint that provides the assistance data may be an LMF; FFS if the endpoint that receives the assistance data may be an LMF.  (This FFS point relates to the possibility of using the assistance data transfer procedures to inform the server of the anchor or target UE configurations.)

Proposal 11: For the SLPP location information transfer procedures, the endpoint that receives the location information may be an LMF; FFS if the endpoint that provides the location information may be an LMF (applicable to the unsolicited case only).  (This FFS point relates to the possibility of using the location information transfer procedures to deliver the location estimate at the end of a sidelink positioning operation.)

Proposal 12: (Obvious) For the server UE selection procedure, neither endpoint can be an LMF.

Proposal 13: In the procedure description in section 5.1.1 of TS 38.355, capture an FFS point on whether the server UE can be interrogated for its capability.

Proposal 14: In the procedure description for the server UE selection indication procedure, capture the restriction that it can take place only between a target or anchor UE and a server UE.

Proposal 15: In the procedure descriptions in section 5 of TS 38.355, capture the restrictions on which endpoint can be an LMF in accordance with RAN2 agreements.

A text proposal to TS 38.355 is appended.

# 4	References

[1]	TS 23.586

[2]	R2-2307340, “SLPP signalling in UE-only sidelink positioning/ranging procedures”, MediaTek Inc., RAN2#123

[3]	TS 38.355 v1.0.0

[4]	R2-2310195, “SLPP forwarding”, MediaTek Inc., RAN2#123bis

# 5	Text proposal

## 5.1	Procedures related to capability transfer

### 5.1.1	General

The purpose of the procedures that are grouped together in this clause is to enable the transfer of capabilities from Endpoint A to Endpoint B. Endpoint A may be a target or anchor UE. Endpoint B may be a target, anchor, or server UE or an LMF. Capabilities in this context refer to positioning and protocol capabilities related to SLPP and the positioning methods supported by SLPP.

Editor’s Note: It is FFS if Endpoint A can be a server UE.

### 5.1.2	Capability Transfer procedure

The Capability Transfer procedure is shown in Figure 5.1.2-1.

Figure 5.1.2-1: SLPP Capability Transfer procedure

1.	Endpoint B sends a Request Capabilities message to Endpoint A. Endpoint B may indicate the types of capabilities needed.

2.	Endpoint A responds with a Provide Capabilities message to Endpoint B. The capabilities shall correspond to any capability types specified in step 1.

## 5.1.3	Capability Indication procedure

The Capability Indication procedure is shown in Figure 5.1.3-1.

Figure 5.1.3-1: SLPP Capability Indication procedure

1.	Endpoint A sends a Provide Capabilities message to Endpoint B.

## 5.1.4	Reception of SLPP Request Capabilities

### 5.1.5	Transmission of SLPP Provide Capabilities

## 5.2	Procedures related to Assistance Data Transfer

### 5.2.1	General

The purpose of the procedures that are grouped together in this clause is to enable Endpoint A to request assistance data from Endpoint B to assist in positioning, and to enable Endpoint B to transfer assistance data to Endpoint A in the absence of a request.

### 5.2.2	Assistance Data Transfer procedure

### The Assistance Data Transfer procedure is shown in Figure 5.2.2-1.

Figure 5.2.2-1: SLPP Assistance Data Transfer procedure

1.	Endpoint A sends a Request Assistance Data message to Endpoint B.

2.	Endpoint B responds with a Provide Assistance Data message to Endpoint A containing assistance data. The transferred assistance data should match or be a subset of the assistance data requested in step 1. Endpoint B may also provide any not requested information that it considers useful to Endpoint A.

3.	Endpoint B may transmit one or more additional Provide Assistance Data messages to Endpoint A containing further assistance data. The transferred assistance data should match or be a subset of the assistance data requested in step 1. Endpoint B may also provide any not requested information that it considers useful to Endpoint A.

### 5.2.3	Assistance Data Delivery procedure

The Assistance Data Delivery procedure is shown in Figure 5.2.3-1.

Figure 5.2.3-1: SLPP Assistance Data Delivery procedure

1.	Endpoint B sends a Provide Assistance Data message to Endpoint A containing assistance data.

2.	Endpoint B may transmit one or more additional Provide Assistance Data messages to Endpoint A containing additional assistance data.

### 5.2.4	Reception of SLPP Request Assistance Data

### 5.2.5	Transmission of SLPP Provide Assistance Data

## 5.3	Procedures related to Location Information Transfer

### 5.3.1	General

The purpose of the procedures that are grouped together in this clause is to enable Endpoint B to request location information from Endpoint A, and to enable Endpoint A to transfer location information to Endpoint A in the absence of a request.

### 5.3.2	Location Information Transfer procedure

The Location Information Transfer procedure is shown in Figure 5.3.2-1.

Figure 5.3.2-1: SLPP Location Information Transfer procedure

1.	Endpoint B sends a Request Location Information message to Endpoint A, indicating the type of location information needed and potentially the associated QoS.

2.	Endpoint A sends a Provide Location Information message to Endpoint B to transfer location information. The location information transferred should match or be a subset of the location information requested in step 1 unless Endpoint B explicitly allows additional location information.

## Editor’s Note: It is FFS if a third step can occur in which Endpoint A sends additional Provide Location Information messages to Endpoint B.

### 5.3.3	Location Information Delivery procedure

The Location Information Delivery procedure allows Endpoint A to provide unsolicited location information to Endpoint B.  The procedure is shown in Figure 5.3.3-1.

Figure 5.3.3-1: SLPP Location Information Delivery procedure

1.	Endpoint A sends a Provide Location Information message to Endpoint B to transfer location information.

## Editor’s Note: It is FFS if a second step can occur in which Endpoint A sends additional Provide Location Information messages to Endpoint B.

### 5.3.4	Reception of SLPP Request Location Information

### 5.3.5	Transmission of SLPP Provide Location Information

## 5.3a	Procedures related to Server UE Selection

### 5.3a.1	General

The purpose of the procedures that are grouped together in this clause is to enable Endpoint A to indicate selection of Endpoint B as a server UE, and to enable Endpoint B to indicate acceptance of the selection. Endpoint A may be a target UE or an anchor UE. Endpoint B may be a server UE only.

### 5.3a.2	Server UE Selection Indication procedure

The Server UE Selection Indication procedure is shown in Figure 5.3a.2-1.

Figure 5.3a.2-1: SLPP Server UE Selection Indication procedure

1.	Endpoint A sends a Server UE Selection Notification message to Endpoint B to indicate that Endpoint A has selected Endpoint B as a server UE. The Server UE Selection Notification message may contain information about candidate anchor UEs.

2.	Endpoint B sends a Server UE Selection Confirm message to Endpoint A to confirm or deny that Endpoint B can accept the role of a server UE for Endpoint A.

### 5.3a.4	Transmission of SLPP Server UE Selection Notification

### 5.3a.5	Reception of SLPP Server UE Selection Confirm

## 5.4	Error Handling Procedures

## 5.5	Abort Procedure