3GPP TSG-RAN WG2 Meeting #123bis	R2-2310206

Xiamen, China, Oct. 9th-13th, 2023

Agenda Item:	7.16.2.1

Source: 	China Unicom

Title:  	Discussion on the AI Functional Framework

Document for:	Discussion

Introduction

Since there are only 2 remaining meetings in RAN2 for this Study Item, in order to finalize the study in RAN2 in due time, the Rapporteur has listed some open issues that need special attention.

In this paper, these issues are discussed, specifically regarding functional framework and UE capability/ applicability-related reporting.

Discussion

- 2.1	Functional framework

In RAN2#122 meeting, RAN2 had a discussion on functional architecture. The main blocks/functions of functional framework have already been discussed and agreed online, the agreed functional framework consists of model training, inference, management, data collection and model storage. But there is no consensus on the name and definition of the arrows among different blocks/functions yet. To resolve this issue, [Post122][059][AIML] email discussion was launched to collect companies’ views. The summary document of that email discussion had been discussed in RAN2#123 meeting, but  there was still no agreements about this issue. The notes are as follows.

R2-2308913 [Post122][059][AIML]: on functional framework, topics to discuss, and FFSs Ericsson discussion Rel-18 FS_NR_AIML_air Chair summary of discussion: - A number of companies want to elaborate the figure so it can show applicability in different scenarios/cases - Multiple companies comment that whether boxes and arrows are dashed, whether things are optional in some scenarios/cases, is not important for this figure. It fullfills sufficient purpose the way it is, and it is also not useful to have FFSes. - Chair: nothing agreeable from this discussion. - Chair comment: We could of course consider removing the word model from the data/information flow ‘Model selection/(de)activation/switching/fallback’ as this seems to add confusion. Noted

Besides, the identified open issues regarding functional framework (in [AIML] R2-23XXXXX - Open Issues - Rapporteur Insights) includes the following:

- Functional framework - Considering RAN2#123’s discussion, is the current version stable? - Do we really need anything else? (e.g., separate figures for model-ID-based LCM and functionality-based LCM) On this the Rapporteur’s suggestion would be to agree that there is no need to consider separate figures for model-ID- and functionality-based LCM. And in case of real need, RAN2 can address these aspects in the TR / text description.

- For this issue, we also think there is no need to consider separate figures for model-ID- and functionality-based LCM. However, precisely because of this, it is necessary to use more general terminology  for the names of arrows in functional framework (e.g.,  the arrow from Management to Model Training could use “management output” instead of “Performance Feedback/ Retraining Request”, the arrow from Management to Model Inference could use “management Instruction ” instead of “Model selection/ (de)activation/ switching/ fallback”).

Proposal 1: The arrows in functional framework figure should use more general terminology.

Proposal 2: The arrow from Management to Model Training could use “management output” instead of “Performance Feedback / Retraining Request” .

Proposal 3:  The arrow from Management to Model Inference could use “management Instruction ” instead of Model selection/(de)activation/switching/fallback.

- 2.2	UE capability and applicable

To gather comments on the topic of applicability conditions and UE capability,  [AT123][001][AIML] email discussion was launched during RAN2#123 meeting. The discussion was not smooth, especially regarding applicability conditions. The agreements  are as follows.

- AIML algorithm for a certain use case may be tailored towards and applicable to certain scenarios/location/configuration/deployment etc. AIML algorithm may be updated, e.g. by model change (these are observations): - RAN2 assumes that for UE-side AIML, the UE may inform the RAN about applicability conditions of AIML algorithm(s) available to the UE, to support RAN control (e.g. activation/deactivation/switching). - The procedure for UE reporting of AIML applicability conditions is FFS.

According to the agreements, there are some issues mentioned in the summary report of [AT123][001][AIML] email discussion (R2-2309202) that have not been agreed upon. The identified open issues regarding UE capability reporting & Applicability-related reporting (in [AIML] R2-23XXXXX - Open Issues - Rapporteur Insights) are as follows.

- UE capability reporting & Applicability-related reporting - These aspects were first addressed during RAN2#123 - For capabilities: can we take on some of the proposals from [AT123][001][AIML]? - For applicability; - Is the issue clear? - Is there any terminology roadblock? Do we need to clarify differences between terms? - Can we agree on certain mechanisms to address the issue? If so, which? The Summary report of [AT123][001][AIML] UE capability and applicability conditions in R2-2309202, included some proposals (e.g., P1 and P2) with full support on how to proceed with the UE capability reporting for AIML for PHY use cases. The Rapporteur suggest quickly agreeing to those as it would allow to clarify some aspects discussed by RAN1 in the TR and some related RAN2 discussion.

For capability,  we agree to the proposals (P1 and P2) on how to proceed with the UE capability reporting for AIML for PHY use cases. For applicability, if the two terms are different, the differences between the two terms should be clarified as it allow to clarify which issues regarding applicability should be discussed and addressed in RAN2. If the two terms are the same, RAN2 should follow RAN1, using “additional conditions” instead of “applicability conditions”.

Proposal 4: If the two if the two terms are different, the differences between these two terms should be clarified as it allow to clarify which issues regarding applicability should be discussed and addressed in RAN2. If the two terms are the same, RAN2 should follow RAN1, using “additional conditions” instead of “applicability conditions”

For reporting mechanisms, it should be discussed separately per model-ID based LCM and functionality based LCM, because applicability conditions can be used to assist model identification in model-ID based LCM, while it may be used to assist function identification in functionality based LCM. For detailed metric(s) of applicability conditions, it is fine to agree that RAN2 may study whether any RAN2 impact case by case only after RAN1 made concrete conclusion on metrics of applicability conditions.

Proposal 5: For reporting mechanisms, it should be discussed separately per model-ID based LCM and functionality based LCM.

Summary

RAN2 is kindly asked to discuss the following proposals:

Proposal 1: The arrows in functional framework figure should use more general terminology.

Proposal 2: The arrow from Management to Model Training could use “management output” instead of “Performance Feedback / Retraining Request” .

Proposal 3:  The arrow from Management to Model Inference could use “management Instruction ” instead of Model selection/(de)activation/switching/fallback.

Proposal 4: If the two terms are different, the differences between these two terms should be clarified. If the two terms are the same, RAN2 should follow RAN1, using “additional conditions” instead of “applicability conditions ”.

Proposal 5: For reporting mechanisms, it should be discussed separately per model-ID based LCM and functionality based LCM.

References

- R2_123_ChairNotes_EOM

- [AIML] R2-23XXXXX - Open Issues - Rapporteur Insights

- [AT123][001][AIML]UE capability and applicability conditions (Apple)