3GPP TSG-RAN WG2 Meeting #123bis	                                                        R2-2310220

Xiamen, China, October 9th – 13th, 2023

Agenda item:			7.2.1

Source:					Intel Corporation

Title:						SLPP TP on ASN.1 part

Document for:	 	Discussion and decision

# 1	Introduction

As mentioned in [Post123][POS] Remaining open issues (CATT), so far for TS 38.355, the content of clause 6	 ASN.1 part is not completed.

In this document, Rapporteur provides the Text proposal on the ASN.1 part.

# 2	Discussion

Following were listed in the open issue list for TS 38.355.

| 6.2 SLPP messages | To complete 6.2 SLPP messages - - Request Capabilities; - - Provide Capabilities; - - Request Assistance Data; - - Provide Assistance Data; - - Request Location Information; - - Provide Location Information; - - Abort; - - Error. | Yes | Rapporteur will provide TP on this, e.g. Change method A/B/C to SL AoA, SL TDOA, SL RTT, also add the abbreviation on them, complete the location request, RequestAssistanceData , Abort , Error, etc. See Rapporteur TP in [5] |
| --- | --- | --- | --- |

Considering the TP on “Change method A/B/C to SL AoA, SL TDOA, SL RTT” has been provided in the email discussion [401]. Here we only focus the messages not covered by the TP in [401], e.g. RequestCapabilities, ProvideCapabilities, RequestAssistanceData, ProvideAssistanceData, About and Error messages .

So far C1 extension is used in the TS 38.355 which is to follow LPP. However it is not used in RRC specification for message level. Even in LPP, the extension was not used. Therefore we propose to follow RRC style, i.e. remove C1 extension from message level. Also a lateNonCriticalExtension is added in the IE level.

In addition in LPP, extension mark “…”is used under ENUMERATE which will cause more bits when add extension. Therefore Rapporteur suggests not use this style in SLPP, and could leave spare if needed.

Proposal 1: Follow RRC style, remove C1 extension from message level and add a lateNonCriticalExtension under message IE.

Proposal 2: Follow RRC style, not use extension mark “…” under ENUMERATE, spare could be used instead.

Proposal 3: RAN2 to endorse the Text proposal on ASN.1 part as baseline.

# 3	Conclusion

Based on the discussion above, we have following proposals:

Proposal 1: Follow RRC style, remove C1 extension from message level and add a lateNonCriticalExtension under message IE.

Proposal 2: Follow RRC style, not use extension mark “…” under ENUMERATE, spare could be used instead.

Proposal 3: RAN2 to endorse the Text proposal on ASN.1 part as baseline.

# 4	Text proposal

### 6.2.2	Message definitions

#### –	RequestCapabilities

-- ASN1START

-- TAG-REQUESTCAPABILITIES-START

RequestCapabilities ::= SEQUENCE {

criticalExtensions      CHOICE {

requestCapabilities    RequestCapabilities-IEs,

criticalExtensionsFuture    SEQUENCE {}

}

}

RequestCapabilities-IEs ::= SEQUENCE {

commonIEsRequestCapabilities         OCTET STRING    OPTIONAL, -- Containing CommonIEsRequestCapabilities

method-SL-AoA-RequestCapabilities    OCTET STRING    OPTIONAL, -- Containing Method-SL-AoA-RequestCapabilities

method-SL-RTT-RequestCapabilities    OCTET STRING    OPTIONAL, -- Containing Method-SL-RTT-RequestCapabilities

method-SL-TDOA-RequestCapabilities   OCTET STRING    OPTIONAL, -- Containing Method-SL-TODA-RequestCapabilities

lateNonCriticalExtension             OCTET STRING    OPTIONAL,

nonCriticalExtension                 SEQUENCE {}     OPTIONAL

}

-- TAG-REQUESTCAPABILITIES-STOP

-- ASN1STOP

#### –	ProvideCapabilities

-- ASN1START

-- TAG-PROVIDECAPABILITIES-START

ProvideCapabilities ::= SEQUENCE {

criticalExtensions      CHOICE {

provideCapabilities     ProvideCapabilities-IEs,

criticalExtensionsFuture    SEQUENCE {}

}

}

ProvideCapabilities-IEs ::= SEQUENCE {

commonIEsProvideCapabilities         OCTET STRING    OPTIONAL, -- Containing CommonIEsProvideCapabilities

method-SL-AoA-ProvideCapabilities    OCTET STRING    OPTIONAL, -- Containing Method-SL-AoA-ProvideCapabilities

method-SL-RTT-ProvideCapabilities    OCTET STRING    OPTIONAL, -- Containing Method-SL-RTT-ProvideCapabilities

method-SL-TDOA-ProvideCapabilities   OCTET STRING    OPTIONAL, -- Containing Method-SL-TDOA-ProvideCapabilities

lateNonCriticalExtension             OCTET STRING    OPTIONAL,

nonCriticalExtension        SEQUENCE {}     OPTIONAL

}

-- TAG-PROVIDECAPABILITIES-STOP

-- ASN1STOP

#### –	RequestAssistanceData

-- ASN1START

-- TAG-REQUESTASSISTANCEDATA-START

RequestAssistanceData ::= SEQUENCE {

criticalExtensions        CHOICE {

requestAssistanceData     RequestAssistanceData-IEs,

criticalExtensionsFuture  SEQUENCE {}

}

}

RequestAssistanceData-IEs ::= SEQUENCE {

commonIEsRequestAssistanceData         OCTET STRING    OPTIONAL, -- Containing CommonIEsRequestAssistanceData

method-SL-AoA-RequestAssistanceData    OCTET STRING    OPTIONAL, -- Containing Method-SL-AoA-RequestAssistanceData

method-SL-RTT-RequestAssistanceData    OCTET STRING    OPTIONAL, -- Containing Method-SL-RTT-RequestAssistanceData

method-SL-TDOA-RequestAssistanceData   OCTET STRING    OPTIONAL, -- Containing Method-SL-TDOA-RequestAssistanceData

lateNonCriticalExtension             OCTET STRING    OPTIONAL,

nonCriticalExtension                   SEQUENCE {}     OPTIONAL

}

-- TAG-REQUESTASSISTANCEDATA-STOP

-- ASN1STOP

#### –	ProvideAssistanceData

-- ASN1START

-- TAG-PROVIDEASSISTANCEDATA-START

ProvideAssistanceData ::= SEQUENCE {

criticalExtensions        CHOICE {

provideAssistanceData     ProvideAssistanceData-IEs,

criticalExtensionsFuture    SEQUENCE {}

}

}

ProvideAssistanceData-IEs ::= SEQUENCE {

commonIEsProvideAssistanceData         OCTET STRING    OPTIONAL, -- Containing CommonIEsProvideAssistanceData

method-SL-AoA-ProvideAssistanceData    OCTET STRING    OPTIONAL, -- Containing Method-SL-AoA-ProvideAssistanceData

method-SL=RTT-ProvideAssistanceData    OCTET STRING    OPTIONAL, -- Containing Method-SL-RTT-ProvideAssistanceData

method-SL-TDOA-ProvideAssistanceData   OCTET STRING    OPTIONAL, -- Containing Method-SL-TDOA-ProvideAssistanceData

lateNonCriticalExtension             OCTET STRING    OPTIONAL,

nonCriticalExtension                   SEQUENCE {}     OPTIONAL

}

-- TAG-PROVIDEASSISTANCEDATA-STOP

-- ASN1STOP

#### –	RequestLocationInformation

-- ASN1START

-- TAG-REQUESTLOCATIONINFORMATION-START

RequestLocationInformation ::= SEQUENCE {

criticalExtensions             CHOICE {

requestLocationInformation     RequestLocationInformation-IEs,

criticalExtensionsFuture    SEQUENCE {}

}

}

RequestLocationInformation-IEs ::= SEQUENCE {

commonIEsRequestLocationInformation         OCTET STRING    OPTIONAL, -- Containing CommonIEsRequestLocationInformation

method-A-RequestLocationInformation         OCTET STRING    OPTIONAL, -- Containing Method-A-RequestLocationInformation

method-B-RequestLocationInformation         OCTET STRING    OPTIONAL, -- Containing Method-B-RequestLocationInformation

method-C-RequestLocationInformation         OCTET STRING    OPTIONAL, -- Containing Method-C-RequestLocationInformation

lateNonCriticalExtension                    OCTET STRING    OPTIONAL,

nonCriticalExtension                        SEQUENCE {}     OPTIONAL

}

-- TAG-REQUESTLOCATIONINFORMATION-STOP

-- ASN1STOP

#### –	ProvideLocationInformation

-- ASN1START

-- TAG-PROVIDELOCATIONINFORMATION-START

ProvideLocationInformation ::= SEQUENCE {

criticalExtensions             CHOICE {

provideLocationInformation    ProvideLocationInformation-IEs,

criticalExtensionsFuture    SEQUENCE {}

}

}

ProvideLocationInformation-IEs ::= SEQUENCE {

commonIEsProvideLocationInformation         OCTET STRING    OPTIONAL, -- Containing CommonIEsProvideLocationInformation

method-A-ProvideLocationInformation         OCTET STRING    OPTIONAL, -- Containing Method-A-ProvideLocationInformation

method-B-ProvideLocationInformation         OCTET STRING    OPTIONAL, -- Containing Method-B-ProvideLocationInformation

method-C-ProvideLocationInformation         OCTET STRING    OPTIONAL, -- Containing Method-C-ProvideLocationInformation

lateNonCriticalExtension                    OCTET STRING    OPTIONAL,

nonCriticalExtension                        SEQUENCE {}     OPTIONAL

}

-- TAG-PROVIDELOCATIONINFORMATION-STOP

-- ASN1STOP

#### –	Abort

-- ASN1START

-- TAG-ABORT-START

Abort ::= SEQUENCE {

criticalExtensions    CHOICE {

abort                 Abort-IEs,

criticalExtensionsFuture    SEQUENCE {}

}

}

Abort-IEs ::= SEQUENCE {

CommonIEsAbort             CommonIEsAbort  OPTIONAL,

lateNonCriticalExtension   OCTET STRING    OPTIONAL,

nonCriticalExtension       SEQUENCE {}     OPTIONAL

}

-- TAG-ABORT-STOP

-- ASN1STOP

#### –	Error

-- ASN1START

-- TAG-ERROR-START

Error ::= CHOICE {

criticalExtensions    CHOICE {

error                 Error-IEs,

criticalExtensionsFuture    SEQUENCE {}

}

}

Error-Ies ::= SEQUENCE {

CommonIEsError              CommonIEsError  OPTIONAL,

lateNonCriticalExtension    OCTET STRING    OPTIONAL,

nonCriticalExtension        SEQUENCE {}     OPTIONAL

}

-- TAG-ERROR-STOP

-- ASN1STOP

## 6.3	SLPP information elements

### 6.3.1	Common information elements

#### –	CommonIEsAbort

The CommonIEsAbort carries common IEs for an Abort SLPP message Type.

-- ASN1START

-- TAG-COMMONIESABORT-START

CommonIEsAbort ::= SEQUENCE {

AbortCause        ENUMERATED {

undefined,

stopPeriodicReporting,

}

}

-- TAG-COMMONIESABORT-STOP

-- ASN1STOP

CommonIEsAbort field descriptions

abortCause This IE defines the request to abort an ongoing procedure. The abort cause 'stopPeriodicReporting' should be used by the location server to stop any ongoing location reporting configured as periodicalReporting or triggeredReporting in the CommonIEsRequestLocationInformation.

#### –	CommonIEsError

The CommonIEsError carries common IEs for an Error SLPP message Type.

-- ASN1START

-- TAG-COMMONIESERROR-START

CommonIEsError ::= SEQUENCE {

ErrorCause         ENUMERATED {

undefined,

slppMessageHeaderError,

slppMessageBodyError,

incorrectDataValue,

}

}

-- TAG-COMMONIESERROR-STOP

-- ASN1STOP

CommonIEsError field descriptions

errorCause This IE defines the cause for an error. 'slppMessageHeaderError' and 'slppMessageBodyError' is used if a receiver is able to detect a coding error in the SLPP header (i.e., in the common fields) or SLPP message body respectively. 'incorrectDataValue' is used if a receiver receives an incorrect data value.