3GPP TSG-RAN WG2 Meeting #123bis	R2-2310224

# Xiamen, China, October 9-13, 2023

Agenda item:	7.4.4

Source: 	China Telecom

Title:	Discussion on open issues of CHO with candidate SCGs

WID/SID:	NR_Mob_enh2-Core

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting, CHO including target MCG and candidate SCGs related issues have been widely discussed and much progress has been made. However, there are still some open issues and stage-3 details left for further discussion. Some agreements or working assumptions on the configuration and execution of CHO with candidate SCG(s) are listed as below.

Agreements in RAN2#123 UE does not remove the configuration for CHO including target MCG and candidate SCG configuration automatically when SCG is to be released. Working assumptions in RAN2#123 R2 assumes for this R18 feature that the UE does not need to continue conditional reconfiguration evaluation for CHO with Candidate SCG(s) upon initiating SCG failure information procedure R2 assumes for this R18 feature that the evaluation of the execution conditions for CHO with Candidate SCG(s) do not need to continue once PSCell change is triggered.

In this contribution, we would like to share our opinions on the following open issues of CHO with candidate SCGs based on the agreements reached in previous RAN2 meetings.

- Confirm the working assumptions made in last RAN2 meeting.

- Whether the legacy CHO recovery mechanism applies to the configuration for CHO with candidate SCG(s).

- Whether UE should remove the configuration for CHO with candidate SCG(s) when PSCell changes.

- Whether to prioritize the execution of CHO with candidate SCG, if both PCell for CHO only or CHO including target MCG and target SCG, and the PCell and the associated PSCell for CHO with candidate SCG(s) is triggered.

- Discussion

Stop CHO with candidate SCG(s) evaluation upon SCG failure

Regarding whether to stop CHO with candidate SCG(s) evaluation upon SCG failure, RAN2 made the assumption that the UE does not need to continue conditional reconfiguration evaluation for CHO with candidate SCG(s) upon initiating SCG failure information procedure in last RAN2 meeting.

Based on the current mechanism, the evaluation of CPC or CPA shall be stopped upon initiating the SCG failure information procedure. To follow the legacy behaviour, the UE shall stop the evaluation of CPC or CPA conditions configured in Rel-18 CHO with candidate SCG(s) configuration. As per the agreements made in RAN2#122 meeting, only when BOTH conditions (CHO+CPC/CPA) are met will the UE perform the CHO with candidate SCG(s), then the execution condition will never be met in this case. But if there is an available CHO-only or Rel-17 CHO with SCG configuration, the evaluation of CHO conditions is not affected. Thus, we think the working assumption is quite straightforward and aligned with the legacy mechanism and the previous RAN2 agreement.

Proposal 1: RAN2 confirms that for this R18 feature the UE does not need to continue conditional reconfiguration evaluation for CHO with Candidate SCG(s) upon initiating SCG failure information procedure.

CHO recovery for CHO with candidate SCG(s)

In last RAN2 meeting, the issue of whether the legacy CHO recovery mechanism applying to CHO with candidate SCG(s) was discussed with no consensus reached. Generally, we tend to agree the motivation of enabling the legacy CHO recovery mechanism also to apply for CHO with candidate SCG(s), but the details may need further discussion.

Based on the agreements reached in RAN2#122 meeting, RAN2 confirms that CHO with candidate SCG(s) can only be executed when both conditions (CHO+CPA/CPC) are met. However, the legacy CHO recovery mechanism is based on cell reselection criteria for a suitable cell, so only the quality of PCell will be evaluated which is not aligned with the principle used in Rel-18 CHO with candidate SCG(s). In this sense, we think the legacy CHO recovery mechanism could not apply to the Rel-18 configuration for CHO with candidate SCG(s) directly. When a candidate PCell with candidate SCG(s) is selected during RRC re-establishment procedure, if there is an available CHO-only or Rel-17 CHO with SCG configuration associated to the selected PCell, then the CHO recovery mechanism could be conducted. Otherwise, it is better not to conduct the CHO recovery towards the selected PCell with candidate SCG(s), which may cause SCG addition failure since only the quality of PCell is evaluated.

Proposal 2: If there is an available CHO-only or Rel-17 CHO with SCG configuration associated to the selected PCell with a Rel-18 CHO with candidate SCG(s) configuration, then the CHO recovery mechanism could be applied to CHO-only configuration or Rel-17 CHO with SCG configuration.

Handling of CHO with candidate SCG(s) when PSCell changes

In last RAN2 meeting, the issue of whether to continue evaluating the execution conditions once PSCell change is triggered was discussed and a working assumption was made that the evaluation of execution conditions for CHO with candidate SCG(s) do not need to continue once PSCell change is triggered.

From the UE’s perspective, the key point on whether to continue the evaluation may depend on whether the configuration is still valid or not after the PSCell change. It is possible that the candidate SCG configuration is the delta configuration based on the source SCG. When PSCell is changed, the conditional reconfiguration will be invalid. Thus, it is much safer for the UE to stop evaluating the execution conditions upon the trigger of PSCell change.

Proposal 3: RAN2 confirms that for this R18 feature that the evaluation of the execution conditions for CHO with Candidate SCG(s) do not need to continue once PSCell change is triggered.

Regarding how to handle the configuration for CHO with candidate SCG(s) in this PSCell change case, we think it is similar to the case of SCG release and the same principle could be followed. When SCG is to be released, RAN2 has already agreed that UE does not remove the configuration for CHO including target MCG and candidate SCG configuration automatically. Whether to remove the configuration could rely on the NW reconfiguration. Similarly, when PSCell change is triggered, the UE does not remove the configuration for CHO including target MCG and candidate SCG configuration automatically. And it could be up to the NW control to release the configuration in this case.

Proposal 4: When PSCell change is triggered, the UE does not remove the configuration for CHO including target MCG and candidate SCG configuration automatically.

Prioritization of CHO with candidate SCG execution

Another issue to be addressed in RAN2 is whether to prioritize the execution of CHO with candidate SCG, if both PCell for CHO only or CHO including target MCG and target SCG, and the PCell and the associated PSCell for CHO with candidate SCG(s) is triggered.

We understand the main motivation of introducing CHO with candidate SCG(s) in Rel-18 is to have a relatively good enough PSCell together with the target PCell after HO, so that the service data rate and UE experience could be guaranteed. As per the agreements made in previous RAN2 meeting, only when BOTH conditions (CHO+CPC/CPA) are met will the UE perform the CHO with candidate SCG(s). Comparing to CHO only or CHO including target MCG and target SCG, the execution of CHO with candidate SCG could help the UE access to a PCell together with a more reliable PSCell, which could provide better service performance and user experience. Thus, we propose to prioritize CHO with candidate SCG execution.

Proposal 5: Prioritize the execution of CHO with candidate SCG, if both PCell for CHO only or CHO including target MCG and target SCG, and the PCell and the associated PSCell for CHO with candidate SCG(s) is triggered.

- Conclusion

In this contribution, we discuss some remaining issues on CHO with candidate SCG(s) in Rel-18. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: RAN2 confirms that for this R18 feature the UE does not need to continue conditional reconfiguration evaluation for CHO with Candidate SCG(s) upon initiating SCG failure information procedure.

Proposal 2: If there is an available CHO-only or Rel-17 CHO with SCG configuration associated to the selected PCell with a Rel-18 CHO with candidate SCG(s) configuration, then the CHO recovery mechanism could be applied to CHO-only configuration or Rel-17 CHO with SCG configuration.

Proposal 3: RAN2 confirms that for this R18 feature that the evaluation of the execution conditions for CHO with Candidate SCG(s) do not need to continue once PSCell change is triggered.

Proposal 4: When PSCell change is triggered, the UE does not remove the configuration for CHO including target MCG and candidate SCG configuration automatically.

Proposal 5: Prioritize the execution of CHO with candidate SCG, if both PCell for CHO only or CHO including target MCG and target SCG, and the PCell and the associated PSCell for CHO with candidate SCG(s) is triggered.

- Reference

RP-22xxxxx	Report of 3GPP TSG RAN WG2 meeting #122, 123

3GPP TS 38.331 17.0.0: "NR; Radio Resource Control (RRC) protocol specification".