3GPP TSG-RAN WG2 Meeting #123bis                      	                                   R2-2310255

Xiamen, China, October 9th – 13th, 2023

Agenda Item:	7.5.3

Source:	CMCC

Title:	Discussion on the DRX enhancement

WID/SID:	             NR_XR_enh-Core

Document for:	Discussion

Introduction

In the RAN2#123bis meeting, the XR-specific power saving issue has been discussed as below:

The maximum value of the counter (NSFN) is 2^16 = 65536. 8. Network sets DRX reference SFN (drx-ReferenceSFN) to either 0 or 512, in the same way as in Rel-16 IIoT. 9. Use the following option (option A): both the counter NSFN and the DRX reference SFN drx-ReferenceSFN are added to the DRX formula. NSFN is initialized to 0. The maximum value of the counter (NSFN) is 2^16 – 1 = 65535.

Therefore, we provide our views on the DRX enhancements for XR in this document.

Discussion

  - Definition of DRX formula

- In [1], the suggested formula used for the Short DRX cycle and the Long DRX cycle was listed below:

1> if the Short DRX cycle is used for a DRX group, and floor ([(SFN × 10) + subframe number] modulo (drx-ShortCycle)) = floor ((drx-StartOffset) modulo (drx-ShortCycle)): 2> start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe. 1> if the Long DRX cycle is used for a DRX group, and floor ([(SFN × 10) + subframe number] modulo (drx-LongCycle)) = drx-StartOffset:

- The drx-ShortCycle and the drx-LongCycle in formula also includes the non-integer value of DRX periodicity.  The DRX short cycle is defined for the power saving case and the DRX long cycle could be used in more power saving efficient case when the data is not frequently transmitted. Therefore both the DRX long and short cycle based on rational numbers should be supported for the non-integer value of DRX periodicity.

- Proposal 1: Both the DRX long and short cycle based on rational numbers are supported for the non-integer value of DRX periodicity

- The floor operation in above suggested formula [1] is used to round down the result of MOD operation since the divisor of MOD operation is a rational number. The floor and MOD operation in above suggested formula [1] is left to the UE implementation and rounding error caused by the MOD and floor operation should be controlled by the UE.

- Proposal 2: The floor and MOD operation in DRX formula is left to the UE implementation and rounding error caused by the MOD and floor operation should be controlled by the UE.

  - DRX Cycle Based On Rational Numbers

In the RAN2 meeting #122, the following agreements have been achieved and the rational numbers solution of DRX cycle issue was selected:

- #122: Define DRX cycle based on rational numbers. Inform RAN1/4 about this and ask them to indicate if this causes issues in their specifications.

The typical frame rate for XR traffic described in [2] and [3] varies from 15 to 120 frames per second. The average packet arrival periodicity of XR is given by the inverse of the frame rate for XR traffic. Since the typical frame rates consist of [15,30,60,90,120] frames per second, the rational numbers could be introduced for the supported periodicity of XR, including [200/3, 100/3, 50/3,100/9,25/3] in milliseconds value.

Observation 1:The frame rate for XR traffic described in [2] and [3] varies from 15 to 120 frames per second.

Proposal 3: The rational numbers could be introduced for the supported periodicity of XR, including [200/3, 100/3, 50/3,100/9,25/3] in milliseconds value.

  - Definition of the Start Offset

- In [4], the allowed range of drx-LongCycleStartOffset were listed for each drx-LongCycle as below:

drx-LongCycleStartOffset CHOICE { ms10 INTEGER(0..9), ms20 INTEGER(0..19), ms32 INTEGER(0..31), ms40 INTEGER(0..39), ms60 INTEGER(0..59), ms64 INTEGER(0..63), ms70 INTEGER(0..69), ms80 INTEGER(0..79), ms128 INTEGER(0..127), ms160 INTEGER(0..159), ms256 INTEGER(0..255), ms320 INTEGER(0..319), ms512 INTEGER(0..511), ms640 INTEGER(0..639), ms1024 INTEGER(0..1023), ms1280 INTEGER(0..1279), ms2048 INTEGER(0..2047), ms2560 INTEGER(0..2559), ms5120 INTEGER(0..5119), ms10240 INTEGER(0..10239) },

- In [1], the suggested formula used for the Long DRX cycle based on  rational numbers was listed below for example:

1> if the Long DRX cycle is used for a DRX group, and floor ([(SFN × 10) + subframe number] modulo (drx-LongCycle)) = drx-StartOffset:

- Reusing the legacy structure for the IE drx-LongCycleStartOffset, the start offset of drx-onDurationTimer could also listed for each DRX long cycle based on rational numbers.  Based on the suggested formula in [1], the maximum value of the start offset of drx-onDurationTimer for each non-integer DRX long cycle is result of the floor operation of the non-integer DRX long cycle. The following table lists the definition of the start offset of drx-onDurationTimer for non-integer DRX periodicity:

drx-LongCycleStartOffset CHOICE { ms200-3 INTEGER(0..66), ms100-3 INTEGER(0..33), ms50-3 INTEGER(0..16), ms100-9 INTEGER(0..11), ms25-3 INTEGER(0..8), },

Proposal 4: Reusing the legacy structure for the IE drx-LongCycleStartOffset, the start offset of drx-onDurationTimer could also listed for each DRX long cycle based on rational numbers.

Proposal 5: The maximum value of the start offset of drx-onDurationTimer for each non-integer DRX long cycle is result of the floor operation for the non-integer DRX long cycle.

Conclusions

- In this contribution, we analyse the open issues of XR DRX enhancements Following is our observations and proposals.

- Proposal 1: Both the DRX long and short cycle based on rational numbers are supported for the non-integer value of DRX periodicity.

- Proposal 2: The floor and MOD operation in DRX formula is left to the UE implementation and rounding error caused by the MOD and floor operation should be controlled by the UE.

Observation 1:The frame rate for XR traffic described in [2] and [3] varies from 15 to 120 frames per second.

Proposal 3:The rational numbers could be introduced for the supported periodicity of XR, including [200/3, 100/3, 50/3,100/9,25/3] in milliseconds value.

Proposal 4: Reusing the legacy structure for the IE drx-LongCycleStartOffset, the start offset of drx-onDurationTimer could also listed for each DRX long cycle based on rational numbers.

Proposal 5: The maximum value of the start offset of drx-onDurationTimer for each non-integer DRX long cycle is result of the floor operation for the non-integer DRX long cycle.

References

- R2-2304709 DRX enhancements for XR, Qualcomm Incorporated, MediaTek, CATT, vivo, NEC, Meta

- 3GPP TR 38.835 V1.0.0, Study on XR enhancements for NR(Release 18), 2022-12

- 3GPP TR 26.926 V2.0.0 (2023-09), Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems;(Release 18)

- 3GPP TS 38.321 V17.5.0 Medium Access Control (MAC) protocol specification (Release 17), 2023-06