# 3GPP TSG-WG2 Meeting #123bis                                R2-2310257

# Xiamen, China, Oct, 2023

# Source: 			CMCC

# Title:  			Remaining issues on service continuity

# Agenda item:	7.9.3

# Document for: 	Discussion

Introduction

In this contribution, we discuss some remaining open issues on service continuity.

Discussion

The order of RRCReconfiguration of Relay UE and Remote UE

As shown in Fig.1, gNB sends an RRCReconfiguration message to the target L2 U2N Relay UE in step 3 and sends the RRCReconfiguration message to the L2 U2N Remote UE in step 4, during intra-gNB i2i path switching [1]. The order of the two steps are still FFS.

We understand the similar issue has been discussed in RAN3, about whether the reconfiguration to target relay UE should be performed before the reconfiguration to remote UE. Some companies think that there is a certain probability that failure will happen when gNB reconfigures the target relay UE. Such configuration failure may impact the subsequent procedure, because the reconfiguration to remote UE includes the information of candidate relay UE, e.g. the L2 U2N relay UE ID. So, it will be more effective to wait for the step 3. However, some other companies understand, it is very rare case that the reconfiguration to candidate relay UE failure, when the relay UE is in RRC_CONNECTED. gNB selects it among the candidate relay UEs and make decision to path switch, hence, the uu condition can be reliable. The restriction may bring some latency to path switching.

From our perspective, the order of the two steps can be changeable, which is based on network implementation. However, considering the other path switching procedures in 16.12.6 in TS38.300, alignment with the existing flow charts can void more controversy.

Proposal 1: The order of RRCReconfiguration of Relay UE and Remote UE should be fixed.

Figure 1: Procedure for L2 U2N Remote UE intra-gNB switching from indirect to indirect path

Conclusions

According the above discussion we have following proposal:

Proposal 1: The order of RRCReconfiguration of Relay UE and Remote UE should be fixed.

Reference

[1] R3-2309339, draft stage2 running CR(TS38.300) for SL relay enhancements, LG