# 3GPP TSG-WG2 Meeting #123bis                                   R2-2310259

# Xiamen, China, Oct, 2023

Source: 			CMCC

Title:  			Remaining open issues on multi-path

Agenda item:	7.9.4

Document for: 	Discussion

Introduction

The overall procedure for multi-path scenario 1 and scenario 2 have be discussed and concluded. Some remaining open issues are list in rapporteur’s summary contribution [1]. In this contribution, we will discuss the remaining issues.

Discussion

Scenario 1

MP Indirect path change trigger event

Indirect path change is supported in scenario 1. As shown in the TS 38.300 running CR, the measurement and report event for indirect path change are still FFS. In our understanding, the motivation of multi-path is to improve the transmission reliability and throughput. So, indirect path change can be triggered when the serving relay UE become worse and can not satisfy the transmission requirement, or/and a candidate relay UE is better than an threshold. The condition of the relay UE can be evaluated with considering the PC5 link based on the measurement and the uu link based on the gNB implementation. In addition, the UL transmission requirements can be considered, for example, high priority data arriving or UL data exploded. However, the UL transmission requirements could be reflected to gNB with buffer status report to some extent.

Proposal 1: MP indirect path change trigger events for scenario 1 include:

- Serving relay UE becomes worse than configured threshold 1;

- Candidate relay UE becomes better than configured threshold 2;

- UL high priority data arriving or UL data exploded.

Proposal 2: ‘Serving relay UE becomes worse than configured threshold 1’ and ‘Candidate relay UE becomes better than configured threshold 2’ can be reported via measurement reporting message.

Proposal 3: ‘UL high priority data arriving or UL data exploded’ can be reported via UE Assistance Information.

Scenario 2

MP Indirect path addition/change trigger event

For scenario 2, indirect path addition and change are supported in RAN2.

As RAN2 concluded that the association between remote UE and relay UE are pre-configured and static. The remote UE can report candidate UE identifications to gNB for indirect path addition and change. Those identifications of relay UEs are used to inform gNB about the association between the remote UE and its associated relay UE. Such reporting does not trigger the indirect path addition for scenario 2. The trigger events for indirect path addition and change for scenario 2 should be discussed in RAN2.

gNB can decide whether to add the indirect path immediately unless it is necessary after it receives the relay UE identification from the remote UE. The gNB makes the decision based on gNB implementation or based on remote UE assistance information report. For scenario 2, the UE to UE link is non-3gpp connection, no measurement event is supported in the non-3gpp link. However, the link condition can be informed to gNB, for example, the link condition of serving relay UE becomes worse, or candidate relay UE condition becomes better. In addition, the UE assistance information, for example, the buffer status report, PHR, UL high priority data arriving or UL data exploded. gNB can perform indirect path addition or change based on the UE assistance information. The above relay UE assistance information can be included in the UE Assistance Information.

Proposal 4: Indirect path addition and change should be triggered by separate signaling, rather than by reporting of the relay UE’s ID, for scenario 2.

Proposal 5: MP indirect path change trigger events for scenario 2 include:

- UE to UE link condition change;

- UL high priority data arriving or UL data exploded;

Proposal 6: ‘UE to UE link condition change’ and ‘UL high priority data arriving or UL data exploded’ can be reported via UE Assistance Information.

Conclusions

According the above discussion we have following proposals:

For scenario 1:

Proposal 1: MP indirect path change trigger events for scenario 1 include:

- Serving relay UE becomes worse than configured threshold 1;

- Candidate relay UE becomes better than configured threshold 2;

- UL high priority data arriving or UL data exploded.

Proposal 2: ‘Serving relay UE becomes worse than configured threshold 1’ and ‘Candidate relay UE becomes better than configured threshold 2’ can be reported via measurement reporting message.

Proposal 3: ‘UL high priority data arriving or UL data exploded’ can be reported via UE Assistance Information.

For scenario 2:

Proposal 4: Indirect path addition and change should be triggered by separate signaling, rather than by reporting of the relay UE’s ID, for scenario 2.

Proposal 5: MP indirect path change trigger events for scenario 2 include:

- UE to UE link condition change;

- UL high priority data arriving or UL data exploded;

Proposal 6: ‘UE to UE link condition change’ and ‘UL high priority data arriving or UL data exploded’ can be reported via UE Assistance Information.

Reference

[1] [post123] [relay] Remaining open issues(LG).