# 3GPP TSG-RAN WG2 Meeting #123bis	   R2-2310260

# Xiamen, CN, Oct. 9th - Oct. 13th, 2023

# Agenda Item:	7.13.5

# Source:	CMCC

# Title:	SON enhancement for NR-U

# Document for:	Discussion and Decision

1	Introduction

In RAN2#123, we discussed SON/MDT for NR-U and the following agreements are made.

Agreements:

1  Introduce a new field that counts the number of preamble transmissions blocked by LBT for the last BWP selected for the RA procedure. FFS how to solve the issue of no preamble transmission attempts transmitted in a selected beam due to LBT blockage.

2  All the BWPs (including the first one) in which the UE experienced the consistent UL LBT failure, prior to the successful completion of the RA, are included in the RA-Report.

3  UE log the RA-InformationCommon in the RLF-Report when the RLF cause is lbtFailure and the UE was performing random access at the moment of RLF.

4  The UE logs the following information in the SHR:

- The ra-InformationCommon including the new Rel.18 information (i.e. the number of UL LBT failures during HO, the info on the multiple BWPs in which consistent UL LBT failures was triggered), if T304 triggering conditions is fulfilled.

- FFS: The RSSI measurements of the frequencies associated to the source/target/neighbouring cells, if the measRSSI-ReportConfig is configured for those frequencies.

5  BWPs information included in the RA-Report can be included, within the list of attempted BWP(s), in chronological order of BWP selection.

In this contribution we provide our opinions about the FFS issue for NR-U.

2	Discussion

2.1 RLF report:

In RAN2#123, the remaining issues for RLF report are including as following:

FFS1: BWP information should be included in the RLF-Report for all the BWPs in which the UE detected the consistent UL LBT failure, right before the RLF/HOF.

FFS2:	RAN2 agrees to include the RSSI measurements of the frequency associated to the source PCell in the RLF report in case of HOF, if the measRSSI-ReportConfig is configured for such frequency.

FFS3:	RAN2 agrees to include in the RLF-Report the available RSSI measurement results of the frequencies associated to the neighbouring cells, if the measRSSI-ReportConfig is configured for such frequencies.

FFS4:	If Proposal 8 is not agreed, RAN2 to discuss if the UE logs in the RLF-Report the latest measured RSSI of the frequency associated to the target cell in case of HOF, if measRSSI-ReportConfig is configured for such frequency.

FFS5:	UE logs lbt-FailureRecoveryConfig in the RLF-Report only upon re-establishment procedure failure.

FFS6:	For the sake of progress and alignment with RAN3, RAN2 confines the discussion on the configuration index to the SHR and SPR discussion.

For BWP information, RAN2 has agreed to log the BWP information (e.g. locationAndBandwidth, subcarrierSpacing and absoluteFrequencyPointA) in RA-InformationCommon, and leaving log the BWP information in the RLF-Report as FFS.

As UE may experience some consistent LBT failures in other BWPs before RLF occurs, when the lbt-FailureRecoveryConfig is configured, it’s beneficial for the network to understand which BWP is suffered from consistent LBT failures. And the network may derive the load information for different BWPs for configuration optimization, such as the BWP in which UE detected consistent LBT failures may experience heavy load and network may not configure too many UE/traffic to the BWP which is reported with consistent LBT failures. And also, since RA-InformationCommon IE is included in both RLF report and SHR, the BWP information is naturally added to the RLF report and SHR. So, it’s reasonable to log the BWP information of all the BWPs, in which the UE detected the consistent UL LBT failures before RLF/HOF, in RLF report.

Observation 1: As RA-InformationCommon IE is included in both RLF report and SHR, the BWP information is naturally added to the RLF report and SHR.

Proposal 1: UE logs the BWP information in RLF report for all the BWPs in which the UE detected the consistent UL LBT failures.

Then, for the RSSI measurements, as we already agreed to log the available RSSI measurement in the RLF report in RAN2#122, the FFS 2-4 listed in the above are discussed to handle the alignment of legacy spec. For the source PCell frequency, including the RSSI measurements may help to analyze the NR-U channel condition, even if the RLF is not caused by LBT issue, it can be used to analyze the channel quality of NR-U channel. For the neighbour cell frequencies, the RSSI measurements can be used to compare with the RSSI measurements of source PCell frequency, and analyze the quality of source cell frequency and target cell frequency. For example, in case of HOF, compare the RSSI measurements between the source PCell frequency and neighbouring cells frequencies, the network can know whether the HOF is due to the handover to the wrong cell or handover at wrong time point, and optimize the handover configuration based on the analysis. So, we propose to include the RSSI measurements for frequencies associated to the source PCell and neighbouring cells in RLF report.

Observation 2: Log RSSI measurements for frequencies associated to the source PCell and neighbouring cells in RLF report is beneficial for the network to analyze the source cell quality and whether the HOF is due to the handover to the wrong cell or handover at wrong time point (by comparing the RSSI measurements between the source PCell frequency and neighbouring cells frequencies).

Proposal 2: Include the RSSI measurements for frequencies associated to the source PCell and neighbouring cells in RLF report, if the measRSSI-ReportConfig is configured for such frequencies.

Then, for whether to log the lbt-FailureRecoveryConfig in RLF report, according to the RAN3 LS, the network can retrieve the UE context or the configuration used for the UE in the last serving node based on the information provided from the UE (last serving PCell ID and C-RNTI). But it’s only available when UE attempts re-connection and reports the failure right after connection failure. If the failure information is fetched from the UE hours after the failure, whether the UE configuration can be retrieved is depended on the RAN implementation.

Besides, in RAN2#123, the configuration index to the SHR and SPR from RAN3 had been discussed. From our understanding, the configuration index is related to SHR and SPR, whether it’s benefit to RLF report is unclear. And as the lbt-FailureRecoveryConfig contains only two parameters, including lbt-FailureInstanceMaxCount and lbt-FailureDetectionTimer, we think that UE logs lbt-FailureRecoveryConfig in RLF report may not result in too much signaling overhead.

Observation 3: The configuration index discussed in RAN3 is related to SHR and SPR, whether it’s benefit to RLF report is unclear. And as lbt-FailureRecoveryConfig contains only two parameters, logging lbt-FailureRecoveryConfig in RLF report may not result in too much signaling overhead.

So, in order to enable the network obtain the UE configuration, it is simple for UE to log lbt-FailureRecoveryConfig in the RLF-Report upon re-establishment procedure failure.

Proposal 3: UE logs lbt-FailureRecoveryConfig in the RLF-Report upon re-establishment procedure failure.

2.2 SHR

In RAN2#123, the remaining issues for SHR are including as following:

FFS7:	Agree logging the LBT information of the source cell at the moment of performing HO. FFS the details (e.g., number of LBT failure or consistent LTB failure, etc.)

FFS: The RSSI measurements of the frequencies associated to the source/target/neighbouring cells, if the measRSSI-ReportConfig is configured for those frequencies.

For SHR, one of the remaining issues is whether to log the LBT information of the source cell at the moment of performing HO. Upon initiating HO, UE will not perform LBT in source cell, but in the target cell. The benefit of logging the LBT information of source cell is unclear when performing HO. For source cell, the RA report and RLF report can be used to log LBT information. So, we propose to log the LBT information, e.g. number of LBT failures, for target cell.

Proposal 4: Log the LBT information in SHR, e.g. number of LBT failures, for target cell.

Besides, for RSSI, logging RSSI measurements for frequencies associated to the source/target/neighbouring cells is beneficial for the network to analyze the HO performance, e.g. whether HO is near failure, and optimize the HO configuration based on these information. Hence, the RSSI measurements of the frequencies associated to the source/target/neighbouring cells are proposed to be logged in SHR.

Proposal 5: Log the RSSI measurements of the frequencies associated to the source/target/neighbouring cells in SHR.

2.3 RA report

In RAN2#123, the remaining issues for RA report are including as following:

FFS8:	 how to solve the issue of no preamble transmission attempts transmitted in a selected beam due to LBT blockage.

As discussed in the previous meeting, when all the preambles are blocked by LBT, no preamble will be transmitted. But in the current spec, the range of numberOfPreamblesSentOnSSB-r16/numberOfPreamblesSentOnCSI-RS-r16 is 1~200, if no preamble is transmitted, no proper value can be set. One potential solution is to introduce a new field to indicate all the preambles are blocked by LBT, and we think that this may reduce the impact on the existing field of numberOfPreamblesSentOnSSB-r16/numberOfPreamblesSentOnCSI-RS-r16.

Proposal 6: Introduce a new field to indicate all the preambles are blocked by LBT.

3	Conclusion

Observation 1: As RA-InformationCommon IE is included in both RLF report and SHR, the BWP information is naturally added to the RLF report and SHR.

Proposal 1: UE logs the BWP information in RLF report for all the BWPs in which the UE detected the consistent UL LBT failures.

Observation 2: Log RSSI measurements for frequencies associated to the source PCell and neighbouring cells in RLF report is beneficial for the network to analyze the source cell quality and whether the HOF is due to the handover to the wrong cell or handover at wrong time point (by comparing the RSSI measurements between the source PCell frequency and neighbouring cells frequencies).

Proposal 2: Include the RSSI measurements for frequencies associated to the source PCell and neighbouring cells in RLF report, if the measRSSI-ReportConfig is configured for such frequencies.

Observation 3: The configuration index discussed in RAN3 is related to SHR and SPR, whether it’s benefit to RLF report is unclear. And as lbt-FailureRecoveryConfig contains only two parameters, logging lbt-FailureRecoveryConfig in RLF report may not result in too much signaling overhead.

Proposal 3: UE logs lbt-FailureRecoveryConfig in the RLF-Report upon re-establishment procedure failure.

Proposal 4: Log the LBT information in SHR, e.g. number of LBT failures, for target cell.

Proposal 5: Log the RSSI measurements of the frequencies associated to the source/target/neighbouring cells in SHR.

Proposal 6: Introduce a new field to indicate all the preambles are blocked by LBT.

4	Reference

[1] 	RP-221825, “New WID on Further Enhancement of Data Collection for SON_MDT in NR standalone and MR-DC”, CMCC