3GPP TSG-RAN WG2 Meeting #123bis    	                           R2-2310268

Xiamen, China, 9 - 13 October 2023

Agenda item:	7.4.3

Source:	CMCC

Title:	Discussion on remaining open issues for subsequent CPAC

WID/SID:	NR_Mob_enh2-Core

Document for:	Discussion and Decision

# 1	Introduction

In last meeting [1], RAN2 discussed the MN/SN-initiated subsequent CPAC procedure, release of configuration and security issue. As indicated by Rapporteur in RAN2 reflector, here is the list of remaining open issues for subsequent CPAC.

Subsequent CPAC in NR-DC - Reference SCG configuration (Optionality FFS). Assume as for LTM Reference configuration may be empty. - FFS whether MCG configuration is included. - FFS RRC model for the reference configuration. - Stage-3 RRC details - FFS on whether to release intra-SN subsequent CPAC configuration autonomously by UE upon SCG release. - FFS on how to configure the subsequent execution conditions. - FFS on whether to keep candidate evaluation upon transmission of the SCG Failure Information message to the MN. - FFS on following LTM on the complete configuration generation/application related part. - FFS on whether to rely on the full configuration procedure as specified in 5.3.5.11 or new complete configuration procedure when the UE applies a complete configuration. - FFS on the IE/message to transfer the reference configuration - FFS on the IE/message to transfer the candidate info/execution conditions for subsequent execution condition preparation between MN and SN.. - Stage-2 signaling details - FFS whether to support subsequent CPA - FFS if to release the conditional reconfigurations in other cases, e.g. at intra-MN PCell change, SCG release, etc. - FFS whether to support the coexistence of subsequent CPAC and SCG deactivation - FFS which node initially generates the reference configuration in subsequent CPAC - FFS how/when to generate execution conditions for subsequent CPC - FFS whether to support the coexistence of legacy CPA/CPC and subsequent CPAC - FFS whether to have a separate signaling flow for subsequent CPAC procedure - Detailed aspects of the security solution - FFS on how to determine the inter-SN mobility for security key update for subsequent CPAC - FFS on how UE select sk-Counter for key update for subsequent CPAC. - FFS on whether and how to inform the selected sk-Counter to MN/SN. - FFS on the handling of used sk-Counter. - FFS on whether to release the configured sk-Counters if subsequent CPAC configuration is released. - FFS on the maximum number of maxSecurityCellSet-r18 and maxSK-Counter-r18. - FFS on the granularity of SK counter configuration update/release, e.g. per SK-counter list or per SK-counter. - FFS how/whether the NW can configure same SPCell/SN to be include in R17/R16 CPC and R18 CPC.

Considering some open issues on stage-2 signaling details and security aspects have been discussed in post email discussion [2] and [3], we will focus on other open issues in this contribution.

- 2 Discussion

  - SCG Failure handling

In legacy CPA/CPC procedure, the UE stops evaluating the CPA/CPC execution condition upon transmission of the SCGFailureInformation message to the MN as specified in TS 37.340 [4]. And in R18, RAN2 agreed that the UE does not need to continue conditional reconfiguration evaluation for CHO with Candidate SCG(s) upon initiating SCG failure information procedure. For subsequent CPAC, we think the same principle can be applied.

7.7 SCG/MCG failure handling <omitted> In case of CPA/CPC, upon transmission of the SCGFailureInformation message to the MN, the UE stops evaluating the CPA/CPC execution condition. The UE is not required to continue measurements for candidate PSCell(s) for execution condition upon transmission of the SCGFailureInformation message to the MN.

Proposal 1: The UE is not required to continue evaluation for subsequent CPAC upon transmission of the SCGFailureInformation message to the MN.

  - Reference configuration

In RAN2#121bis-e meeting, RAN2 agreed to aim at following similar design as LTM for the reference configuration for subsequent CPAC.

For the reference configuration for SCG Selective Activation, aim at following similar design as LTM.

During the discussion on LTM, the following agreements were achieved:

- Reference config can be empty

- In the RRC procedures, the candidate delta configuration is applied on top of the reference configuration to form a complete candidate configuration when the UE receives the LTM configuration (before the LTM cell switch). UE implementation can postpone that step to the reception of the LTM cell switch command. FFS Discuss early vs late compliance check.

- In the RRC procedures, the complete candidate configuration is applied and replacing the current UE configuration (at the time of reconfiguration execution/cell switch), by a RRC reconfiguration procedure that makes replacements of configuration but doesn’t necessarily reset MAC, RLC or PDCP. FFS whether we can rely on a modified version of the reconfiguration procedure with fullconfig flag set. FFS how to make sure the procedures work in case the LTM candidate configuration is a complete configuration.

- Whether the Reference configuration is a complete configuration or not is up to the network implementation.

- Reference configuration + LTM candidate configuration (in combination) has to be a complete configuration.

- The reference configuration is always explicitly signalled (not automatically derived from any other config, e.g. current).

- A UE capability to indicate the support of the reference configuration is introduced. If reference configuration is not supported then complete candidate configurations has to be used.

Based on the above agreements, the UE can always have a complete configuration for candidate PSCell via reference configuration + candidate configuration. And when the reference configuration is empty, the network can guarantee the candidate configuration is a complete configuration since the reference configuration is always explicitly signaled.

For legacy full configuration procedure, fullConfig indicated the RRC signaling is full or delta which is only applied for HO case, and this field is absent if the RRCReconfiguration message contains SCG configuration in another RRCReconfiguration as specified in TS 38.331 [5].

fullConfig Indicates that the full configuration option is applicable for the RRCReconfiguration message for intra-system intra-RAT HO. For inter-RAT HO from E-UTRA to NR, fullConfig indicates whether or not delta signalling of SDAP/PDCP from source RAT is applicable. This field is absent if any DAPS bearer is configured or when the RRCReconfiguration message is transmitted on SRB3, and in an RRCReconfiguration message for SCG contained in another RRCReconfiguration message (or RRCConnectionReconfiguration message, see TS 36.331 [10]) transmitted on SRB1.

Based on the above analysis, we understand the applicable scenario of full configuration procedure is different from the new complete configuration, and it will not impact the new complete configuration procedure. Therefore, we support to follow the LTM mechanism on the complete configuration generation/application for subsequent CPAC.

Proposal 2: For subsequent CPAC, follow the similar mechanism on the complete configuration generation/application as LTM.

- The reference configuration is always explicitly signaled (not automatically derived from any other config, e.g. current).

- The reference configuration can be empty or a complete configuration which is up to network implementation.

- Reference configuration + candidate configuration (in combination) has to be a complete configuration.

- In the RRC procedures, the complete candidate configuration is applied and replacing the current UE configuration (at the time of subsequent CPAC execution).

- A UE capability to indicate the support of the reference configuration is introduced. If reference configuration is not supported then complete candidate configurations has to be used.

During the post email discussion [2], there are two understanding on the previous RAN2 agreement “Assume for now that there is only one reference configuration” :

- - Understanding1: There is only one reference configuration for MN/SN initiated inter-SN SCPAC and SN-initiated intra-SN SCPAC.

- - Understanding2: There is only one reference configuration for each scenario, i.e., one reference configuration for MN/SN initiated inter-SN SCPAC and one reference configuration for SN-initiated intra-SN SCPAC.

Similar to legacy CPC, the MN is not involved for SN-initiated intra-SN subsequent CPAC, however, MN is always involved for MN/SN-initiated subsequent inter-SN procedure. Therefore, it is straightforward and simple to introduce separate reference configuration for MN/SN initiated inter-SN SCPAC and SN-initiated intra-SN SCPAC. Moreover, SN can be responsible to generate the reference configuration for intra-SN SCPAC without MN involvement.

Proposal 3: There is only one reference configuration for MN/SN initiated inter-SN SCPAC and one reference configuration for SN-initiated intra-SN SCPAC.

Proposal 4：The SN is responsible to generate the reference configuration for intra-SN SCPAC without MN involvement.

  - Security

For SK-counter configuration update/release, we prefer the granularity is per SK-counter list since it is simple and has minor spec impacts. When the UE receives the updated SK-counter list, the UE replaces the complete list stored in its variable.

Proposal 5: The granularity of SK-counter configuration update/release is per SK-counter list.

Another issue is the UE behavior if all the configured SK-counters has been used out. In our understanding, we think the UE should stop subsequent CPAC evaluation for the corresponding SN since the UE cannot access this SN again due to lack of unused SK-counter in this scenario. If we leave it to NW implementation to ensure there is at least a unused SK-counter, that means the NW should always update the SK-counter list when there is no unused SK-counter which will limit the NW flexibility. Thus, we prefer to specify UE behaviour in this case.

Proposal 6: The UE should stop subsequent CPAC evaluation for the corresponding SN if all the configured SK-counters has been used out.

# 3	Conclusion

Here are the observations and proposals for NR-DC with selective activation cell of groups.

SCG failure handling:

Proposal 1: The UE is not required to continue evaluation for subsequent CPAC upon transmission of the SCGFailureInformation message to the MN.

Reference configuration:

Proposal 2: For subsequent CPAC, follow the similar mechanism on the complete configuration generation/application as LTM.

- The reference configuration is always explicitly signaled (not automatically derived from any other config, e.g. current).

- The reference configuration can be empty or a complete configuration which is up to network implementation.

- Reference configuration + candidate configuration (in combination) has to be a complete configuration.

- In the RRC procedures, the complete candidate configuration is applied and replacing the current UE configuration (at the time of subsequent CPAC execution).

- A UE capability to indicate the support of the reference configuration is introduced. If reference configuration is not supported then complete candidate configurations has to be used.

Proposal 3: There is only one reference configuration for MN/SN initiated inter-SN SCPAC and one reference configuration for SN-initiated intra-SN SCPAC.

Proposal 4：The SN is responsible to generate the reference configuration for intra-SN SCPAC without MN involvement.

Security:

Proposal 5: The granularity of SK-counter configuration update/release is per SK-counter list.

Proposal 6: The UE should stop subsequent CPAC evaluation for the corresponding SN if all the configured SK-counters has been used out.

# 4	Reference

- Draft_R2_123_meeting_report_v1

- R2-230XXXX [Post123][054][feMob] Stage-2 Signalling Open Issues and Running CR 37340 (ZTE)

- R2-230XXXX [Post123][046][feMob] subsequent CPAC security (Nokia)

- TS 37.340 Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2

- TS 38.331 NR; Radio Resource Control (RRC)