3GPP TSG-RAN WG2 Meeting #123bis	 R2-2310285

Xiamen, China, October 9th – 13th, 2023

Agenda item:		7.21.3

Source:	NEC

Title:	Discussion on the remaining UP issues for CE

Document for:		Discussion and Decision

1 Introduction

In the last RAN2 meeting[1], regarding fallback, the following agreements were reached.

Agreements => Each RSRP threshold is configured separately by RRC, which is associated with a repetition number if configured (for each carrier). => Regarding the framework for Msg1 repetition and whether to support fallback from lower number to higher number, Fallback is supported. All repetitions are treated as a single feature, but within the feature, different repetition numbers are treated as different RACH type. => UE selects higher repetition number upon Msg1 retransmission when the number of Msg1 retransmission reaches a configured value. FFS whether we need to also check DL RSRP at the time of switching (can ask RAN1) discuss as part of offline 801. => support fallback from CFRA with Msg1 repetition to 4-step CBRA with Msg1 repetition. Details are FFS.

In the LS from RAN1[2], regarding waveform switching, there was the following agreements.

Agreement For reporting of power headroom information for assumed PUSCH using target waveform different from waveform of actual PUSCH, support the following: - Power headroom information for assumed PUSCH is based on an actual PUSCH transmission. - In case of no actual PUSCH transmission on a serving cell, power headroom information for assumed PUSCH is not supported. - DWS field needs to be configured for at least one DCI format for the BWP of the actual PUSCH, otherwise power headroom information for assumed PUSCH is not supported. - If actual PUSCH transmission is with DFT-S-OFDM waveform, UE computes power headroom information of an assumed PUSCH with CP-OFDM waveform. If actual PUSCH transmission is with CP-OFDM waveform, UE computes power headroom information of an assumed PUSCH with DFT-S-OFDM waveform. - All parameters that are used for the calculation of PCMAX,f,c(i), except waveform, are the same between assumed PUSCH and actual PUSCH. - In case assumed PUSCH transmission is not supported for the parameters that are used for the calculation of PCMAX,f,c(i), power headroom information for assumed PUSCH is not computed or reported. - Power headroom information for assumed PUSCH contains: - PCMAX,f,c(i) of assumed PUSCH - Accounting for applicable MPR, A-MPR and P-MPR for the assumed PUSCH. - If UE reports power headroom information for assumed PUSCH in a PUSCH transmission, legacy PHR is also reported in the same PUSCH transmission. - No consensus in RAN1 if the following applies or not: if UE reports legacy PHR in a PUSCH transmission, power headroom information for assumed PUSCH is also reported. - Note: RAN endorsed the following at RAN#100: “RAN2 will not work on PHR triggering procedure for dynamic waveform switching in Rel-18 UL Coverage enh WI” [RP-231498].

In this contribution, we would like to discuss some remaining issues and RAN1 LS impact on RAN2, and provide our views.

2 Discussion

RACH partition selection

Regarding Msg1 repetition, it has multiple separate RSRP thresholds, which is different with Msg3 repetition. For the application of Msg1 repetition, in our understanding, when DL RSRP is less than the RSRP threshold for higher repetition number, the UE should consider Msg1 repetition with higher repetition numbers are also applicable. Taking Example #2 in [3] as an example, if DL RSRP is less than the RSRP threshold for Msg1 repetition number 4, Msg1 repetition number 8 is also considered applicable. Then both partition 1 and partition 2 in this example are applicable.

# Example 2:

              - Partition 1: Msg1 repetition (Num_2 + Num_4 +Num_8)

              - Partition 2: RedCap + Msg1 repetition (Num_8)

              - Feature priority: RedCap > Msg1 repetition

The main reason is that if the UE consider the lower repetition number to be applicable, it is very possible to transmit unsuccessfully due to the lower repetition number. One may argue that the higher repetition number can cause RACH resource wasted. In our understanding, compared to RACH resource waste, transmission failure can have a big impact on the probability of successful access. So we think wasting some RACH resources are deserved.

Proposal 1: When DL RSRP is less than the RSRP threshold for a higher repetition number (x), the UE should consider Msg1 repetition with higher repetition numbers (> x) are also applicable.

For the above example,  both partition 1 and partition 2 are applicable. Since the priority of RedCap is higher than that of Msg1 repetition, partition 2 can be selected. Then Msg1 repetition number 8 is used. When one RACH partition is selected, some rules for selecting Msg1 repetition number needs to be considered. If only one Msg1 repetition number within the selected RACH partition, this repetition number can be selected. If there are multiple Msg1 repetition numbers and there is suitable repetition number, the suitable Msg1 repetition number can be selected. Otherwise, the lowest applicable repetition number is selected within these applicable repetition numbers.

Proposal 2: When one RACH partition is selected, the following rules for the selection of Msg1 repetition number are used:

- If there is only one repetition number within this RACH partition, this repetition number is selected.

- If there are multiple repetition numbers and there is suitable repetition number, the suitable repetition number is selected.

- Otherwise, the lowest applicable repetition number within these applicable repetition numbers is selected.

RSRP check for fallback

According to the agreements in the last meeting, fallback from lower repetition number to higher repetition number is supported. The UE selects higher repetition number when the number of Msg1 retransmission number reaches a threshold. Regarding whether DL RSRP is checked or not is FFS. In our understanding, it should be similar to LTE, only whether Msg1 transmission number reaches a threshold is checked when fallback is performed. Besides, DL RSRP cannot change so fast. Hence, there is no necessary to check DL RSRP.

Proposal 3: For fallback from lower repetition number to higher repetition number, DL RSRP threshold does not need to be checked.

In the email discussion of UP issues[3], fallback from CFRA with repetition to CBRA with repetition is being discussed, two options are listed as below:

- Option 1: Upon fallback from CFRA with repetition to CBRA with repetition, the UE only selects the RACH resources that associated the same repetition number that indicated for CFRA.

- Option 2: Upon fallback from CFRA with repetition to CBRA with repetition, the UE determines the repetition number and corresponding RACH resource based on DL RSRP. (note: if the UE does not fulfill any Msg1 repetition RSRP threshold, the UE selects the RACH resources for repetition number 2)

Similar to RedCap, RACH resource for CBRA should be selected in the initiation phase of RACH. For CFRA, NW can configure one repetition number based on the reported measurement result. Then for CBRA, the same repetition number may be used. Besides, if the same repetition number is used for fallback form CFRA to CBRA, NW should ensure that the same repetition number should be configured for CBRA.

Proposal 4: Upon fallback from CFRA with repetition to CBRA with repetition, the UE only selects the RACH resources that associated the same repetition number that indicated for CFRA.

PHR

In RAN1, waveform switch between DFT-S-OFDM and CP-OFDM is supported. In order to support it, some assistance information needs to be reported, such as PCMAX,f,c(i) of assumed PUSCH. According to RAN1’s LS, Power headroom information for assumed PUSCH is based on an actual PUSCH transmission, i.e. Power headroom information for assumed PUSCH is reported only when actual PUSCH transmission is performed on this serving cell. If actual PUSCH transmission is with DFT-S-OFDM waveform, UE computes power headroom information of an assumed PUSCH with CP-OFDM waveform. If actual PUSCH transmission is with CP-OFDM waveform, UE computes power headroom information of an assumed PUSCH with DFT-S-OFDM waveform. In general, power headroom information of an assumed PUSCH can be reported together with type 1 PH which is based on a real transmission.

In TS 38.321[4], PHR has the following six formats:

- Single Entry PHR MAC CE

- Multiple Entry PHR MAC CE

- Enhanced Single Entry PHR MAC CE

- Enhanced Multiple Entry PHR MAC CE

- Enhanced Single Entry PHR for multiple TRP MAC CE

- Enhanced Multiple Entry PHR for multiple TRP MAC CE

The above six formats can be used for the different scenarios. In our understanding, waveform switching should be used for these scenarios, such as multiple TRPs. For PHR format, a simple way is to place PCMAX,f,c(i) of assumed PUSCH in these formats. Taking Enhanced Single Entry PHR for multiple TRP MAC CE as an example, PHR format with PCMAX,f,c(i) of assumed PUSCH is shown as in Figure 1.

Figure 1 Enhanced Single Entry PHR for multiple TRP MAC CE

Proposal 5: PCMAX,f,c(i) of assumed PUSCH can be reported together with type 1 PH which is based on a real transmission.

3 Conclusion

In this contribution, we have further discussed the potential RAN2 impacts of multiple PRACH transmissions from UP perspective, all the proposals are summarized below:

Proposal 1: When DL RSRP is less than the RSRP threshold for a higher repetition number (x), the UE should consider Msg1 repetition with higher repetition numbers (>x) are also applicable.

Proposal 2: Once one RACH partition is selected, the following rules are used:

- If there is only one repetition number within this RACH partition, this repetition number is selected.

- If there are multiple repetition numbers and there is suitable repetition number, the suitable repetition number is selected.

- Otherwise, the lowest applicable repetition number within these applicable repetition numbers is selected.

Proposal 3: For fallback from lower repetition number to higher repetition number, DL RSRP threshold does not need to be checked.

Proposal 4: Upon fallback from CFRA with repetition to CBRA with repetition, the UE only selects the RACH resources that associated the same repetition number that indicated for CFRA.

Proposal 5: PCMAX,f,c(i) of assumed PUSCH can be reported together with type 1 PH which is based on a real transmission.

4 References

[1] Draft_R2_123_meeting_report_v1.

[2] R2-2309256_R1-2308477  LS on Details of power headroom information for assumed PUSCH.

[3] R2-230xxxx UP open issues on CE.

[4] TS 38.321 h50.