# s3GPP TSG RAN WG2 Meeting #123b      	                           R2-2310290
Xiamen, China, 9th – 13th Oct., 2023

Agenda item:	7.3.2

Source:	Apple

Title:	Remaining key open issues of Cell DTX / DRX

WID/SID:	Netw_Energy_NR-Core– Release 18

Document for:	Discussion and Decision

# 1 Introduction

- Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3]

- Note: No change for SSB transmission due to cell DTX/DRX.

- Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

The Network Energy saving (NES) WID RP-223540 was agreed in RAN#98-e [1], the WI objective on Cell DTX / DRX is copied below:

By RAN2#123 [2], most major issues were addressed. However, there are still some open issues, including the key open issues summarized by CR Rapporteur [3][4].

In this contribution, we share our view on some key open issues from below aspects:

- Key open issue on RRC configuration

- Key open issue on activation / deactivation of Cell DTX/DRX

- Key open issue on UE MAC behavior

- Alignment between Cell DTX and UE CDRX

# 2 Discussion

## 2.1 key open issues on RRC configuration

We address below RRC issues identified by RRC CR Rapporteur [3]:

Issue 1-5: Whether the start offset (and slot offset) configuration should be the same for cell DTX and cell DRX.

Issue 1-6: Whether standalone cell DRX is needed.

Issue 1-7: Whether the fields in cellDTX-config should be optional or mandatory.

Issue 1-8: Consider a simplification of cell DTX/DRX RRC configuration IE once issues 1-5, 1-6 and 1-7 are resolved.

In RAN2#123 [2], it was agreed that on-duration and cycle parameters are common between Cell DTX and Cell DRX, when both are configured. But it is FFS whether different start offset can be configured for Cell DTX and Cell DRX.

8	On-duration and Cycle parameters are common between cell DTX and DRX, when both are configured.  FFS if we have different start offset configuration for cell DTX and cell DRX

We prefer that Cell DTX and Cell DRX have the same start offset due to below considerations:

- There is no valid use case to allow different start offset.

  - Some concern about TDD was raised in RAN2#123. However, please note that Cell DRX only suspends transmission of SR and CG whose slot offset can be reconfigured to adapt to specific TDD configuration via RRC.

- There is no extra NES gain of different start offset compared with same start offset, because the length of non-active duration of Cell DRX is agreed to be always same as Cell DTX.

- It can simplify UE behaviors and reduce extra standard work to handle corner cases during the non-overlapping duration.

Thus, we propose that start offset configuration is also common between cell DTX and Cell DRX.

Proposal 1: Start offset configuration is also common between Cell DTX and Cell DRX when both are configured.

In RAN2#123 [2], it was also agreed that Cell DTX can be configured without Cell DRX. But it is not clear whether Cell DRX can be configured without Cell DTX.

7	Separate DTX and DRX configuration means that the features can be enabled separately (i.e. Cell DTX can be configured without Cell DRX)

We prefer to disallow Cell DRX can be configured without Cell DTX:

- Its NES benefit is not clear because the power consumption of DL is much higher than UL according to TR38.864 [5].

- The same purpose can be achieved by deconfiguration of SR and CG.

- It may complicate UE behaviors and bring extra standard work for corner case handling.

Thus, we propose:

Proposal 2: Cell DRX can be configured only when Cell DTX is configured (i.e. Cell DRX can't be configured without Cell DTX).

Then, issue 1-7 and issue 1-8 can be resolved by introducing a CellDTXDRX-Config IE which includes mandatory IEs of On-duration, Cycle parameters, start offset, and a 1bit indication on whether it is jointDTX/DRX or DTX. For example:

CellDTXDRX-Config-r18 ::=                  SEQUENCE {

cellDTXDRX-onDurationTimer-r18          CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

}

cellDTXDRX-CycleStartOsffset-r18        CHOICE {

ms10                                INTEGER(0..9),

ms20                                INTEGER(0..19),

ms32                                INTEGER(0..31),

ms40                                INTEGER(0..39),

ms60                                INTEGER(0..59),

ms64                                INTEGER(0..63),

ms70                                INTEGER(0..69),

ms80                                INTEGER(0..79),

ms128                               INTEGER(0..127),

ms160                               INTEGER(0..159),

ms256                               INTEGER(0..255),

ms320                               INTEGER(0..319),

ms512                               INTEGER(0..511),

ms640                               INTEGER(0..639),

ms1024                              INTEGER(0..1023),

ms1280                              INTEGER(0..1279),

ms2048                              INTEGER(0..2047),

ms2560                              INTEGER(0..2559),

ms5120                              INTEGER(0..5119),

ms10240                             INTEGER(0..10239)

}

cellDTXDRX-SlotOffset-r18               INTEGER (0..31),

jointcellDTXDRX-r18                       ENUMERATED {true}

}

Proposal 3: Simplify Cell DTX/DRX RRC configuration by introducing a CellDTXDRX-Config IE which includes mandatory IEs of On-duration, Cycle parameters, start offset, and a 1bit indication on whether it is jointDTX/DRX or DTX.

## 2.2 Key open issues on activation / deactivation of Cell DTX/DRX

We first address below open issue identified by RRC CR Rapporteur [3]:

Issue 1-2: Explicit vs implicit activation based on RRC and relation to L1 activation (how UE derives NW L1 capability and is there a need of explicit RRC indication that the NW will use L1).

In RAN1 LS (R1-2306552) on RRC parameters [6], it has included a field of dci-Format2-X:

| Parameter name in the spec | Description | Per (UE, cell, TRP, …) |
| --- | --- | --- |
| dci-Format2-X | If configured, the UE monitors the DCI format 2_X with CRC scrambled by cellDTRX-RNTI according to TS 38.213, clause [10.X]. | Per UE/serving cell group |

Our understanding is that it is corresponding to "higher layer signalling" which is highlighted in below RAN1#114 agreement:

Agreement

- An information block field of DCI format 2_X is variable size either 1 or 2 bits.

  - Higher layer signaling configures whether the activation/deactivation of cell DTX and/or cell DRX is indicated in DCI format 2_X for a serving cell.

    - If both cell DTX and cell DRX are configured for a serving cell,

      - 1st bit corresponds to activation/deactivation of cell DTX configuration, and

      - 2nd bit corresponds to activation/deactivation of cell DRX configuration,

    - otherwise, the 1 bit corresponds to the configured cell DTX or cell DRX configuration.

  - Note: this does not imply there may be separate higher layer signaling to enable L1 signaling based activation/deactivation for a cell DTX and/or cell DRX configuration. Signaling design is up to RAN2.

We think this IE is sufficient to serve as indication on whether Cell DTX/DRX is activated via DCI or RRC. No need to introduce a new separate RRC IE for same purpose.

Proposal 4: Reuse the absence/presence status of the IE dci-Format2-X in RAN1 LS (R1-2306552) as indication on whether Cell DTX/DRX is activated via DCI or RRC. No need to introduce a new separate RRC IE for same purpose.

Then, we address below open issue identified by RRC CR Rapporteur [3]:

Issue 1-3: Is deactivation by RRC performed only by deconfiguration, or should we have an explicit RRC deactivation without deconfiguration option (to later activate the same configuration by RRC).

Similar to CG type 1, we think it is sufficient to rely on RRC deconfiguration. The benefit to introduce explicit RRC deactivation is not clear. Meanwhile, if the NW prefers the explicit deactivation / activation, it can configure the UE to use DCI 2-X.

Proposal 5: Deactivation of Cell DTX/DRX by RRC is via deconfiguration (i.e. no need to introduce explicit RRC deactivation).

Finally, we address below open issue identified by MAC CR Rapporteur [4]:

Issue 1: How RRC-based activation works when L1-based cell DTX/DRX activation is configured One issue that was discussed in the running CR email discussion is default activation/deactivation status if Cell DTX/DRX is configured but no L1 signalling is received though configured: - Alt-1: Activation is default status: - Immediate activation if cell DTX is configured, which is more aligned with the R2 early agreement for a baseline. - if NW don't want Cell DTX/DRX to work immediately, it needs to explicitly send a DCI with deactivation to the UE. - Alt-2: Deactivation is default status: - The UE regards Cell DTX/DRX as deactivated only after a DCI with activation is received. - Can be useful if gNB intends to activate cell DTX by L1 signaling but it has not sent L1 activation indication yet (e.g. due to loading consideration). The draft running CR thus captures the following editor’s note: Editor’s note: FFS if/how RRC-based activation works when L1-based cell DTX/DRX activation is configured.

We think both Alt-1 and Alt-2 can work. Between them, we prefer Alt-2 because Alt-2 can provide more flexibility to the NW and lower latency to the UE. As illustrated in Figure.1, UE 1 stays in the cell before and UE 2 camps in the cell from time T1 during non-active duration of Cell DTX/DRX. With Alt-2, even if gNB configures UE 2 Cell DTX / DRX in time T3, it can still complete all NAS and RRC procedure of UE 2 within non-active duration because UE 2 regards the Cell DTX/DRX as deactivated and thereby keeps its TX/RX ON. As comparison, if Alt-1 is adopted:

- Either gNB postpones UE 2's NAS and RRC procedure to next active duration of Cell DTX/DRX (i.e. after T5) because UE 2 regards the Cell DTX/DRX as activated and thereby turns off its TX/RX;

- Or gNB also sends group common DCI in T3 to deactivate Cell DTX/DRX to complete NAS and RRC procedure of UE 2. However, in this case, UE 1 will unnecessarily wake up because the DCI is group common.

- Or gNB is forced to send a separate RRC message to configure Cell DTX/DRX after NAS and RRC procedure of UE 2 is complete (e.g. T4 in Figure.1). It restricts NW flexibility, and alleviates the benefit to introduce group common DCI.

Figure. 1 Illustration of benefit of Alt-2

Thus, we propose:

Proposal 6: If the UE is configured with Cell DTX/DRX and to monitor DCI 2-X, it regards Cell DTX/DRX as deactivated until a DCI 2-X with activation is received.

## 2.3 Key open issues on UE MAC behaviour

RAN2#121 [7] left a FFS on whether to introduce DTX UE specific inactivity timer.

- Pattern configuration for cell DRX/DTX is common for Rel-18 UEs in the cell.   FFS whether we have DTX UE specific inactivity timer .  FFS on configuration signaling and stage 3.

In RAN2#123 [2], it was agreed that RAN2 focus on the case where Cell DTX can only be configured when UE CDRX is configured.

11	We focus on the case where DTX in RRC can only be configured when C-DRX is configured.  We will not optimize for the case where C-DRX is not configured.

Meanwhile, running MAC CR [4] has specified the UE to exceptionally monitor PDCCH when retransmission timer is running, during RACH and when SR is pending. We think it is sufficient, and thereby prefer not to introduce any new timer for Cell DTX/DRX, including new RTT / retransmission / inactivity timer.

Proposal 7: Not introduce any new timer (including new RTT / retransmission / inactivity timer) for purpose of extending active time of Cell DTX / DRX pattern, i.e. Cell DTX / DRX pattern is a fixed pattern.

## 2.4 Alignment between Cell DTX and UE CDRX

In RAN2#123 [2], it was agreed that gNB only ensures partial overlapping between UE C-DRX and Cell DTX, where partial overlapping means the cell DTX/DRX and C-DRX periodicity should be multiple of each other.

3	The gNB should ensures that there is at least partial overlapping between UE C-DRX on-duration and cell DTX/DRX on-duration.  It is up to network implementation to ensure the alignment.  We will capture this in stage 2 specification.

Understanding is that alignment means that the cell DTX/DRX and C-DRX periodicity should be multiple of each other.   FFS if we anything needs to be specified in stage 3 (i.e. in IE description)

Obviously, such partial overlapping can't ensure the active duration of Cell DTX always contains active duration of UE CDRX. Actually, 3 possible overlapping durations between Cell DTX and UE CDRX may happen which are illustrated in Figure. 2. Among them, we think UE behaviour in T2 and T3 is clear, i.e. the UE doesn't monitor PDCCH and can't transmit SR/CG if Cell DRX is configured, according to running MAC CR [4] and below agreement made RAN2#121b-e [8].

- The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time.   UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time.   FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

FFS how to deal with retransmissions

However, the UE/gNB behaviour during T1 (i.e. Part of non-active time of UE DRX is overlapped with active duration of Cell DTX) in Figure.2 is not clear because it is active duration of Cell DTX. Thus, further discussion is needed.

Observation 1: The UE/gNB behaviour during T1 is not clear because it is active duration of Cell DTX/DRX which is not covered by RAN2#121b-e agreement.

Figure.2 Illustration of possible overlapping scenarios of Cell DTX and UE CDRX alignment

For T1, we think the reasonable UE behaviour is to follow UE CDRX behaviour (i.e. stop monitoring PDCCH but allows transmission of SR/CG and reception of SPS) by assuming that gNB will not transmit DCI for it (i.e. gNB may transmit DCI for other UEs). We think it is necessary for RAN2 to confirm this understanding:

Proposal 8: During T1 (i.e. non-active time of UE CDRX is overlapped with active duration of Cell DTX/DRX), the UE follows UE CDRX behaviour (i.e. stop monitoring PDCCH but allow transmission of SR/CG and reception of SPS).

One example to capture Proposal 8 in TS 38.321 [4] is provided below:

### 5.x.1 Cell Discontinuous Transmission ...Omit.. For each Serving Cell configured with CellDTX-Config, the MAC entity shall: 1> if cell DTX operation is deactivated for this Serving Cell; or 1> if the Serving Cell is in the cell DTX Active Period and in the active time of the DRX group; or 1> if any drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL (as described in clause 5.7) is running on any Serving Cell in the DRX group; or 1> if ra-ResponseWindow (as described in clause 5.1.4), ra-ContentionResolutionTimer (as described in clause 5.1.5), or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or 1> if a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5); or 1> if a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a): 2> monitor PDCCH on the Serving Cells in this DRX group, as specified in TS 38.213 [6] and other clauses of this specification.

# 3 Conclusion

In this contribution, we further discuss key open issues of Cell DTX / DRX. Our proposals can be found below.

Key open issues on RRC configuration

Proposal 1: Start offset configuration is also common between Cell DTX and Cell DRX when both are configured.

Proposal 2: Cell DRX can be configured only when Cell DTX is configured (i.e. Cell DRX can't be configured without Cell DTX).

Proposal 3: Simplify Cell DTX/DRX RRC configuration by introducing a CellDTXDRX-Config IE which includes mandatory IEs of On-duration, Cycle parameters, start offset, and a 1bit indication on whether it is jointDTX/DRX or DTX.

Key open issues on activation / deactivation of Cell DTX/DRX

Proposal 4: Reuse the absence/presence status of the IE dci-Format2-X in RAN1 LS (R1-2306552) as indication on whether Cell DTX/DRX is activated via DCI or RRC. No need to introduce a new separate RRC IE for same purpose.

Proposal 5: Deactivation of Cell DTX/DRX by RRC is via deconfiguration (i.e. no need to introduce explicit RRC deactivation).

Proposal 6: If the UE is configured with Cell DTX/DRX and to monitor DCI 2-X, it regards Cell DTX/DRX as deactivated until a DCI 2-X with activation is received.

Key open issues on UE MAC behavior

Proposal 7: Not introduce any new timer (including new RTT / retransmission / inactivity timer) for purpose of extending active time of Cell DTX / DRX pattern, i.e. Cell DTX / DRX pattern is a fixed pattern.

Alignment between UE CDRX and Cell DTX/DRX

Observation 1: The UE/gNB behaviour during T1 is not clear because it is active duration of Cell DTX/DRX which is not covered by RAN2#121b-e agreement.

Proposal 8: During T1 (i.e. non-active time of UE CDRX is overlapped with active duration of Cell DTX/DRX), the UE follows UE CDRX behaviour (i.e. stop monitoring PDCCH but allow transmission of SR/CG and reception of SPS).

# 4 References

[1] RP-223540, New WID: Network energy savings for NR, Huawei

[2] RAN2#123, Chair Notes

[3] R2-230xxxx, Running 38.331 CR - Introduction of Network energy savings for NR, Huawei, HiSilicon

[4] R2-230xxxx, Running CR to 38.321 for Network energy savings, InterDigital.

[5] TR 38.864-v1.0.0, Study on Network energy savings for NR, 2022-12.

[6] R1-2306552, Higher layer signalling for Rel-18 NES, Huawei, HiSilicon.

[7] RAN2#121, Chair Notes

[8] RAN2#121b-e, Chair Notes