3GPP TSG-RAN WG2 Meeting #123-bis	R2-2310319

Xiamen, China, October 09-13, 2023

Agenda Item:	[7.17.3] Allowed MUSIM temporary capability restriction

Source: 	Apple Inc

Title:  	Allowed aspects and UE capability for MUSIM temporary capability restriction

Document for:	Discussion and Decision

# Introduction

In the last RAN2#123 meeting the following agreements were made with regards to the signalling procedures for temporary UE capability restriction [1].

1: The UE can indicate that some frequencies (e.g. frequency ranges, bands or BCs) are impacted by NW B so that they are:

- 1) forbidden because of collision

- 2) having restricted (lower) capabilities (e.g. with lower MIMO layer).

4: The restrictions can apply to CA, DC and/or single CC.

5: The UL/DL MIMO layer and/or the UL/DL supported bandwidth restriction (if supported) shall only work for the restricted frequencies for the proactive case.

1	For dual active MUSIM, the UE can indicate the temporary maximum MIMO layers for specific serving cells for both UL and DL.

Maximum MIMO layers/bandwidth restriction is reported per CC ((FFS how we signal this).

FFS whether we support indicating temporary capability restrictions on SRS Tx switching capability. FFS whether this could be already indicated by the MIMO layer restrictions.

In this contribution, we present our views on some of the open items in this topic.

# Discussion

## SRS Tx Switching Capability

The reallocation of RF chains from one SIM instance to another would be the primary motivation for the UE to report reduced UE capability. This is done to accommodate the concurrent connected mode use cases across two SIM instances. In such cases, the UEs SRS capability also gets impacted temporarily as well.

The reduction in the SRS Tx switching capability is also impacted by the reduction in the number of MIMO layers. So, this means that whenever the UEs MIMO layers gets reduced, there is going to be a corresponding impact on the SRS Tx switching capability as well.

Observation 1: Any change to MIMO layer configuration impacts the SRS switching capability on the UE side.

As per [1] there is already an agreement for the UE to indicate restriction on maximum MIMO layers for specific serving cells for both UL and DL. So in cases, when there is a need for the UE to temporarily reduce its UE capability for MUSIM purpose, and if the UE choses to do it by requesting for reduced UL and/or DL MIMO layers, its SRS switching capability is also reduced accordingly. So there is no additional need for the UE to request for reduced capability for SRS switching.

Proposal 1: UE need not indicate additional SRS Tx switching capability restriction over and above MIMO layer restriction.

## UE capability for Proactive and Reactive temporary capability restriction

For both proactive and reactive use cases, it is the UE which determined when and how to request for temporary capability restriction. As a result, UE is best positioned to determine which option to choose whenever the need for temporary UE capability restriction arises. To keep the overall design simple, having a unified framework for capability restriction would be useful.

Observation 2: UE is best informed of when to request for temporary capability restriction.

Observation 3: To keep the design simple, having a unified framework for temporary capability restriction helps.

Hence, a single capability field shall be used for both proactive and reactive approach. The choice of which approach to apply shall be left to UE implementation.

Proposal 2: Choice of Proactive or Reactive temporary UE capability restriction should be left to UE implementation.

Proposal 3: One single capability which indicates that UE can request for temporary capability restriction

# Conclusion

In summary, we have the following observations for prohibit timer design and handling for the temporary UE capability restriction signalling issue for MUSIM UEs.

Observation 1: Any change to MIMO layer configuration impacts the SRS switching capability on the UE side.

Observation 2: UE is best informed of when to request for temporary capability restriction.

Observation 3: To keep the design simple, having a unified framework for temporary capability restriction helps.

Based on the above observation, we have the following proposals for prohibit timer design and handling for the temporary UE capability restriction signalling issue for MUSIM UEs.

Proposal 1: UE need not indicate additional SRS Tx switching capability restriction over and above MIMO layer restriction.

Proposal 2: Choice of Proactive or Reactive temporary UE capability restriction should be left to UE implementation.

Proposal 3: One single capability which indicates that UE can request for temporary capability restriction

# Reference

[1] R2-2308962	Report from session on LTE legacy, XR, QoE and Multi-SIM	Vice Chairman (Nokia)	Report