3GPP TSG-RAN WG2 Meeting #123bis	R2-2310342

Xiamen, China, 9 – 13 October 2023

Agenda item:	6.1.3.1

Source:	Apple Inc, T-Mobile USA, Qualcomm Inc

Title:	Emergency call handling in RedCap in barred cells

WID/SID:	NR_redcap-Core

Document for:	Discussion / Decision

# Introduction

Among the different types of Release 17 RedCap UEs, wearables and other forms of UEs where voice service is offered to the user, also exist. RAN2 introduced the concept of UE barring the cell if the cell does not support a specific set of redcap capabilities that cell wants to the UE to support. While it does make sense to have the UE bar the cell if the UE cannot meet the capability requirements of the cell, for the RedCap case, this needs to be revisited.

In this short paper, we provide reasons for allowing such RedCap UEs to camp on ‘any cell’ state in the urgent cases where the UE needs to offer emergency call services to the user.

# Discussion

## Background on UE barring the cell

Traditionally RAN2 implemented the concept of UE barring the NR cell (which is functional and serving other UEs) in case the UE does not support some “minimum” radio access capabilities that the UE needs to have implemented, in order to sustain a call with that NR cell. The driving reason behind this is that if the UE cannot support these minimum capabilities, the UE cannot get into CONNECTED mode and maintain this state with the cell. Some of the parameters that are considered here are the support of channel bandwidth by the UE, or the support of a particular sub-carrier spacing etc., where many of these are evaluated on the UE support for both uplink and downlink directions.

Correspondingly, even in the case of emergency call, the UE is not expected to try placing the call in these UE barred cells.

TS 38.304 sec 5.3.1 When cell status "barred" is indicated or to be treated as if the cell status is "barred", - The UE is not permitted to select/reselect this cell, not even for emergency calls. - The UE shall select another cell according to the following rule: - If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the MIB: - the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds. - the UE may select another cell on the same frequency if the selection criteria are fulfilled.

Observation 1: Driving reason behind RAN2’s intention of UE barring a functional cell (i.e, serving other UEs) if it cannot support the SIB1 broadcasted requirements (for eg., CH BW, SCS support etc) is that the UE cannot sustain a CONNECTED mode call with the NW in case the UE does not support these.

## Changed dynamics with RedCap

The earlier assumption (from observation 1 in sec 2.1) does not hold very well with the evolution of NR through releases 16 and 17. Specially with the introduction of RedCap devices in Rel-17, the mechanism of UE barring the cell is now used to prevent the UE from accessing a cell that does not/have not implemented some of the features. In other cases, the UE barring the cell is used to prevent certain UEs which may not be efficient in operating in the cell.

In other words, the UE barring the cell concept is now used for other purposes like enabling the NR cell to allow/not allow certain type of UEs based on the NW implementation or based on the efficiency of allowing/not allowing certain UEs.

We do note that cell barring due to other “situations” in the NW (for eg., cell is under maintenance – ‘reserved for operator’) are not considered here, as we are focusing on UE barring the cell based on certain NW ‘filters’.

Observation 2: The UE barring the cell concept is also used for other purposes like enabling the NR cell to allow/not allow certain type of UEs based on the NW implementation or based on the system efficiency of allowing/not allowing certain UEs.

While we agree with this (after all, this has been standardized in NR), we think this line of reasoning needs to be revisited in ‘special’ cases where it is very useful (and in some cases, very likely a regulatory requirement) to allow the UE to access the UE barred cell.

For us the prime use-case is the handling of emergency call in RedCap UEs which offer voice services to the user. In cases where the current UE capability is “able” to sustain a voice call with the cell (assuming that the cell can support this with this UE), we think that this RedCap UE accessing the NR cell in ‘camped any cell selection’ state for initiating the emergency call, should be allowed.

## Further evaluation of handling EM calls

Let’s dive a bit further on why a 3GPP RAN2 discussion would be helpful for this situation. We list the below observations:

Observation 3: While it is possible for a RedCap UE which had to bar the current RedCap supporting NR cell (as the UE could not support the cell required RedCap specific capabilities), to “artificially” advertise the NW required capabilities – just for the purpose of placing the emergency call,  it creates a mis-match at the NW in terms of what the NW thinks the UE supports vs what the UE actually supports, which can result in the call getting dropped!

Observation 4: While the UE can try to find a suitable/acceptable cell in NR,  (and possibly advertise itself as a non-RedCap UE just to get the emergency call to go through), we run the risk of observation 3 – a mis-match between what the NW thinks the UE supports vs what the UE supports, and the risk is even more with this approach, as the likelihood of using a >20MHz BW for a BWP is pretty high!

Observation 5: While the UE can try to find a suitable cell in LTE (one can argue that LTE coverage should be available more widespread than RedCap support), and we do have the advantage of limiting the BW to 20MHz (or less), this means that we are indirectly nudging the UE to perform IRAT selection for handling EM call, and also, we would be not addressing the case where the UE couldn’t find an acceptable cell in LTE! – cases where the current NR cell on which the UE has camped on ‘any cell selection’ state is better and can be used!

We understand that opening up the UE barring cell logic would create discussions and compatibility issues, and we intend to minimize the discussion. So we want to narrow this to the only case of 1Rx/2Rx incompatibility for the below reasons:

- Both UE and the NW are expected to sustain a call with the minimal 1Rx support from the UE, atleast for providing voice services ( i.e, low data rate IMS call).

- While it has been contentious among the 3GPP community on the 1Rx/2Rx support from the devices, the discussion has been on the loss of performance in allowing such UEs and for emergency call, the metric of efficiency is not as important as the ability to allow the user to access the emergency voice service.

  - Also, the occurrence of such calls should be very low, to factor in the inefficiency.

Observation 6: To limit the impact on the specification and implementation, we can limit the cases where the RedCap UE can be allowed to use the NR cell only to one where the UE could not satisfy the 1Rx/2Rx SIB1 requirement as broadcasted in the SIB1 of the RedCap supporting NR Cell.

Observation 6.1: Both UE and the NW are expected to sustain a call with the minimal 1Rx support from the UE, atleast for providing voice services ( i.e, low data rate IMS call).

Observation 6.2: Considering the low occurrences of this, the metric of efficiency is not as important as the ability to allow the user to access the emergency voice service

Based on the above arguments/reasons, we propose that RAN2 agreed to the below.

Proposal 1: A RedCap UE (that provides voice services) that has considered the RedCap supporting NR cell as barred due to not being able to support the SIB1 broadcasted 1Rx/2Rx requirement (from the IEs cellBarredRedCap1Rx-r17 and cellBarredRedCap2Rx-r17), is allowed to consider this NR cell as an acceptable cell for the purpose of placing an emergency call. This is only in the case where the RedCap UE supports all the capabilities based on SIB1 except for the 1Rx/2Rx requirement.

We can propose how to capture this in the 38.304 (and if needed in 38.331) based on the progress made in RAN2-123bis.

# Conclusions

Observation 1: Driving reason behind RAN2’s intention of UE barring a functional cell (i.e, serving other UEs) if it cannot support the SIB1 broadcasted requirements (for eg., CH BW, SCS support etc) is that the UE cannot sustain a CONNECTED mode call with the NW in case the UE does not support these.

Observation 2: The UE barring the cell concept is also used for other purposes like enabling the NR cell to allow/not allow certain type of UEs based on the NW implementation or based on the system efficiency of allowing/not allowing certain UEs.

Observation 3: While it is possible for a RedCap UE which had to bar the current RedCap supporting NR cell (as the UE could not support the cell required RedCap specific capabilities), to “artificially” advertise the NW required capabilities – just for the purpose of placing the emergency call,  it creates a mis-match at the NW in terms of what the NW thinks the UE supports vs what the UE actually supports, which can result in the call getting dropped!

Observation 4: While the UE can try to find a suitable/acceptable cell in NR,  (and possibly advertise itself as a non-RedCap UE just to get the emergency call to go through), we run the risk of observation 3 – a mis-match between what the NW thinks the UE supports vs what the UE supports, and the risk is even more with this approach, as the likelihood of using a >20MHz BW for a BWP is pretty high!

Observation 5: While the UE can try to find a suitable cell in LTE (one can argue that LTE coverage should be available more widespread than RedCap support), and we do have the advantage of limiting the BW to 20MHz (or less), this means that we are indirectly nudging the UE to perform IRAT selection for handling EM call, and also, we would be not addressing the case where the UE couldn’t find an acceptable cell in LTE! – cases where the current NR cell on which the UE has camper on ‘any cell selection’ state is better and can be used!

Observation 6: To limit the impact on the specification and implementation, we can limit the cases where the RedCap UE can be allowed to use the NR cell only to one where the UE could not satisfy the 1Rx/2Rx SIB1 requirement as broadcasted in the SIB1 of the RedCap supporting NR Cell.

Observation 6.1: Both UE and the NW are expected to sustain a call with the minimal 1Rx support from the UE, atleast for providing voice services ( i.e, low data rate IMS call).

Observation 6.2: Considering the low occurrences of this, the metric of efficiency is not as important as the ability to allow the user to access the emergency voice service

Proposal 1: A RedCap UE (that provides voice services) that has considered the RedCap supporting NR cell as barred due to not being able to support the SIB1 broadcasted 1Rx/2Rx requirement (from the IEs cellBarredRedCap1Rx-r17 and cellBarredRedCap2Rx-r17), is allowed to consider this NR cell as an acceptable cell for the purpose of placing an emergency call. This is only in the case where the RedCap UE supports all the capabilities based on SIB1 except for the 1Rx/2Rx requirement.

# References