3GPP TSG-RAN WG2 #123bis	R2-2310382
Xiamen, China, October 2023

Agenda Item:	7.3.2

Source:	OPPO

Title:	Discussion on DTX/DRX mechanism

Document for:	Discussion, Decision

Introduction

This paper will further discuss the cell DTX/DRX, which is one objective of NES WI.

- Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3]

- Note: No change for SSB transmission due to cell DTX/DRX.

- Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

Discussion

In the previous RAN2 meetings, RAN2 concluded the following on the cell DTX/DRX pattern.

Agreements

- Pattern configuration for cell DRX/DTX is common for Rel-18 UEs in the cell.   FFS whether we have DTX UE specific inactivity timer .  FFS on configuration signaling and stage 3.

- Confirm study item agreement that we can have separate DTX and DRX configuration.   We will focus on designing DTX/DRX for at least single configuration.  FFS whether multiple configuration of cell DTX or DRX will be supported.

- A periodic cell DTX/DRX configuration is explicitly signalled to the UEs.

- A periodic cell DTX/DRX pattern is configured by UE specific RRC signalling.

- The Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration.

One left issue on the cell DTX/DRX is whether we can have a specific inactivity timer. It means that the cell DTX/DRX active time would be extended due to some event. The detail is not clear now, but in our view, the current mechanism to extend UE-specific C-DRX active time can be taken as a reference. For example, the UE/gNB starts or restarts a specific inactivity timer when conditions fulfil, and then considers the time duration between the starting and expiry of this inactivity timer as a kind of the cell DTX/DRX active time. From our perspective, such flexible cell DTX/DRX active/non-active duration complicates the UE/gNB implementation and introduces much normative work. Also, it would reduce the benefit of network energy saving due to the shortened non-active time of the network. Besides, if the cell DTX/DRX pattern is extended by one UE’s requirements but applies to all UEs in the cell, other UEs’ power would undoubtedly increase because other UEs are required to e.g. also monitor PDCCH and periodic/semi-persistent CSI-RS. Additionally, RAN2 needs to consider how other UEs know whether/how the cell DTX/DRX pattern extends. On the other hand, if the cell DTX/DRX pattern is extended and applies to a specific UE, it saves other UEs’ power but may contradict the agreement “pattern configuration for cell DRX/DTX is common for Rel-18 UEs in the cell”. Thus, we prefer not to pursue the inactivity timer for the cell DTX/DRX.

Not pursue the inactivity timer for the cell DTX/DRX.

Another issue left is whether we allow exceptional SR transmission during the cell DRX non-active period. The intention is to avoid the impact on high-priority traffic, which can be an emergency call or public safety-related service (e.g. MPS/MCS).

Agreements

- As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period

FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to to support high priority traffic

As we already agreed in RAN2#122, it relies on the network to avoid the impact on high-priority traffic, e.g. the network may deactivate the cell DTX/DRX. Thus, it is not necessary to introduce any exceptional SR transmission.

RAN2 confirms no exceptional handling for the SR transmission during the cell DRX non-active duration.

In the SI phase, companies have confirmed the benefit of the alignment between UE C-DRX with the cell DTX as well as C-DRX among multiple UEs. On C-DRX alignment among multiple UEs (at least for part of parameters related to the UE-specific C-DRX configuration), we understand it can be implemented by the RRC configuration, thus no extra spec change is needed. On the alignment of the cell DTX/DRX and UE C-DRX, some requirements are achieved in RAN2#123 and we rely on the gNB to ensure the alignment. However, as UE-specific C-DRX active time would be extended when conditions are fulfilled, it would happen that the active duration of one discontinuous mechanism collides with the non-active duration of another discontinuous mechanism. We would like to analyse the collision case by case since the UE behaviour for the cell DTX/DRX non-active is different from the one for UE-specific C-DRX non-active.

Figure 1 cell DTX/DRX and UE-specific DRX

- For case ①, both the cell DTX/DRX and UE-specific C-DRX are in active time, thus, the UE/network transmits as normal.

- For case ②, the cell DTX/DRX is in active time and UE-specific C-DRX is in non-active time, which equals no cell DTX/DRX configured. Thus, the UE/network works as legacy, i.e. the UE/network performs the behaviour for UE-specific C-DRX non-active time.

- For case ③, both the cell DTX/DRX and UE-specific DRX are in non-active time. Since the expected behaviour for the cell DTX/DRX non-active is different from the one for UE-specific C-DRX non-active, we prefer the UE to follow the way expected for the cell DTX/DRX non-active, otherwise, the NES gain cannot be achieved as we designed.

- For case ④, the cell DTX/DRX is in non-active time and UE-specific C-DRX is in active time. For good energy saving gain, the UE is recommended to perform the behaviour for the cell DTX/DRX non-active, except for the exceptional cases agreed in RAN2#122 on the retransmission timer and RACH procedure.

When both the cell DTX/DRX and UE-specific C-DRX are configured, in cell DTX/DRX active, the UE always follows the behaviour designed for the UE-specific C-DRX.

When both the cell DTX/DRX and UE-specific C-DRX are configured, in cell DTX/DRX non-active, the UE always follows the behaviour designed for cell DTX/DRX non-active, except for the cases agreed in RAN2#122 on the C-DRX retransmission timer.

The latest RAN1/2 meeting agrees with the per-cell activation/deactivation of the cell DTX/DRX and leaves an open issue, i.e. FFS if the cell DTX/DRX configuration is per cell or MAC entity.

Agreements

1	Activation/deactivation is per serving cell.  FFS if the configuration is per cell or per MAC entity

Although the per-MAC entity configuration may simplify the UE behaviour, e.g. the UE only needs to maintain one set of cell DTX/DRX configurations for all serving cells, it is not well applicable in the practice, since 1) The gNB is hard to allocate resources and coordinate with others since different UEs may be configured with different serving cell(s). 2) Anyway, the UE maintains the per-cell activation/deactivation of the cell DTX/DRX since the gNB may activate some cells and deactivate others in a certain time duration.

The cell DTX/DRX is configured per cell.

In RAN2#121bis, the RRC signalling is agreed as a baseline to activate/deactivate the cell DTX/DRX.

- As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released.

Furthermore, RAN1#113 confirms the introduction of DCI format 2_X at least to activate/deactivate the cell DTX/DRX. Per the agreements below, the design detail now seems determined and stable.

Agreement For at least the case where one cell DTX/DRX pattern is configured, an information block field of DCI format 2_X for activation and deactivation of cell DTX and DRX configuration supports the following: - Separate (activation/deactivation) signaling for cell DTX and cell DRX, i.e. one activation/deactivation signaling sub-field for cell DTX configuration and one activation/deactivation signaling sub-field for cell DRX configuration - Separate 1 bit indication for each of activation/deactivation for one cell DTX and one cell DRX Above does not imply that multiple DTX/DRX patterns is not supported. Agreement The following high layer signaling are to be included to the RRC parameter list for new DCI format 2_X for activation and deactivation of cell DTX/DRX - search space set configuration with new DCI format 2_X - DCI size for new DCI format 2_X Agreement - An information block field of DCI format 2_X is variable size either 1 or 2 bits. - Higher layer signaling configures whether the activation/deactivation of cell DTX and/or cell DRX is indicated in DCI format 2_X for a serving cell. - If both cell DTX and cell DRX are configured for a serving cell, - 1st bit corresponds to activation/deactivation of cell DTX configuration, and - 2nd bit corresponds to activation/deactivation of cell DRX configuration, - otherwise, the 1 bit corresponds to the configured cell DTX or cell DRX configuration. - Note: this does not imply there may be separate higher layer signaling to enable L1 signaling based activation/deactivation for a cell DTX and/or cell DRX configuration. Signaling design is up to RAN2.

As the activation/deactivation of the cell DTX/DRX can be indicated/implemented by either the RRC or L1 signalling as well as the RRC method is a baseline, one follow-up issue is how to indicate the NW will use the L1 method, i.e. how UE derives NW L1 capability and is there a need of explicit RRC indication that the NW will use L1. In our view, normally for a common DCI, if the corresponding RRC parameters (e.g. search space set and DCI size) are configured, the UE monitors the PDCCH based on such a DCI format and processes the related procedure. As this logic is widely applied in the current system, it makes sense to reuse the new DCI format 2_X and there is no need to introduce any new explicit RRC indication to know the NW will use L1. In other words, the L1 method for the cell DTX/DRX activation/deactivation is applied if the RRC parameters for DCI format 2_X are configured, and the RRC method for the cell DTX/DTX activation/deactivation is obeyed otherwise.

If the RRC parameters for DCI format 2_X are configured, the L1-based cell DTX/DRX activation/deactivation is used. Otherwise, the RRC-based cell DTX/DRX activation/deactivation is used.

As the DCI signalling would be sent after the RRC configuration, the initial status of the cell DTX/DRX should be set as deactivated to keep the spec and the UE behaviour simple as well as make the UE always follow one mechanism.

If the RRC parameters for DCI format 2_X are configured, the initial status of the cell DTX/DRX should be set as deactivated.

Conclusion

We have the following proposals:

Proposal 1	Not pursue the inactivity timer for the cell DTX/DRX.

Proposal 2	RAN2 confirms no exceptional handling for the SR transmission during the cell DRX non-active duration.

Proposal 3	When both the cell DTX/DRX and UE-specific C-DRX are configured, in cell DTX/DRX active, the UE always follows the behaviour designed for the UE-specific C-DRX.

Proposal 4	When both the cell DTX/DRX and UE-specific C-DRX are configured, in cell DTX/DRX non-active, the UE always follows the behaviour designed for cell DTX/DRX non-active, except for the cases agreed in RAN2#122 on the C-DRX retransmission timer.

Proposal 5	The cell DTX/DRX is configured per cell.

Proposal 6	If the RRC parameters for DCI format 2_X are configured, the L1-based cell DTX/DRX activation/deactivation is used. Otherwise, the RRC-based cell DTX/DRX activation/deactivation is used.

Proposal 7	If the RRC parameters for DCI format 2_X are configured, the initial status of the cell DTX/DRX should be set as deactivated.

Reference

[1] RP-223540 New WID: Network energy savings for NR

[2] TR 38.864 Study on network energy savings for NR

[3] R1-2306246 Reply LS on Cell DTX/DRX activation/deactivation (contact: Huawei, Intel)