3GPP TSG-RAN WG2 #123bis	R2-2310388
Xiamen, China, October 2023

Agenda Item:	7.5.4.1

Source:	OPPO

Title:	Discussion on BSR enhancement for XR

Document for:	Discussion, Decision

Introduction

This paper will further discuss the BSR enhancement.

Discussion

Delay status reporting

In RAN2#120, RAN2 agrees to report delay information which is associated with data volume.

- RAN2 will introduce data volume information associated with delay information (e.g. remaining time) in a MAC CE. FFS if this is extension of BSR or new format. FFS how to do that (e.g. what exactly is reported) and how to ensure this information is up-to-date e.g. considering UL scheduling delay.

In RAN2#122, RAN2 further agrees that the remaining time is derived from the PDCP discard timer value.

- 1: UE calculates the remaining time based on the PDCP discard timer value. FFS if UE reports one or multiple values. FFS how this is modelled in PDCP specification. FFS which UEs support this.

In RAN2#123, further agreements are achieved on DSR.

- Network can configure the UE whether to trigger delay status reporting. FFS if we have some thresholds per LCG.

- When UE triggers reporting delay information for a LCG, and UE also reports the buffer status associated with the remaining time.RAN2 aims to define a single MAC CE for the DSR reporting (including the buffer status). FFS if this extends BSR MAC CE or is a new MAC CE.

- Working assumption: Define a new separate MAC CE for DSR (remaining delay and associated data volume) reporting, e.g. DSR reporting is not coupled with BSR reporting. Detailed Definition of associated data volume is FFS.

- Support threshold based DSR reporting, e.g. DSR reporting is triggered when remaining delay of a PDU/PDU set is below a NW configured threshold. The threshold is configured per LCG. FFS whether configuring multiple thresholds for a LCG is supported. Definition of remaining time is FFS.

As agreed, for the LCG which is eligible for the delay status report, the DSR is triggered based on the NW configured threshold. The question is whether multiple thresholds are needed for such an LCG. In our view, a single value per LCG is sufficient since all LCHs associated with the same LCG would have similar QoS requirements (e.g. the latency requirement), otherwise, the LCHs would be linked with different LCGs.

A single threshold is configured per LCG for the DSR triggering.

In the MAC running CR, the MAC triggers a DSR when the remaining time of a PDU becomes shorter than the configured threshold. One follow-up issue is how to define the remaining time, especially when the PDU set-based operation is enabled.

- 5.4.X Delay status reporting The Delay Status Reporting (DSR) procedure is used to provide the serving gNB with delay status of UL data. This delay status includes remaining time of UL data, which is based on the value of its associated PDCP discardTimer (as specified in clause 7.3 in TS 38.323 [4]) at the time of the first symbol of the PUSCH transmission in which the DSR is sent, as well as the amount of data associated with the reported remaining time. RRC controls the DSR operation by configuring the following parameter: - remainingTimeThreshold: the threshold on remaining time of UL data configured for triggering DSR for an LCG. The MAC entity triggers a DSR when the remaining time of a PDU in an LCG, if configured for DSR, becomes shorter than its associated remainingTimeThreshold. Editor’s note: This section will be updated after more agreements are made, e.g. a more precise definition on “the amount of data associated with the reported remaining time” needs to be specified. FFS Definition of remaining time. FFS if one or more values are reported for an LCG. FFS if data with delay below the threshold is reported.

In our view, once the PDU set-based operation is enabled, the remaining time should be the remaining time of a PDU set, since QoS-related characteristics (e.g. PSDB) are per PDU set and all PDUs of a PDU set are required to handle in a whole. As the PDUs of a PDU set would arrive at the same or different times, the remaining time to be reported would be derived by the discardTimer associated with the first SDU of the PDU set. Accordingly, only the remaining time of the first SDU of this PDU set would trigger the DSR. In other words, the UE can trigger a DSR once the remaining time of the first PDU of a PDU set becomes lower than the NW-configured threshold.

Once the PDU set-based operation is enabled, the trigger of a DSR is judged based on the first PDU of a PDU set.

Once the PDU set-based operation is enabled, the reported remaining time of a PDU set in the DSR procedure is derived by the discardTimer associated with the first SDU of its PDU set.

Typically, how much data volume is buffered at the UE side depends on several factors, e.g. UL delay budget, scheduling strategies and radio link quality. As time goes on, an LCG would have a different buffer status. Thus, when a DSR is triggered and if the associated LCG have more than one PDU set buffered, one thing to discuss is how many DS values will be reported. The following shows several potential solutions.

- Solution 1: Only one DS info is reported for the associated LCG. The DS info is associated with the shortest value of the remaining times in the LCG.

- Solution 2: More than one DS info is reported for the associated LCG.

  - Solution 2-1: Each DS info is associated with an individual PDU/PDU set which is buffered in the LCG.

  - Solution 2-2: Several delay ranges are designed for the delay status report, which covers the time distribution of all PDU/PDU sets in an LCG. Each DS info represents an individual delay range.

Compared to other solutions, Solution 2-1 may have an uncontrolled signalling overhead issue. As multiple PDU/PDU sets in an LCG may arrive one by one, several DSRs would be triggered in a short time duration, which induces frequent DSR triggering and reporting. To resolve such an issue, a per-LCG DSR prohibit timer could be considered. Accordingly, the UE would not trigger or report a DSR for a specific LCG if its DSR prohibit timer is running. Based on the above assumption, Solution 2-2 is preferred since, besides the reporting of the most urgent requirement, Solution 2-2 contains enough delay info to let the gNB know the delay status of that LCG.

In the MAC CE for DSR, more than one DS info is reported for the associated LCG. Each DS info represents an individual delay range.

In RAN2#122, it concludes that the reference time for the remaining time relates to the point of the first transmission of the information. What is unclear is whether the intra-UE prioritization impacts this agreement.

- When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this.

In the intra-UE prioritization procedure, the MAC layer triggers a new transmission for the deprioritized MAC PDU when criteria are fulfilled. In this case, the MAC layer directly delivers the deprioritized MAC PDU to the identified HARQ process instead of invoking the Multiplexing and assembly entity to regenerate a new MAC PDU. If the remaining time to be reported is already contained in the deprioritized MAC PDU, the MAC layer has no chance to update the remaining time contained, even if it is the time is outdated. Similar issues also exist in e.g. BSR/PHR and no enhancement has been agreed upon before. Thus, we prefer no impact on the determination of reference time for the remaining time due to intra-UE prioritization.

No impact on the determination of reference time for the remaining time due to intra-UE prioritization.

New BSR

In the current system, the BS information buffered in the uplink is reported by BSR MAC CE. The Index field in the current BSR format is used to represent the BS value, which is a quantization indication of the BS of an LCG in the UE’s buffer. The Index field indicates a range of buffer size, i.e. the actual buffer size is between the pre-defined maximum and minimum values. The larger the Index is, the larger the quantization error is introduced. The quantization error is tolerant in legacy but cannot be neglected for Rel-18 XR since the XR service has a low latency requirement as well as large and various packet sizes.

In the previous meetings, RAN2 agreed with the following on the BS reporting.

- 1. 	As a working assumption, at most one BS index or BS value is reported by an LCG. This assumption can be revisited if new BSR table design cannot achieve a target level of quantization error. FFS what this target level should be.

- 3.	Design/configuration for new BSR table(s) should include support for narrower ranges (i.e. finer granularity) than the legacy. Details can be discussed after an agreement on how UE obtains new BSR table(s) (e.g. pre-definition vs RRC configuration) is made.

- 8.	New BSR table(s) can be used by any UEs that support such a capability. However, design of the new BSR table(s) should be based on XR-specific use cases and requirements.

- 6. 	Network can configure which BSR table(s) an LCG is eligible to use. UE determines which BSR table (i.e. legacy or something else) the LCG should use. FFS details of this determination (e.g. based on buffer size) and how network knows which BSR table each LCG uses.

- 2a. 	Deprioritize Option 2c (static + dynamic BSR tables) and Option 2d (reference table + scaling factor).

- 2b. 	Have more discussions on Option 2a (static BSR tables) vs Option 2b (RRC configured BSR tables). In next meeting, companies should explain how BSR table(s) are created and how many tables would be needed, and how the MAC CE structure will look like. Should also explain what is the expected quantization error.

- Support one static BSR table with 8 bits BS field for Rel-18 XR (for all cases).

- We do not support additional piecewise linear BSR table in Rel-18. Can consider piecewise linearity when discussing how the BSR table values are defined.

In summary, a new static BS table with an 8-bit BS field and finer granularity than the legacy is introduced for Rel-18 XR.

According to the current spec, the existing BS tables cover the BS range from 0 to infinite, i.e. the 8-bit BS table indicates the amount of available data from 0 to (larger than) 81M bytes. In short, the legacy BS table is sufficient to indicate any amount of available data with the drawback of large quantization error for the larger BS value. As the XR service has a large packet size and the new BS table tends to match the XR requirements with finer granularity, we understand the new BS table only needs to cover the upper BS value range in the legacy 8-bit BS table.

The new 8-bit BS table covers the upper BS value range in the legacy 8-bit BS table.

As assumed, the new BS table has a different BS range from the legacy BS tables. When the UE reports the data volume of the XR service, the UE needs to decide which BS table to be selected. Normally, the BS table determination depends on the data volume the UE buffered and the BS range the BS table covered. In one implementation, for the LCG configured to use both the new and legacy BS table, the UE selects the new BS table for encoding if the data volume the UE buffered is larger than the minimum value of the new BS table. Otherwise, the UE selects the legacy BS table. Accordingly, the UE needs to indicate to the gNB which BS table is selected. Thus, a new MAC CE associated with a new eLCID is introduced to contain the BS information of the XR service as well as a new field indicating the selected BS table.

If an LCG is eligible to use both the legacy and new BS tables, the UE chooses the new BS table for encoding if the data volume the UE buffered is larger than the minimum value of the new BS table.

A new MAC CE associated with a new eLCID is introduced to contain the BS information of the XR service as well as a new per-LCG field indicating the selected BS table.

Conclusion

We have the following proposals:

Proposal 1	A single threshold is configured per LCG for the DSR triggering.

Proposal 2	Once the PDU set-based operation is enabled, the trigger of a DSR is judged based on the first PDU of a PDU set.

Proposal 3	Once the PDU set-based operation is enabled, the reported remaining time of a PDU set in the DSR procedure is derived by the discardTimer associated with the first SDU of its PDU set.

Proposal 4	In the MAC CE for DSR, more than one DS info is reported for the associated LCG. Each DS info represents an individual delay range.

Proposal 5	No impact on the determination of reference time for the remaining time due to intra-UE prioritization.

Proposal 6	The new 8-bit BS table covers the upper BS value range in the legacy 8-bit BS table.

Proposal 7	If an LCG is eligible to use both the legacy and new BS tables, the UE chooses the new BS table for encoding if the data volume the UE buffered is larger than the minimum value of the new BS table.

Proposal 8	A new MAC CE associated with a new eLCID is introduced to contain the BS information of the XR service as well as a new per-LCG field indicating the selected BS table.

Reference

[1] RP-230786 Updated WID on XR Enhancements for NR, Nokia, Qualcomm

[2] TR 23.700-60 Study on XR (Extended Reality) and media services

[3] R2-2302309 TR 38.835 v1.0.2, Nokia (Rapporteur)

[4] R2-2309316 Introduction of XR enhancements to TS 38.321, Qualcomm