3GPP TSG-RAN WG2 Meeting #123bis                            R2-2310392 Xiamen, China, October 9th – 13th, 2023

Agenda Item:	7.4.2.2

Source: 	MediaTek Inc.

Title:  	RACH-Less LTM

Document for: Discussion and decision

Introduction

In previous RAN2 meetings, following agreements were made for early TA acquisition and RACH-Less LTM.

RAN2#119bis-e RAN2 assumes that both RACH-based (CFRA, CBRA) and RACH-less procedures for L1 L2 mobility switch may be supported. RACH-less if the UE doesn’t need to acquire TA during the cell switch. RAN2 understands that the feasibility of RACH-less may depend on RAN1, and expect that RAN1 is working on this. RAN2 assumes RACH resource for CFRA for L1 L2 dynamic switch may be provided in RRC configuration (or potentially by MAC CE FFS). RAN2#121bis-e From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order). RRC RACH configuration for early TA acquisition (e.g., including whether RAR needs to be received) is specific per target cell and is signalled separately (separate IEs) from the candidate cell configuration (the part that need to be applied at cell switch). R2 assumes that Early TA RACH option 3 (with RAR from candidate cell) is not needed in Rel-18. With the assumption that the UE will skip RACH in the target cell if a TA value is given in the cell switch command: It is FFS if the following TA values can be given to the UE: - - Value 0, - - Value indicating that the UE shall apply the TA of one source cell. In RACH-less LTM, the target cell is aware of the UE’s arrival based on reception of the first UL transmission from this UE In RACH-less LTM, RRCReconfigurationComplete can be the content of the first UL MAC PDU/transmission to indicate UE arrival, i.e. no need to introduce any new signaling to indicate UE arrival (for the MCG-switch case) RAN2#122 Dynamic grant can be used for RACH-less LTM, for the first UL data transmission to the target cell: - - the UE monitors PDCCH for dynamic scheduling from the target cell, upon LTM cell switch. - - upon cell switch decision, R2 assumes that the source DU informs the target DU about the selected beam, so that the target DU can start scheduling dynamic UL grant. Configured grant can be used for RACH-less LTM, for the first UL data transmission to the target cell, the UE selects the configured grant occasion, which is associated with the beam indicated in the LTM MAC CE (as set by source cell). FFS further optimization - For early TA acquisition for candidate Cells For PDCCH ordered early TA acquisition without RAR, there is no need for UE to maintain the TA timer for candidate cell (i.e. it is NW implementation to determine the TA validity), TA is given in the cell switch MAC CE (when available in the network). RAN2 doesn’t see a need for a solution with RAR in for Rel-18. Observation: Without RAR (without UE maintaining TA), the UE will need to do RACH for link recovery and/or conditional (if supported), which is acceptable in Rel-18 The UE determines to trigger RACH-less cell switch in MAC layer, if the LTM cell switch MAC CE provides the TA value (no RAR is assumed). If the TA maintenance etc for candidate cell(s) in the UE is needed, the TA(s) associated with candidate cell(s) can be maintained during LTM (TDB exactly which cells decide stage-3).

In this contribution, we review the design of Rel-14 RACH-less, consider the different scenarios of RACH-less LTM, and discuss the potential issues in RACH-less LTM.

Discussion

Rationale of RACH-less HO

In Rel-14 LTE, RACH-less HO/SeNB change is defined as ‘skip random access procedure during handover or change of SeNB’. In current running CR for Rel-18 NR LTM, RACH-less LTM is defined as ‘an LTM cell switch procedure where UE skips the RA procedure’. Even though there is no difference between the definitions of RACH-less HO and RACH-less LTM, the scenarios concerned in Rel-14 RACH-less HO and RACH-LTM are different. Despite there being no distinction in the definitions for both Rel-14 RACH-less HO and RACH-less LTM, the scenarios concerning the two in Rel-14 RACH-less HO and RACH-LTM are indeed different.

In the context of LTE Rel-14 and its RACH-less handover, it only supports two specific scenarios.

- Scenario 1: TA value set to 0 for small cell deployment,

- Scenario 2: TA value set to the latest NTA when the TA value of the target cell is the same as one of the serving cells.

In RAN2#123 meeting, RAN2 discussed TA information in LTM MAC CE. Although the content of the MAC CE to indicate TA information was not agreed, RAN2 made following agreements for RACH-less LTM:

The UE will do RACH-less when:

- - TA value is provided in the cell switch MAC CE (already agreed, TA=0 is assumed to be covered by this)

- - When the UE shall apply the same TA value as the source (already agreed) FFS how the UE knows this.

Based on the agreement, RAN2 assumes that Scenario 1 is supported by indicating TA=0 in the LTM MAC CE. However, the method for informing the User Equipment (UE) that the same TA value as the source cell is used in Scenario 2 needs further discussion. One approach is that NTA is provided in the form of bitmap indicating selection among mcg-PTAG, scg-PTAG, mcg-STAG, and scg-STAG in the MAC CE. An alternative approach is that NTA is provided with the actual TA value, even if it has the same value as one of the serving cells. We favor the approach of directly indicating the actual TA value in the MAC CE, as using a bitmap could complicate the MAC CE design.

Proposal 1: RACH-less LTM supports the scenario with small cell deployment and TA value 0 is indicated in the LTM cell switch MAC CE.

Proposal 2: RACH-less LTM support the scenario in which the TA value of the target cell matches that of one of the serving cells, with the TA value provided in the LTM cell switch MAC CE.

UE capability for RACH-less LTM

For LTM, RACH-less LTM should be optionally supported. If UE doesn’t have the capability to support RACH-less LTM, RACH-based LTM procedures should be performed.

If RACH-less LTM needs to support TA acquisition w/wo preamble transmission, the question is whether to differentiate those two types of scenarios. One option is that only one capability bit is used to indicate whether RACH-less LTM is supported or not by the UE. The other option is that two separate capability bits are used to indicate support of early TA acquisition through PDCCH-ordered preamble transmission and support of pre-configured TA without preamble transmission respectively. Two separate capability bits indicating support of TA acquisition w/wo preamble transmission is preferred, since they allow more implementation flexibility in both network side and UE side.

Proposal 3: RACH-less LTM is made available as an optional feature in the context of LTM.

Proposal 4: Two separate capability bits are required for RACH-less LTM, one for early TA acquisition via PDCH-ordered preamble transmission and the other for pre-configured TA value without preamble transmission.

Utilization of CG and DG for RACH-less LTM

In RAN2#122 meeting, it was agreed that both CG and DG are supported for RACH-less LTM. The problem is how UE decides to monitor CG or DG for the first UL data transmission to the target cell. If RACH-less LTM is configured with preallocated CG provided in the pre-configuration RRC message, UE accesses the target cell via CG. Otherwise, UE monitors the DG from the target cell.

From a network implementation perspective, it is not anticipated that DG will be scheduled prior to the availability of the first CG. Moreover, given that the purpose of LTM is to reduce mobility latency and RACH-less LTM can further enhance this, the CG resources should not be configured with a long periodicity. If they were, there would be no benefit to configure CG resources for RACH-less LTM.

Proposal 5: For RACH-less LTM, UE monitors CG for the first UL transmission to the target cell if CG of the target cell is preallocated to the UE in the pre-configuration RRC message. Otherwise, UE monitors PDCCH for DG for the first UL transmission to the target cell.

Successful Completion of RACH-less LTM by the UE

In RAN2#123 meeting, RAN2 made the following working assumption:

RAN2 assumes For RACH-less LTM, the UE determines successful reception of its first UL data based on receiving a PDCCH addressing the UE’s C-RNTI in the target cell scheduling a new transmission after the first UL data, (FFS if specified contents should be transmitted with this transmission, e.g. as LTE MAC CE).

Compared with LTE Rel-14 RACH-less HO, we tend to agree that the false alarm for determining successful RACH-less LTM completion based on PDCCH addressing the UE’s C-RNTI is low. Network doesn’t need to send specific MAC CE to UE.

Proposal 6: For RACH-less LTM, the UE determines successful reception of its first UL data based on receiving a PDCCH addressing the UE’s C-RNTI in the target cell scheduling a new transmission after the first UL data. No specified contents need be transmitted with the new transmission from the target cell.

Misalignment of the TA Timer between UE and NW

In the contribution [2], the issue of the misalignment of the TA timers between UE and NW was raised and discussed. The issue comes from the assumption that the starting point of TA timer at the NW side is earlier than the starting point of TA timer at the UE, and the time gap may not be negligible. Just as illustrated in Figure 1, after the UE sends the preamble, the target DU derives the TA value for the UE and starts the TA timer at the network side. After the coordination among source DU, target DU and CU, the source DU sends the LTM command to UE with the TA value provided. UE starts the TA timer upon reception of the LTM cell switch command. Therefore, there is a time gap between the network and UE for starting the TA timer.

Two options are mentioned in [2], Option a) An indication of the TA timer (or delta) is included in the LTM command; Option b) UE figures out the time difference between preamble transmission and reception of LTM command and abstracts the time difference from the TA timer value configured by RRC.

Both options result in substantial specification impacts and require additional changes to the UE behaviour. Given that the time gap significantly depends on network coordination and implementation, the desired solution is for it to be addressed through network implementation. For example, the TA timer at target DU is started when cell switch decision is made by the source DU and informed to the target DU. Consequently, its impact could be deemed negligible.

Proposal 7: The issue of TA timer misalignment between the UE and network should be addressed by network implementation.

Conclusion

Proposal 1: RACH-less LTM supports the scenario with small cell deployment and TA value 0 is indicated in the LTM cell switch MAC CE.

Proposal 2: RACH-less LTM support the scenario in which the TA value of the target cell matches that of one of the serving cells, with the TA value provided in the LTM cell switch MAC CE.

Proposal 3: RACH-less LTM is made available as an optional feature in the context of LTM.

Proposal 4: Two separate capability bits are required for RACH-less LTM, one for early TA acquisition via PDCH-ordered preamble transmission and the other for pre-configured TA value without preamble transmission.

Proposal 5: For RACH-less LTM, UE monitors CG for the first UL transmission to the target cell if CG of the target cell is preallocated to the UE in the pre-configuration RRC message. Otherwise, UE monitors PDCCH for DG for the first UL transmission to the target cell.

Proposal 6: For RACH-less LTM, the UE determines successful reception of its first UL data based on receiving a PDCCH addressing the UE’s C-RNTI in the target cell scheduling a new transmission after the first UL data. No specified contents need be transmitted with the new transmission from the target cell.

Proposal 7: The issue of TA timer misalignment between the UE and network should be addressed by network implementation.

Reference

[1] 3GPP TS 38.300 V17.5.0

[2] R2-2306011, Discussion on TA handling aspects for LTM, Ericsson

[3] R2-2308614, LTM procedures, Qualcomm Incorporated