3GPP TSG-RAN WG2 Meeting #123bis	R2-2310408

Xiamen, China, 9 - 13 Oct, 2023

Title: 	Discussion on CHO enhancements for NES

Source: 	Huawei, HiSilicon, Turkcell

Agenda item:	7.3.5

Document Type:	Discussion and Decision

- Introduction

In previous meetings, RAN2 has agreed to introduce an NES-specific execution condition. In RAN2 #123, RAN2 discussed the signalling for triggering NES-specific CHO execution condition, and ruled out the time-based trigger (at least for cell DTX/DRX) and indication in SIB1 [1]:

Agreements

1	We will support the CHO triggers for the use case of turning off the cell

2	(At least for cell DTX/DRX) Time-based CHO is not to be considered in NES.

3	Do not consider using an indication in SIB1 for triggering NES CHO execution condition

The solutions on the table are:

- L1 activation/deactivation signalling for cell DTX/DRX

- MAC CE

- RRC reconfiguration (apply NES CHO condition after RRCReconfiguration is received)

In this contribution, we provide our views on the CHO execution trigger, and other remaining issues.

- Discussion

It was agreed in RAN2 #123 that cell turning off is one of the target scenarios. Also, the CHO enhancement should fit with the NES techniques introduced in the R18 NES WI.

In our understanding, SSB-less technique is not relevant to CHO enhancement, because the SSB-less cell only serves as SCell and CPAC enhancement is not in the WID scope; Spatial/power domain techniques are UE-specific, the gNB implementation can decide not to enable these techniques if the corresponding UE is of high QoS requirement, therefore the SD/PD techniques are not relevant to CHO enhancement either. The only NES technique in R18 suitable for CHO enhancement is Cell DTX/DRX.

Based on the above, we discuss the CHO trigger for Cell DTX/DRX and cell switching off respectively.

2.1 Cell DTX/DRX

- L1 activation/deactivation signalling for cell DTX/DRX

During the offline discussion in RAN2 #123 [2], some companies are concerned about the reliability issues and security issues with the L1 group signalling.

On reliability, this has been discussed in RAN1. RAN1 conclusion is to support the group common L1 signalling using PDCCH for cell DTX/DRX activation and deactivation without HARQ feedback, which means, the reliability of L1 signalling for cell DTX/DRX is not an issue (e.g. it can be ensured by increasing the aggregation level). In the reply LS [3], RAN1 does not indicate any reliability issue either. We think this should not be a blocking point in RAN2 decision on the CHO trigger, if reliability of L1 signalling is to be worried about, we think the Cell DTX/DRX mechanism itself is more dangerous as any misalignment will affect UE and gNB behaviour, while in the context of CHO enhancement the decoding of L1 activation signalling is only used to trigger the UE to apply NES-specific CHO evaluation conditions.

On security, we think this is common for both L1 signalling and MAC CE. NR has introduced quite many L1 control signalling (e.g. CG activation in R15, DCP in R16 UE power saving, PEI in R17 UE power saving) and some of them are also group common L1 signalling (e.g. DCP) or cell-specific L1 signalling (e.g. PEI). Besides, this L1 signalling is only used to trigger the CHO condition execution (e.g. in event A4, the UE still needs to compare the target cell quality with the configured threshold) rather than triggering the HO directly. L1 signalling is not protected, this does not necessarily mean it is not secure, but it means we should also have RRC based mechanism. The legacy HO/CHO is performed by RRC signalling, and NW implementation can choose to use the legacy RRCReconfiguration signalling to trigger HO or to reconfigure CHO conditions, if someone finds a way to make it unsecure in the future.

Another concern on reusing L1 activation/deactivation signalling is that, L1 signalling is not always sent since RAN2 also agreed to use implicit activation by RRC configuration. Some companies think, if the Cell DTX/DRX are implicitly activated once configured, the NW does not send L1 activation signalling to UEs anymore. From our perspective, in this case, the NW can still send L1 activation signalling even though the Cell DTX/DRX is activated. In the DCI format 2_9 design, there is explicit indication for activation/deactivation, rather than letting the UE toggle the previous state (from activation to deactivation, or vice versa), so there will be no confusion on the activation/deactivation state even if the UE receives the information repeatedly.

Reusing L1 activation/deactivation signalling implies that CHO enhancement is always used together with Cell DTX/DRX feature, and this is not desired by some companies. Some companies want the CHO enhancement to be a general feature, leaving some room for SD/PD techniques or future NES techniques. As stated previously, in our understanding, at least among the R18 NES techniques, only Cell DTX/DRX is relevant to CHO enhancement, to hand over some UEs of high QoS requirements to other cells as fast as possible before the serving cell enters NES mode. SD/PD techniques are UE-specific, and SSB-less feature only applies to SCell.

It was also commented in the previous meeting that, reusing this L1 activation/deactivation signalling would imply different UE behaviours for UEs supporting NES CHO or not. We think UEs not supporting NES CHO can be handled by legacy HO command, and there is no problem.

Based on the above analysis, we do not see an issue with reusing the L1 activation/deactivation signalling.

Observation 1: No issue is identified for reusing the L1 activation/deactivation signalling for CHO condition execution (e.g. reliability; security; Cell DTX/DRX already activated; coupling with Cell DTX/DRX feature).

A further optimization would be using the spare bits in DCI format 2_9, considering RAN1 already allows some spare bits:

Agreement DCI format 2_X, for activation and deactivation of cell DTX and DRX configuration, - at least includes following fields, - N information block field(s), - Spare/reserved padding bits to match the size configured for DCI 2_X (if needed) - payload size is configurable and within the bounds set by existing RAN1 specification - an information block field contains signaling of activation or deactivation of ‘a configuration of cell DTX and/or DRX’ of ‘a serving cell’ - for serving cell configured with SUL, the same bit is applicable for both NUL and SUL Above applies at least for sTRP case.

This requires RAN1 work on defining additional bits in DCI 2_9. And if more bits are specified in DCI format 2_9, the NW need to configure a relatively bigger size for the DCI payload, i.e. larger signalling overhead. Also, we had an agreement in RAN2 #122 that we will not introduce new L1 signalling. It is a bit ambiguous whether additional fields in the “existing” DCI format are considered as new signalling or not.

- MAC CE

Compared with reusing L1 signalling, we do not see benefit of designing a new MAC CE: the MAC CE is not faster or more flexible than L1 signalling, and the MAC CE cannot bring signalling reduction because group signalling for MAC CE is not feasible.

- RRC reconfiguration (apply NES CHO condition after RRCReconfiguration is received)

This solution was also proposed by some companies, but was not discussed in detail due to lack of time. The general idea is that, the NW configures an NES specific CHO event (and the NW could also configure a normal CHO event together), and when NW decides to enter NES mode (i.e. apply NES techniques) it uses some indication in RRCReconfiguration to trigger the UE to apply the NES specific CHO execution condition.

In our understanding, this solution has no clear benefit compared with the legacy method of using RRCReconfiguration to reconfigure the CHO parameters.

Proposal 1: In the context of Cell DTX/DRX, L1 activation/deactivation signalling is reused for CHO condition execution.

2.2 Cell turning off

The case of cell turning off was not explained in the WID descriptions, it looks more of an implementation issue to us, where the serving cell intends to switch off to obtain deeper and longer sleep, e.g. at two o'clock in the midnight.

Some companies proposed to reuse the time-based CHO introduced in R17 NR NTN. We also consider it feasible, and the GNSS capability issue can be solved by broadcasting SIB9 so that UEs not capable of GNSS can deduce the relationship between SFN/subframe and the UTC time. However, some other companies are concerned that the time-point for cell switching off is not that deterministic as satellite switching in NTN scenarios. RAN2 discussed for several meetings and could not conclude whether time-based CHO can be used in cell turning off cases. Our suggestion is to leave it to NW implementation to use legacy RRCReconfiguration to configure suitable CHO parameters or to configure HO command to the UE, as in cell turning off case it is not very urgent to hand over the UEs and the legacy mechanism works well.

Proposal 2: In the context of cell turning off, it is left to NW implementation to use legacy CHO or HO to handover the UEs.

2.3 Target cell NES mode

According to the time budget, RAN4 needs to finish RRM/RF core requirements by Q4 2023, and it is critical that RAN2 concludes the CHO enhancements in October meeting.

In the email discussion during RAN2 #121bis-e, the majority of companies are not convinced any enhancement related to target cell NES mode is necessary:

Proposal 8: RAN2 to discuss following two options to ensure that the UE can find a suitable target cell: - Option 1: [11/ 25] NW implementation to reconfigure candidate cells, i.e., no spec impact - Option 2: [9/ 25] Network provides additional prioritization for candidate cells that UE can take into consideration.

And currently no agreement is made for target cell. Therefore, we think RAN2 should stop wasting time on discussing enhancements related to target cell NES mode and can leave it to the next release if needed.

Proposal 3: Enhancement related to target cell NES mode is not considered.

In the RAN2 #121bis-e meeting, the failure case (i.e. there are no good candidate cells for CHO when the source cell is going to enter NES mode) and the candidate UE behaviour were discussed.

In our understanding, such case could be avoided by NW implementation. For example, the source cell could monitor the state of candidate cells based on UE measurement reports, and it could reconfigure the candidate cells or postpone the sleeping if it finds that the current candidate cells are not good enough. Even this case happens, legacy mechanisms, e.g. RRC re-establishment, could be used and there is no need to introduce anything new.

Proposal 4: No new mechanisms are introduced for handling the failure case.

- Conclusion

In this paper, we discuss CHO enhancements for NES, and propose the following:

Observation 1: No issue is identified for reusing the L1 activation/deactivation signalling for CHO condition execution (e.g. reliability; security; Cell DTX/DRX already activated; coupling with Cell DTX/DRX feature).

Proposal 1: In the context of Cell DTX/DRX, L1 activation/deactivation signalling is reused for CHO condition execution.

Proposal 2: In the context of cell turning off, it is left to NW implementation to use legacy CHO or HO to handover the UEs.

Proposal 3: Enhancement related to target cell NES mode is not considered.

Proposal 4: No new mechanisms are introduced for handling the failure case.

- Reference

- R2-2308963	Report from UP, Small data, URLLC/IIoT, RACH indication, NWES and UAV, Session chair (InterDigital)

- R2-2309211, Report of offline discussion on NES specific CHO, Huawei, HiSilicon

- R2-2307017 (R1-2306246), Reply LS on Cell DTX/DRX activation/deactivation, RAN1 (Contact company: Huawei, Intel)

- R2-2304353, Summary of EMAIL DISC [121bis#xxx] on AI 7.3.5 Connected Mode Mobility, Lenovo