3GPP TSG-RAN WG2 Meeting #123bis		R2-2310424

Xiamen, China, October 9th – 13th, 2023

Source:	vivo

Title:	Discussion on inter-RAT measurements without gap

Agenda Item:	7.25.1

Document for:	Discussion

- Introduction

RAN2#123 meeting has achieved below agreements:

RAN2 introduce new terminology of effective measurement window and new configuration MeasWindowConfig for the effective measurement window, which includes WindowOffset, WindowDuration, and WindowPeriodicity. The effective measurement window is a per-UE configuration (can revisit if need is found). Other values of configuration for effective measurement window are pending on RAN4 further discussion. RAN2 to clarify that value nogap-noncsg in the NeedForNCSG-InfoEUTRA-r17 means no gap without interruption in the Rel-17 and Rel-18 specifications. (this is a rel-17 change, and separate CR is needed, can be provided next meeting). P2 postponed [To introduce a per-UE level capability (e.g. interRAT-LTEMeas-NoGapNoInterr-r18) in MeasAndMobParametersCommon to indicate whether gap and interruption can be avoided when the measured LTE CRS is within UE’s active BWP. The new UE capability is only applicable in FR1.]

In this contribution, we will discuss Rel-18 signalling for inter-RAT measurements without gap.

- Discussion

As per the LS[1],  Rel-18 signalling requirement for inter-RAT measurements without gap is summarized in below table. The greem part has been discussed in the last meeting.

| Subcase | Signalling Requirement |
| --- | --- |
| Case b-1: inter-RAT LTE measurements without gap as there is vacant RF chains for UE measurements | - Reuse NeedForGapNCSG-InfoEUTRA-r17 to support Case b-1. no need to introduce any new Rel-18 signalling from UE - Introduce the effective measurement window, including: - measurement window periodicity: 40ms and 80ms, other values; - time domain offset; - window duration: 5ms, other values. |
| Case b-2: inter-RAT LTE measurements without gap when LTE CRS to be measured are fully contained within UE’s active BWP | - use a new per-UE capability to indicate the supporting of inter-RAT LTE measurements without gap when LTE CRS is fully contained in the active BWP, similar as interFrequencyMeas-NoGap-r16 in Rel-16. - introduce the effective measurement window (same as case b-1); |

2.1 Signaling design for Case b-1

UE capability reporting

It’s clarifed in RAN2#123 meeting that value nogap-noncsg in the NeedForGapNCSG -InfoEUTRA-r17 means no gap without interruption in the Rel-17 and Rel-18 specifications.

If we reuse NeedForGapNCSG-InfoEUTRA-r17 with “nogap-noncsg” for case b-1, it’s natural to reuse per-UE capability  eutra-NeedForGapNCSG-Reporting-r17 to support Case b-1. The UE capability eutra-NeedForGapNCSG-Reporting-r17 is as below refer to TS38.306.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| eutra-NeedForGapNCSG-Reporting-r17 Indicates whether the UE supports reporting of the NCSG and measurement gap requirement information for E-UTRA target bands in the UE response to a network configuration RRC message as specified in TS 38.331 [9]. | UE | No | No | No |

- Reuse UE capability eutra-NeedForGapNCSG-Reporting-r17 to support Case b-1(inter-RAT LTE measurements without gap as there are vacant RF chains for UE measurements).

2.2 Signaling design for Case b-2

NeedForGap info Reporting

For case b-2, the UE can perform inter-RAT LTE measurements without measurement gaps and no interruption if the LTE CRS to be measured is completely contained in the active BWP of the UE. Based on the LS[1], the UE may report “nogap-nointerrupt” in Rel-18 field for case b-2. The Rel-18 indication could be in addition to the legacy NeedForGapNCSG-InfoEUTRA-r17 information.

How does UE determine the gap requirement?  The gap requirement could change in case of different active BWPs. Considering BWP switching, a method is required to determine the needForGap information when multiple BWPs are configured. However, it is not described in the LS[1].

We may wait for RAN4 progress regarding how UE determines and reports needForGap info for case b-2.

- Wait for  RAN4 progress regarding how UE determines needForGap info for case b-2(inter-RAT LTE measurements without gap when LTE CRS to be measured are fully contained within UE’s active BWP)

UE capability reporting

RAN4 agrees to introduce a new per-UE capability to support case b-2 similar to Rel-16 inter-frequency measurement without gap interFrequencyMeas-Nogap-r16.

We may define the UE capability as below.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| interFrequencyMeas-NoGap-r16 Indicates whether the UE can perform inter-frequency SSB based measurements without measurement gaps if the SSB is completely contained in the active BWP of the UE as specified in TS 38.133 [5]. If this parameter is indicated for FR1 and FR2 differently, each indication corresponds to the frequency range of cells to be measured. | UE | No | No | Yes |
| interRAT-LTE-Meas-NoGapNoInterrupt-r18 Indicates whether the UE can perform inter-RAT LTE measurements without measurement gaps and no interruption if the LTE CRS to be measured is completely contained in the active BWP of the UE as specified in TS 38.133 [5]. | UE | No | FFS | FFS |

- Define per-UE capability, e,g. interRAT-LTE-Meas-NoGapNoInterrupt-r18,  to support Case b-2

2.3 Configuration of inter-RAT LTE measurements without gap.

It’s agreed to introduce new terminology of effective measurement window and new configuration MeasWindowConfig for the effective measurement window, which includes WindowOffset, WindowDuration, and WindowPeriodicity.

When does the network configure the effective measurement window?  This is an open issue regarding the effective measurement window. The answer could be after the UE indicates the supporting of Case b-1 or b-2, or after the UE has reported the NeedForGap info for Case b-1 or b-2.

It’s decided to introduce a new UE capability for Case b-2, but the old capability could be used for Case b-1, i.e. the UE capability eutra-NeedForGapNCSG-Reporting-r17, which supports “gap” and “nogap” cases. It’s not enough for the network to identify the Case b-1 capability because both Rel-17 and Rel-18 cases are covered by the same capability. If Network configures the effective measurement window for the UE supporting Case b-1 or Case b-2, and we decide to reuse eutra-NeedForGapNCSG-Reporting-r17 capability for Case b-1, then a new UE capability for the effective measurement window may be needed.

We think further clarification from RAN4 is needed for signaling design.

- Clarify with RAN4 whether a new UE capability for the effective measurement window is needed

- Conclusion

Based on the discussion, we have the following proposals:

Proposal 1: Reuse UE capability eutra-NeedForGapNCSG-Reporting-r17 to support Case b-1(inter-RAT LTE measurements without gap as there are vacant RF chains for UE measurements).

Proposal 2: Wait for RAN4 progress regarding how UE determines needForGap info for case b-2(inter-RAT LTE measurements without gap when LTE CRS to be measured are fully contained within UE’s active BWP)

Proposal 3: Define per-UE capability, e,g. interRAT-LTE-Meas-NoGapNoInterrupt-r18,  to support Case b-2

Proposal 4: Clarify with RAN4 whether a new UE capability for the effective measurement window is needed

- References

- R2-2307041(R4-2310158) LS on inter-RAT measurements without gaps