3GPP TSG-RAN WG2 Meeting #123bis	R2-2310446

# Xiamen, China, 9 – 13 October, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | - | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running CR 38306 for UE capability for R18 SONMDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of R18 features on SON and MDT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | R18 UE capabilities for SON and MDT are defined. Current version include the capabilities of the features below: - Optional UE capability without signalling for RLF Report for Inter-system HO for Voice Fallback; - Optional UE capability with signalling for SPR; - Optional UE capability with signalling for Inter-RAT SHR; - Optional UE capability without signalling for RLF Report for NPN; - Optional UE capability [with] signalling for RA report for RACH partitioning; - Optional UE capability [without] signalling for RLF Report for Fast MCG recovery; - Optional UE capability with signalling for Logged MDT for SNPN; - Optional UE capability with signalling for Logged MDT for PNI-NPN; The capabilities of SON enhancement for other features (e.g. NR-U, CPAC) can wait for more progress. Note: In 5) and 6), [with], [without] means whether this UE capability is with or without signalling need wait for more progress. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No R18 UE capabilities for SON and MDT are defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.17, 4.2.18, 5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.331 CR TBD |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 4.2.17	SON parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| onDemandSI-Report-r17 Indicates whether the UE supports delivery of on-Demand SI information upon request from the network as specified in TS 38.331 [9]. | UE | No | No | No |
| pscell-MHI-Report-r17 Indicates whether the UE supports the storage of PSCell mobility history information and the reporting in UEInformationResponse message as specified in TS 38.331 [9]. | UE | No | No | No |
| [rach-Partitioning-Report-r18] Indicates whether the UE supports the storage and delivery of RACH partitioning related information via RACH report procedure, upon request from the network. Editor’s note: Whether with or without signalling can depend on the online discussion. | UE | No | No | No |
| rach-Report-r16 Indicates whether the UE supports delivery of RA report upon request from the network. | UE | No | No | No |
| rlfReportCHO-r17 Indicates whether the UE supports RLF-Report for conditional handover. | UE | No | No | No |
| rlfReportDAPS-r17 Indicates whether the UE supports RLF-Report for DAPS handover. | UE | No | No | No |
| spr-Report-r18 Indicates whether the UE supports the storage and delivery of Successful PScell Change/Addition Report upon request from the network. | UE | No | No | No |
| success-HO-Report-r17 Indicates whether the UE supports the storage and delivery of Successful Handover Report upon request from the network as specified in TS 38.331 [9]. | UE | No | No | No |
| success-InterRAT-HO-Report-r18 Indicates whether the UE supports the storage and delivery of Successful Handover Report for Handover from NR to E-UTRA, upon request from the network. | UE | No | No | No |
| twoStepRACH-Report-r17 Indicates whether the UE supports the storage and delivery of 2-step RACH related information upon request from the network as specified in TS 38.331 [9]. | UE | No | No | No |

### 4.2.18	UE-based performance measurement parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| barometerMeasReport-r16 Indicates whether the UE supports uncompensated barometeric pressure measurement reporting upon request from the network. | UE | No | No | No |
| earlyMeasLog-r17 Indicates whether the UE supports the storage of Early Measurement Logging in logged measurements and the reporting upon request from the network as specified in TS 38.331 [9]. | UE | No | No | No |
| excessPacketDelay-r17 Indicates whether the UE supports the UL PDCP excess packet delay measurement per DRB as specified in TS 38.314 [26]. A UE that supports the UL PDCP excess packet delay measurement shall also support the measurement configuration and reporting as specified in TS 38.331 [9]. | UE | No | No | No |
| gnss-Location-r16 Indicates whether the UE is equipped with a GNSS or A-GNSS receiver that may be used to provide detailed location information along with SON, MDT, and NTN related measurements in RRC_CONNECTED, RRC_IDLE and RRC_INACTIVE state. A UE shall set this field to supported if it indicates the support of nonTerrestrialNetwork-r17. | UE | CY | No | No |
| immMeasBT-r16 Indicates whether the UE supports Bluetooth measurements in RRC_CONNECTED state. | UE | No | No | No |
| immMeasWLAN-r16 Indicates whether the UE supports WLAN measurements in RRC_CONNECTED state. | UE | No | No | No |
| loggedMDT-PNI-NPN-r18 Indicates whether the UE supports Logged MDT for PNI-NPN(s). | UE | No | No | No |
| loggedMDT-SNPN-r18 Indicates whether the UE supports Logged MDT for SNPN(s) and the storage and delivery of SNPN ID(s) for Logged MDT upon request from the network. Editor’s note: PNI-NPN capability bit seems also needed since the NW should send the logged MDT configuration based on this capability. Here we set 2 separate capabilities for SNPN and PNI-NPN for logged MDT. | UE | No | No | No |
| loggedMeasBT-r16 Indicates whether the UE supports Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state. | UE | No | No | No |
| loggedMeasurements-r16 Indicates whether the UE supports logged measurements in RRC_IDLE and RRC_INACTIVE state. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The minimum memory size of MDT logged measurements is 64KB. | UE | No | No | No |
| loggedMeasWLAN-r16 Indicates whether the UE supports WLAN measurements in RRC_IDLE and RRC_INACTIVE state. | UE | No | No | No |
| multipleCEF-Report-r17 Indicates whether the UE supports the storage and delivery of multiple CEF reports upon request from the network as specified in TS 38.331 [9]. | UE | No | No | No |
| orientationMeasReport-r16 Indicates whether the UE supports orientation information reporting upon request from the network. | UE | No | No | No |
| sigBasedLogMDT-OverrideProtect-r17 Indicates whether the UE supports the override protection of the signalling based logged measurements configured in NR. | UE | No | No | No |
| speedMeasReport-r16 Indicates whether the UE supports speed information reporting upon request from the network. | UE | No | No | No |
| ulPDCP-Delay-r16 Indicates whether the UE supports UL PDCP Packet Average Delay measurement (as specified in TS 38.314 [26]) and reporting in RRC_CONNECTED state. | UE | No | No | No |

Next change

## 5.7	MDT and SON features

Definitions for feature

Mobility history information storage It is optional for UE to support the storage of PCell mobility history information and the reporting in UEInformationResponse message as specified in TS 38.331 [9].

Cross RAT RLF Report It is optional for UE to support the delivery of EUTRA RLF report to an NR node upon request from the network.

Radio Link Failure Report for inter-RAT MRO EUTRA It is optional for UE to support: - Inclusion of EUTRA CGI and associated TAC, if available, and otherwise to include the physical cell identity and carrier frequency of the target PCell of the failed handover as failedPCellId in RLF-Report upon request from the network as specified in TS 38.331 [9]. - Inclusion of EUTRA CGI and associated TAC as previousPCellId in RLF-Report as specified in TS 38.331 [9]. - Inclusion of eutraReconnectCellId in reconnectCellId in the RLF-Report as specified in TS 38.331 [9] upon UE has radio link failure or handover failure and successfully re-connected to an E-UTRA cell.

SCG Failure Report for MRO It is optional for UE to support the delivery of the SCG failure related parameters for MRO in SCGFailureInformation message to the network.

SpCell ID indication It is optional for UE to support the delivery of the spCellID-r17 in the RA-Report, if the RA procedure is performed in a SCell of the MCG/SCG.

[RLF Report for Fast MCG Recovery] It is optional for UE to support the delivery of the Fast MCG recovery related information in the RLF-Report. Editor’s note: Whether with or without signalling can depend on the online discussion.

RLF Report for NPN It is optional for UE to support the delivery of the NPN ID in the RLF-report. Editor’s note: Whether SNPN and PNI-NPN should be separate capabilities for RLF report? Can depend on online discussion.

RLF Report for Inter-system HO for Voice Fallback It is optional for UE to support the delivery of an explicit indication in the RLF-report when mobility from NR due to voice fallback fails.

End of change

Annex:

The following table is the potential R18 Layer-2 and Layer-3 feature list for NR_ENDC_SON_MDT_enh2-Core for reference in TR38.822.

### [7.2.xx]	NR_ENDC_SON_MDT_enh2-Core

Table 7.2.xx-1: Layer-2 and Layer-3 feature list for NR_ENDC_SON_MDT_enh2-Core

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| xx. NR_ENDC_SON_MDT_enh2-Core | xx-1 | RLF Report for Inter-system HO for Voice Fallback | It is optional for UE to support the delivery of an explicit indication in the RLF-report when mobility from NR due to voice fallback fails. |  | N/A | N/A | No | No |  | Optional without capability signalling |
|  | xx-2 | Report for SPR | Indicates whether the UE supports the storage and delivery of Successful PScell Change/Addition Report upon request from the network. |  | spr-Report-r18 | SON-Parameters-r16 | No | No |  | Optional with capability signalling |
|  | xx-3 | Report for Inter-RAT SHR | Indicates whether the UE supports the storage and delivery of Successful Handover Report for Handover from NR to E-UTRA, upon request from the network. |  | success-InterRAT-HO-Report-r18 | SON-Parameters-r16 | No | No |  | Optional with capability signalling |
|  | xx-4 | RLF Report for NPN | It is optional for UE to support the delivery of the NPN ID in the RLF-report. Editor’s note: Whether SNPN and PNI-NPN should be separate capabilities for RLF report? Can depend on online discussion. |  | N/A | N/A | No | No |  | Optional without capability signalling |
|  | xx-5 | RA report for RACH partitioning | Indicates whether the UE supports the storage and delivery of RACH partitioning related information via RACH report procedure, upon request from the network. Editor’s note: Whether with or without signalling can depend on the online discussion. |  | rach-Partitioning-Report-r18 | SON-Parameters-r16 | No | No |  | Optional [with] capability signalling |
|  | xx-6 | RLF Report for Fast MCG recovery | It is optional for UE to support the delivery of the Fast MCG recovery related information in the RLF-Report. Editor’s note: Whether with or without signalling can depend on the online discussion. |  | N/A | N/A | No | No |  | Optional [without] capability signalling |
|  | xx-7 | NR-U | [FFS] Can wait for more progress of the features. |  |  |  | No | [FR1 only] |  |  |
|  | xx-8 | MRO CPAC | [FFS] Can wait for more progress of the features. |  |  |  | No | No |  |  |
|  | xx-9 | Logged MDT for SNPN | Indicates whether the UE supports Logged MDT for SNPN(s) and the storage and delivery of SNPN ID(s) for Logged MDT upon request from the network. |  | loggedMDT-SNPN-r18 | UE-BasedPerfMeas-Parameters-r16 | No | No |  | Optional with capability signalling |
|  | xx-10 | Logged MDT for PNI-NPN | Indicates whether the UE supports Logged MDT for PNI-NPN(s). Editor’s note: PNI-NPN capability bit seems also needed since the NW should send the logged MDT configuration based on this capability. Here we set 2 separate capabilities for SNPN and PNI-NPN for logged MDT. |  | loggedMDT-PNI-NPN-r18 | UE-BasedPerfMeas-Parameters-r16 | No | No |  | Optional with capability signalling |