3GPP TSG-RAN2 Meeting #123-bis	R2-2310461

Xiamen, China, Oct. 9-13, 2023

Agenda item:	7.8.1

Work Item:	NR_UAV-Core

Source: 	Qualcomm Incorporated

Title: 	Proposed resolutions for Editor’s Notes and FFSes in the RRC running CR

Document for:     Discussion/Decision

# Introduction

As the outcome of the email discussion “[POST123][311][UAV] Running CR 38.331 (Qualcomm)”, the latest version of RRC running CR is available in R2-2309611.

There are several Editor’s Notes and FFSes in the running CR. In this document, CR rapporteur’s view is provided aiming to resolve the FFSes and Editor’s Notes.

# UE Height vs Altitude

In LTE, the term UE height and UE altitude were both used. In some cases, the ‘height’ was with respect to the sea level, whereas in other cases it was with respect to a reference elevation.

During NR UAV discussion, so far most of the RAN2 agreements have used the term height.

‘Height’ generally refers to vertical distance from a reference point, generally the ground level, whereas ‘Altitude’ generally refers to vertical distance from the mean sea level. Generally in aviation, ‘Altitude’ is more commonly used instead of ‘height’. Additionally, CT1 and other groups have used ‘Altitude’ in their specifications to refer to the altitude of the UE.

In the RAN2 last meeting, it was discussed and clarified that we should not use the reference+offset way (as used in LTE) of referring to UE’s altitude, instead we will use a value range for configuration and reporting which can indicate the whole range of possible UE heights where ‘height’ was generally understood to be with respect to the MSL (mean sea level), instead of the AGL (above ground level) of the local terrain.

Therefore, it is more appropriate to uniformly use the term ‘altitude’ instead of mixing the terms ‘height’ and ‘altitude’ in NR.

Use ‘altitude’ instead of ‘height’ throughout the running CRs.

Note that in the rest of this paper, we still use the term ‘height’ for the continuity of discussion, however they should also be replaced by ‘altitude’ if Proposal 1 is agreeable.

# Values/ranges for height and related parameters

## Number of height ranges for height-based SSB-ToMeasure

RAN2#122 agreed the following:

      - Add height-based list of SSB-ToMeasure with corresponding height ranges and hysteresis in MeasObjectNR.  FFS on the number of height ranges

In the future when aerial vehicles become more popular and ubiquitous, one can imagine that there could be multiple different layers of aerial highways, and networks may correspondingly optimize and configure the corresponding height dependent parameters. Therefore, for future proofing, we propose the number of height ranges to be 32.

We do not see strong need to be conservative on that max number because:

- It is up to the network how many such ranged are configured regardless of the possibility in signalling.

- Being conservative with this only saves a few bits in signalling. (E.g., if we choose 8 instead of 32 for max number, if the network is actually configuring say only 3 such ranges, the difference in total encoded size is expected to be just 2 bits.)

- If in the future we wanted to extend the max size of the list, then it will add much more overhead due to extension markers compared to the 2 bits savings now.

For height-based list of SSB-ToMeasure, number of height ranges is 32.

## Value range for UE height configuration and reporting

RAN2#122 agreed: “We will use LTE UEheight”.

RAN2#123 agreed: "Signalling should allow the network to simultaneously configure height-based event thresholds for the whole range of possible UE heights”.

Note that UE height parameter in LTE is defined as follows (see TS 36.331):

heightUE-r15                      INTEGER (-400..8880)       OPTIONAL

heightUE

Indicates height of the UE in meters relative to the sea level. Value 0 corresponds to sea level (i.e., negative value indicates depth of the UE below sea level). Value -400 corresponds to -400 m, value -399 corresponds to -399 m and so on.

These values were chosen to cover the lowest exposed point on earth surface (The Dead Sea) to the highest point (The Mt. Everest). Note that the above encoding takes 14 bits. However not all possible codepoints were utilized, as 14 bits could’ve covered the range of (-400..15983m).

In LTE, values for UE height range for configuration and reporting were chosen to cover the lowest exposed point on earth surface (The Dead Sea) to the highest point (The Mt. Everest).

The LTE encoding of heightUE takes 14 bits, however not all possible codepoints were utilized (14 bits could cover -400..15983).

During the LTE discussions on this topic, it was not anticipated that the UE receiving communication services using 3GPP technologies would go beyond that height. However, in present context, we should be open to expand this UE height range well beyond the top of the Mt. Everest.

For example, the high-altitude platforms (HAPs) are typically deployed between 8km and 50km above ground level (see TR 38.811). While HAPs are neither expected to be UE nor UAV, it is reasonable to assume that the UAVs of the future may be able to go beyond 8880m and still be provided services by 3GPP cellular technologies e.g. using HAPs or NTN.  So, rapporteur’s proposal is to add 1 bit in the encoding of the height range compared to LTE in terms of possible signalling values, i.e. go from 14bits to 15bits encoding.

The UAVs of the future may be able to go well beyond 8880m and still be provided services by 3GPP cellular technologies such as HAPs and NTN.  So, it is proposed to make the range about 2x of LTE, i.e. add 1 bit in the encoding.

For RRC signalling, the height range (min to max) for UE height (both for configuration and reporting) is -400m to 32,367m above mean sea level, with granularity of 1m.

## Value range and granularity for Hysteresis for height (h1-Hysteresis, h2-Hystereris, heightHyst)

Note that in LTE, the maximum value for the H1 and H2 hysteresis is 16m with granularity of 1m. Since in NR we are aiming for higher range of flight elevation, one can imagine that the area to avoid ping-pong between different height ranges can be higher or lower. So, it is suggested to increase this to 128m (i.e. additional 3 bits for defining the codepoints).

Value range for h1-Hysteresis, h2-Hystereris, heightHyst is 1m to 128m, with granularity of 1m.

# Ambiguous agreement not yet captured: Standalone reporting of height, location, velocity

Height measurement alone cannot be sent in LTE without including it with other radio measurements. Similarly, UAV’s location and velocity measurements cannot be sent alone in LTE without including it with other radio measurements. In some cases, the network may benefit from the ability to configure height, location and/or velocity report alone (e.g. if a large UL grant could not be provided to fit the full RRM report).  Therefore, RAN2#121 previously agreed the following:

When event H1 or H2 triggers, the content of the measurement report is configurable by the network (i.e. it can contain UAV UEs height, location information and/or RSRP/RSRQ measurement results)

Since the above “and/or” can be interpreted differently, this was again briefly discussed in RAN2#122 but no explicit conclusion was captured. It can be interpreted as standalone height, location and/or velocity reporting without being accompanied by RSRP/RSRQ measurements results is already allowed by the above agreement from RAN2#121 but is not explicitly clear.

So, this proposal is presented again for explicit agreement before capturing in the running CR.

[Clarification to RAN2#121 agreement] UE height, location and/or velocity reporting without being accompanied by RSRP/RSRQ measurement results is supported and is configurable by the network. This applies to all height-related event triggers.

# reportOnLeave and timeToTrigger for the new events

Note that in LTE, some event triggers have reportOnLeave (A3/A4/A5/A6/C1/C2 etc.), while others do not (e.g. A1/A2/V1/V2/H1/H2). On the other hand, timeToTrigger was a common field for all events. In NR, all the event triggers defined thus far have configuration for reportOnLeave and timeToTrigger by default. Therefore, the running CR captures reportOnLeave and timeToTrigger for all new events with Editor’s Notes since there had been no formal agreement yet.

In LTE, some event triggers have reportOnLeave (A3/A4/A5/A6/C1/C2 etc.), while others do not (e.g. A1/A2/V1/V2/H1/H2). However, in NR, all the event triggers defined thus far have configuration for reportOnLeave by default.

Similar to all other event triggers in NR, allow reportOnLeave and timeToTrigger configuration for all newly introduced events (already captured in the running CR).

# Editor’s Note on applicability of FP update thresholds for availability indication in RRCSetupComplete

Relevant text from 5.3.3.4	Reception of the RRCSetup by the UE is copied below for quick reference:

2> if the UE has flight path information available: 3> include flightPathInfoAvailable; Editor’s Note: Whether there is need to check against the FP update thresholds (if previously configured) in this section (e.g. for the case of RRC_CONN -> RRC_IDLE -> RRC_CONN case). This is also related to whether the thresholds are released or kept when moving to IDLE.

Rapporteur’s assumption is the distance and time thresholds for FP update can be (re)configured only in RRCReconfiguration and RRCResume, but not in other messages like RRCSetup and RRCReestablisment. So, it is assumed that for RRCSetupComplete, UE doesn't need to check for the thresholds configuration for determination of indicating the FP availability (i.e. it is always like new FP available indication).

Distance and time thresholds for FP update can be (re)configured in RRCReconfiguration and RRCResume, but not in other messages like RRCSetup and RRCReestablisment.

When UE moves to RRC_IDLE, the thresholds for triggering FP update, if configured, are released.

For RRCSetupComplete, UE doesn't check for the threshold(s) configuration for indicating FP availability (i.e. it is always like new FP available indication).

# Editor’s Notes on UE behaviour when neither distance nor time threshold is configured for triggering FP update

Procedural texts are captured on how to check against time and distance threshold for indicating FP updates. However, it was not explicitly agreed about the UE behaviour in case neither of the thresholds are configured. Rapporteur assumption was it can be left up to the UE in such case. However, some companies assumed that network shall always configure at least one of the two thresholds. It was further commented that it needs to be discussed and agreed by RAN2. Therefore, Editor’s Notes are captured in 5.3.5.3 and 5.7.4.2.

Editor’s Note: FFS how to capture the UE behavior when neither distance nor time threshold is configured, e.g. leave upto UE with a NOTE like below or add network restriction to always configure at least one threshold. “NOTE x:	If neither flightPathUpdateDistanceThr nor flightPathUpdateTimeThr is configured, it is up to UE implementation whether to include flightPathInfoAvailable when updated flight path information is available.”

Editor’s Note: FFS how to capture the UE behavior when neither distance nor time threshold is configured, e.g. leave upto UE with a NOTE like below or add network restriction to always configure at least one threshold “NOTE x:	If neither flightPathUpdateDistanceThr nor flightPathUpdateTimeThr is configured, it is up to UE implementation whether to initiate transmission of the UEAssistanceInformation message when updated flight path information is available.”

Discuss and agree on one of the following options: a) Add network restriction that either time or distance threshold for triggering FP update (at least one of the two) is always configured; or b) Keep Note x from the running CR in 5.3.5.3 and 5.7.4.2 (i.e. it is up to the UE in case neither of the thresholds is configured).

# Editor’s Note on avoiding duplicate cells in cellsTriggeredList

In section 5.5.4.1, an Editor’s Note is captured based on company comments during the email discussion.

2> if the reportType is set to eventTriggered, and if the corresponding reportConfig includes numberOfTriggeringCells, and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig: 3> if the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event): 4> include a measurement reporting entry within the VarMeasReportList for this measId; 3> if the number of cell(s) in the cellsTriggeredList is larger than or equal to numberOfTriggeringCells: 4> include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId; 3> else: 4> include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId; 4> if the number of cell(s) in the cellsTriggeredList is larger than or equal to numberOfTriggeringCells: 5> set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0; 5> initiate the measurement reporting procedure, as specified in 5.5.5; Editor’s Note: To be checked further whether additional conditions are needed to avoid the existing cells in cellsTriggeredList to be added again to the list (i.e. both instances of “4> include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;” above). This applies to LTE spec also.

There can be multiple ways to interpret the statement “include the concerned cell(s) in the cellsTriggeredList…”. One way to interpret is that ‘include the concerned cell(s)’ is not same as ‘append’, so this should mean only add if not already there in the list. Other way to interpret could be that there is no restriction on duplicate entries, so the list can grow due to duplicate entries.

So far, the size of cellsTriggeredList was not a determining factor (other than when it is empty) in the procedural texts. The risk of duplicate entries was not there in the existing procedures since ‘remove the concerned cell(s) in the cellsTriggeredList’ procedure would anyway remove the duplicates, if any.

Now the new procedural text relies on the “number of cell(s) in the cellsTriggeredList”. This is a new behaviour. So, it is worth discussing whether there is a risk of adding the same cells again to the cellsTriggeredList, thereby reaching the number prematurely by counting duplicate entries.

Discuss and agree on one of the following options: a) RAN2 understanding of existing procedure has been that “include the concerned cell(s) in the cellsTriggeredList” means adding only the cells not already in the list; or b) Add conditions to clarify that only the cells not already in the cellsTriggeredList are added. In case of b), further decide what to for LTE.

# Editor’s Note on Resource Pool for BRID and DAA

RAN2#123 agreed: “RAN2 will support (pre-) configured separate SL resource pool for BRID and DAA broadcast”. Relevant text captured in the running CR to implement this agreement is copied below:

SL-ResourcePool-r16 ::= SEQUENCE { sl-PSCCH-Config-r16 SetupRelease { SL-PSCCH-Config-r16 } OPTIONAL, -- Need M <<skip>> sl-InterUE-CoordinationConfig-r17 SetupRelease { SL-InterUE-CoordinationConfig-r17 } OPTIONAL -- Need M ]], [[ sl-A2X-Service-r18 ENUMERATED {brid, daa, bridAndDAA, spare1} OPTIONAL -- Need M -- Editor’s Note: Whether the above is sufficient to capture the agreement “support (pre-) configured separate SL resource pool for BRID and DAA broadcast” or there is need to differentiate Resource Pool in higher level to prevent other UEs from accessing A2X specific resource pool can be further discussed. ]] }

Rapporteur’s assumption is that to implement the previous agreement, a flag to indicate whether the configured resource pool is for BRID, DAA or both, combined with the following field description would be sufficient without having to add separate SL resource pools from the higher level.

sl-A2X-Service Presence of this field indicates the resource pool is dedicated for A2X service, i.e., not to be used for other than A2X service. Value brid indicates the resource pool is for BRID, value daa indicates the resource pool is for DAA, and value bridaAndDAA indicates the resource poos is for both BRID and DAA. If this field is absent for all the configured resource pools, the UE may choose non-dedidcated resource pool for Aerial UE operation.

The Editor’s Note was captured based on company comments that the above may not be sufficient.

Discuss whether the currently captured changes in SL-ResourcePool-r16 IE is sufficient or there is need to differentiate Resource Pool at higher level to prevent other UEs from accessing A2X specific resource pool.

# Forwarding flight path by source gNB during HO

RAN2#121 agreed:

- Flightpath information should be forwarded from source gNB to target gNB during handover. Send LS to RAN3 to check for feasibility

An LS was sent to RAN3 in R2-2304474. RAN3 replied in R2-2307034/R3-233493 with the following:

RAN3 agreed adding the UAV flight path information into RAN3 specifications for NGAP (TS 38.413), and XNAP (TS 38.423). The agreed TP capturing the UAV flight path information are attached, while the referred IE to RRC is FFS to wait until RAN2 finalization of the definition of UAV fight path information.

In other words, RAN3 has already agreed on baseline CRs. From RAN2 side, the flight path information IE could be included in AS-Context within HandoverPreparationInformation such that it can be transferred between the RAN nodes during the HO preparation. This has been captured in the running CR, however the following proposal is presented for explicit agreement since there was a comment that the change in the CR may not be needed.

During HO preparation, source gNB includes FlightPathInfoReport in AS-Context within HandoverPreparationInformation (already captured in the running CR).

# Summary

In this contribution, we made the following observations:

Observation 1.	In LTE, values for UE height range for configuration and reporting were chosen to cover the lowest exposed point on earth surface (The Dead Sea) to the highest point (The Mt. Everest).

Observation 2.	The LTE encoding of heightUE takes 14 bits, however not all possible codepoints were utilized (14 bits could cover -400..15983).

Observation 3.	The UAVs of the future may be able to go well beyond 8880m and still be provided services by 3GPP cellular technologies such as HAPs and NTN.  So, it is proposed to make the range about 2x of LTE, i.e. add 1 bit in the encoding.

Observation 4.	In LTE, some event triggers have reportOnLeave (A3/A4/A5/A6/C1/C2 etc.), while others do not (e.g. A1/A2/V1/V2/H1/H2). However, in NR, all the event triggers defined thus far have configuration for reportOnLeave by default.

Based on the discussion and observations, following are proposed to resolve the outstanding FFSes and Editor’s Notes in the running CR:

Proposal 1.	Use ‘altitude’ instead of ‘height’ throughout the running CRs.

Proposal 2.	For height-based list of SSB-ToMeasure, number of height ranges is 32.

Proposal 3.	For RRC signalling, the height range (min to max) for UE height (both for configuration and reporting) is -400m to 32,367m above mean sea level, with granularity of 1m.

Proposal 4.	Value range for h1-Hysteresis, h2-Hystereris, heightHyst is 1m to 128m, with granularity of 1m.

Proposal 5.	[Clarification to RAN2#121 agreement] UE height, location and/or velocity reporting without being accompanied by RSRP/RSRQ measurement results is supported and is configurable by the network. This applies to all height-related event triggers.

Proposal 6.	Similar to all other event triggers in NR, allow reportOnLeave and timeToTrigger configuration for all newly introduced events (already captured in the running CR).

Proposal 7.	Distance and time thresholds for FP update can be (re)configured in RRCReconfiguration and RRCResume, but not in other messages like RRCSetup and RRCReestablisment.

Proposal 8.	When UE moves to RRC_IDLE, the thresholds for triggering FP update, if configured, are released.

Proposal 9.	For RRCSetupComplete, UE doesn't check for the threshold(s) configuration for indicating FP availability (i.e. it is always like new FP available indication).

Proposal 10.	Discuss and agree on one of the following options: a) Add network restriction that either time or distance threshold for triggering FP update (at least one of the two) is always configured; or b) Keep Note x from the running CR in 5.3.5.3 and 5.7.4.2 (i.e. it is up to the UE in case neither of the thresholds is configured).

Proposal 11.	Discuss and agree on one of the following options: a) RAN2 understanding of existing procedure has been that “include the concerned cell(s) in the cellsTriggeredList” means adding only the cells not already in the list; or b) Add conditions to clarify that only the cells not already in the cellsTriggeredList are added. In case of b), further decide what to for LTE.

Proposal 12.	Discuss whether the currently captured changes in SL-ResourcePool-r16 IE is sufficient or there is need to differentiate Resource Pool at higher level to prevent other UEs from accessing A2X specific resource pool.

Proposal 13.	During HO preparation, source gNB includes FlightPathInfoReport in AS-Context within HandoverPreparationInformation (already captured in the running CR).