3GPP TSG-RAN2 Meeting #123-bis	R2-2310476

Xiamen, China, Oct. 9-13, 2023	Revision of R2-2307639

Agenda Item:	7.11.2.2

Work Item:	NR_MBS_enh-Core

Source: 	Qualcomm Incorporated

Title: 	Views on the FFS on the multicast CFR configuration aspects

Document for:	Discussion/Decision

# Introduction

Rel-17 MBS added support of multicast services reception by UEs in CM_CONNECTED/ RRC_CONNECTED state only. One of the objectives of Rel-18 WI on enhancements of NR MBS is to enable reception of multicast in RRC_INACTIVE, see RP-221458:

Email discussion [Post121][607][eMBS] discussed the multicast CFR configuration aspects and proposed some conclusions (see report in R2-2303420).  Following were agreed in RAN2#121-bis:

From the location&bandwidth and SCS configuration perspective,  follow R17 MBS broadcast CFR principle (i.e. case A,C,E) to provide multicast CFR configuration in RRC_INACTIVE.

Multicast CFR in RRC_INACTIVE and broadcast CFR can be configured differently. FFS whether we need to restrict that one CFR is completely contained within the other in this case (we should understand what the issue is otherwise).

Case B and case D are not supported for multicast CFR in RRC_INACTIVE;

Whether multicast CFR in RRC_CONNECTED and in RRC_INACTIVE are different is up to NW implementation. FFS whether this causes some issues which need to be addressed.

The same CFR is used for multicast MCCH and MTCH. It can be revisited if there is any issue found, e.g. for RedCap UEs.

In this contribution, we provide further views on CFR for multicast reception in RRC_INACTIVE and one of the outstanding FFS items above.

# BWP/CFR Configuration

Note the previous agreements:

From the location&bandwidth and SCS configuration perspective,  follow R17 MBS broadcast CFR principle (i.e. case A,C,E) to provide multicast CFR configuration in RRC_INACTIVE.

Case B and case D are not supported for multicast CFR in RRC_INACTIVE;

This means for multicast reception in RRC_INACTIVE in Rel-18, the configured multicast CFR for RRC_INACTIVE UEs should have the BW same or larger than CORESET#0, fully overlapping with CORESET#0 and with the same numerology as CORESET#0. Different supported and unsupported example CFR cases are illustrated in the figures below.

Figure 1: Supported CFR cases for Rel-18 Multicast reception in RRC_INACTIVE

Figure 2: Not supported CFR cases for Rel-18 Multicast reception in RRC_INACTIVE

RAN2 further agreed the following:

Multicast CFR in RRC_INACTIVE and broadcast CFR can be configured differently. FFS whether we need to restrict that one CFR is completely contained within the other in this case (we should understand what the issue is otherwise).

Note that a UE may support Rel-17/18 broadcast as well as Rel-18 multicast. Also note that Rel-17/18 broadcast may be received in RRC_INACTIVE state, and Rel-18 multicast will be supported for RRC_INACTIVE state. When the UE needs to receive Rel-17/18 broadcast as well as Rel-18 multicast in RRC_INACTIVE state simultaneously, if multicast CFR in RRC_INACTIVE and broadcast CFR are configured differently, then:

- Both the CFRs need to fully contain CORESET#0

  - Given that only cases A/C/E are supported and case B (i.e. CFR smaller than CORESET#0) is not supported for both Rel-17 broadcast and Rel-18 multicast in RRC_INACTIVE, this condition is always fulfilled.

- One of the two CFRs need to be fully contained (or overlapping) with the other CFR, as illustrated in the figure below. This enables UE to just monitor the larger CFR and be able to receive both services without BWP switch. Otherwise, the UE would need to monitor multiple non-overlapping CFRs to receive both the services.

Figure 3 Scenarios for different configurations of CFR for Broadcast and CFR for Multicast

If multicast CFR in RRC_INACTIVE and broadcast CFR are configured differently, the CFR cases agreed to be supported already ensure that both the CFRs fully contain CORESET#0.

If multicast CFR in RRC_INACTIVE and broadcast CFR are configured differently, to enable UE to monitor a single (the larger) CFR and receive both services without BWP switch, one CFR needs to be completely contained within the other.

Therefore, we propose the following clarification related to the FFS.

When Multicast CFR for RRC_INACTIVE and broadcast CFR are configured differently, one of the two CFRs is fully contained (or overlapping) with the other CFR.

Similar to Rel-17 broadcast, if CFR for multicast is not configured, the default should be same as CORESET#0. One may argue the default CFR can be same as the Broadcast CFR, however not all UEs are required to support MBS broadcast. Therefore, following is proposed:

If multicast CFR for RRC_INACTIVE is not configured, the default is same as CORESET#0.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	If multicast CFR in RRC_INACTIVE and broadcast CFR are configured differently, the CFR cases agreed to be supported already ensure that both the CFRs fully contain CORESET#0.

Observation 2.	If multicast CFR in RRC_INACTIVE and broadcast CFR are configured differently, to enable UE to monitor a single (the larger) CFR and receive both services without BWP switch, one CFR needs to be completely contained within the other.

Based on these observations and discussion above, we have the following proposals:

Proposal 1.	When Multicast CFR for RRC_INACTIVE and broadcast CFR are configured differently, one of the two CFRs is fully contained (or overlapping) with the other CFR.

Proposal 2.	If multicast CFR for RRC_INACTIVE is not configured, the default is same as CORESET#0.