3GPP TSG-RAN WG2 Meeting #R2-123-bis		R2-2310530

Xiamen, China, Oct 09-13, 2023

Agenda Item:	7.4.2.1

Source: 	Rakuten Symphony Inc.

Title:  	Delayed Resource Reservation for inter gNB-DU LTM

Document for:	Discussion and decision

# Introduction

In one of the previous RAN1 meetings, the following agreements/working assumptions were made:

Agreement - For L1 measurement report for Rel-18 L1/L2 mobility, if UE event triggered report for L1 measurement is supported based on further study - At least the following aspects may be considered - How to define UE event and exact definition of events, - Report container - Resource allocation/assignment for UE event triggered report - Necessity of indication to gNB when the condition UE event is met, and how - Necessity to define the condition to start/stop the reporting, - Contents of the report/reporting format, PCI, RS ID, measurement result etc. - The interaction with filtered L1 measurement results (if supported) - Support of simultaneous configuration of both UE event triggered and any of periodic/semi-persistence/aperiodic reporting, and solutions when both of them are configured. - Report destination, whether the report is sent to serving cell only or can be sent to one or more candidate cell(s). - Benefit when L3 measurement is involved

In this contribution, we analyse how the network can benefit by delayed target cell resource reservation if the L1 measurement report of the measurements configured by the serving DU are delivered to the target DU also.

# Discussion

Based on the above highlighted agreement, RAN1 has agreed to study further, the L1 measurement reporting destination, whether the report is sent to serving cell only or can be sent to one or more candidate cell(s).

## Radio resource reservation for LTM

The lead time (duration between LTM HO preparation and execution) in an LTM HO could be unpredictably long. Secondly, not all prepared target cell configurations end up in LTM cell switches. Radio resources are scarce and considering that a UE can be configured with multiple LTM candidate cells, resource reservation (including PRACH preambles) is always going to be an issue. Reservation of radio resources in all the configured LTM candidate cells for a UE leads to sub-optimal performance and overhead on the network. Especially considering that this is applicable on a large scale, for all UEs configured with LTM in the network.

RAN1 has agreed UE performing UL sync to acquire timing advance from the candidate/target cell before the serving cell switch with the objective of enabling a RACH-less LTM cell switch.

Hence, resource reservation is a serious issue considering the framework of LTM and for a successful LTM, optimization of resource reservation is of utmost importance.

Observation 1: For LTM to succeed, optimization of radio resource reservation has to be considered, especially with respect to duration of resources reserved.

## L1 Measurement Reporting to target gNB-DU(s) and delayed resource reservation

Based on the UE’s L1 measurements, the serving gNB-DU may ask the UE to perform UL sync with the candidate/target cell to acquire UE’s candidate/cell timing advance. Hence, UE performing UL sync with the candidate/target cell can be considered to indicate a likelihood of UE undergoing LTM HO to the given target cell.

To address the resource reservation issues, the following could be considered - at the time of preparing an LTM target cell, the gNB-CU could indicate to the candidate/target gNB-DU whether or not the UE is configured to perform UL sync (RACH procedure). Based on this, the target gNB-DU shall not reserve all radio resources immediately at the time of LTM candidate/target cell configuration preparation. At this time, the target gNB-DU may also defer reserving radio resources whose availability is limited.

Based on the UE performing a UL sync operation, the candidate/target gNB-DU can anticipate an incoming LTM cell switch and initiate resource reservation at the candidate/target cell including update of a RACH resource configuration corresponding to the UE’s best beam/beam-group at that point of time.

This introduces multiple benefits

- This ensures that the resource reservation at the target gNB-DU is performed very close to the LTM serving cell switch and hence optimized, as the likelihood of an LTM cell switch is strong. This also prevents unnecessary blocking of resources for a longer duration in case of long lead times.

- This also ensures that the right RACH resources corresponding to the best beam/beam-group for the UE is reserved at the target gNB-DU.

- The UE performing UL sync can be a good indicator of the likelihood of a LTM serving cell change. This enables the target gNB-DU to issue a target cell re-configuration (if required) based on the prevalent resource conditions at that time.

The newly reserved radio resources including the new or modified PRACH preamble, resulting in a modified target cell configuration at the candidate/target gNB-DU could be delivered to the UE via the gNB-CU.

Observation 2: UE performing UL Sync with the candidate/target gNB-DU, based on PDCCH order issued by the serving gNB-DU can be used for delayed resource reservation at the candidate/target gNB-DU.

Triggering data forwarding from the serving gNB-DU, based on the L1 measurements requires awareness of target gNB-DU and its associated gNB-CU-UP, which is an overhead. Hence, UL sync procedure from the UE can also be used by the target gNB-DU to request and trigger downlink data forwarding from the gNB-CU-UP.

Observation 3: UE performing UL Sync with the target gNB-DU, based on PDCCH order issued by the serving gNB-DU can be used by the candidate/target gNB-DU to trigger data forwarding from gNB-CU-UP by the candidate/target gNB-DU.

Observation 4: For the observations 2 and 3 to be realistic, the serving gNB-DU shall be restricted to sending a PDCCH order for TA acquisition to the UE, only when there is a strong likelihood of an LTM cell switch.

Proposal 1: RAN2 discusses and agrees using UL sync performed by UE for TA acquisition to the candidate/target gNB-DU as a trigger for delayed resource reservation.

# Conclusion

We have the following observations:

Observation 1: For LTM to succeed, optimization of radio resource reservation has to be considered, especially with respect to duration of resources reserved.

Observation 2: UE performing UL Sync with the candidate/target gNB-DU, based on PDCCH order issued by the serving gNB-DU can be used for delayed resource reservation at the candidate/target gNB-DU.

Observation 3: UE performing UL Sync with the target gNB-DU, based on PDCCH order issued by the serving gNB-DU can be used by the candidate/target gNB-DU to trigger data forwarding from gNB-CU-UP by the candidate/target gNB-DU.

Observation 4: For the observations 2 and 3 to be realistic, the serving gNB-DU shall be restricted to sending a PDCCH order for TA acquisition to the UE, only when there is a strong likelihood of an LTM cell switch.

We have the following proposals:

Proposal 1: RAN2 discusses and agrees using UL sync performed by UE for TA acquisition to the candidate/target gNB-DU as a trigger for delayed resource reservation.

# Reference

- RP-222332, “Revised WID on Further NR mobility enhancements,” MediaTek