3GPP TSG-RAN WG2 Meeting #123bis               	R2-2310535

Xiamen, CN, 9th – 13th Oct, 2023

Agenda Item:	7.4.5

Source: 	vivo

Title:         	Discussion on fast Scell setup

Document for: 	Discussion and Decision

- Introduction

Currently, RAN4 is discussing parallelly two solutions including solution based on existing measurement and solution based on enhanced measurement to solve the issue related on improvement on FR2 Scell/SCG setup delay. In the RAN4#108 meeting, RAN4 sends an LS and asks RAN2 to take RAN4 agreements in [1] into account and provide signaling input. In this contribution, we will provide our consideration on aspects in [1].

- Discussion

  - Solution based on existing measurement

For the below agreements in LS [1], it has been agreed that, for non-EMR capable UE, it can report the measurement results during idle/inactive mode to the network during or after RRC setup or resume procedures and the NW can specify which band(s) to report. What needs to be discussed in RAN2 is reporting procedure.

RAN4 #108 Agreements: Measurement reporting <Agreement> - For UE which is not capable of EMR, when the measurement results during idle/inactive mode are available and valid, UE can report these results to the network during or after RRC setup or resume procedures. The reporting procedure is up to RAN2. - It is up to UE implementation how to select a band to report unless NW specify a band to report.

Basic reporting procedure on EMR:

For the EMR capable UE, UE can measure target frequencies during RRC_IDLE/INACTIVE according to the EMR configuration. If the UE has available measurement results, the UE can include idleMeasAvailable indication in RRCSetupComplete/RRCResumeComplete message. After receiving this availability indication from the UE, the network can request the UE to report its measurement result via UEInformationRequest message. Then, the UE sends measurement result via UEInformationResponse message. For RRC INACTIVE case, for fast CA/DC setup, when sends the RRC resume message, the network can request the UE to report EMR results in RRC resume complete message.

The reporting procedure for non-EMR capable UE

Form our understanding, the similar procedure can also be applied to reporting of non-EMR capable UE.

Proposal 1: The reporting procedure of EMR can be the baseline of the reporting procedure of the measurement results during idle/inactive mode for non-EMR capable UE.

For non-EMR capable UE, the above RAN4#108 agreements mentioned that the measurement results during idle/inactive mode reported by the UE are available and valid, and the similar agreement in RAN4#107 also mentions that UE does not report the invalid results to network. Currently, the definition of validity is discussing in RAN4 and there is no agreement for that. For the specific detail, we think we can wait for RAN4’s discussion.

RAN4 #107: R4-2310064 - Agreements: In new reporting in R18, UE does not report the invalid results to NW.

After the UE has the available and valid measurement results, the next issue needs to discuss whether the  indication of availability and/or validity status of measurement results need to be indicated to network. Currently, the legacy EMR procedure has supported to report availability indication, the similar indication can also apply to the non-EMR capable UE. However, available measurement results only mean the UE has the measurement results but there is no validity verification. Therefore, in order to prevent the NW from requesting the invalid measurement results, the UE can indicate the validity status to network in RRCSetupComplete/RRCResumeComplete message. This issue has also been discussed in EMR reporting in RAN4 with the below FFS. Similarly, we think the UE can also report the indication of valid EMR results to network in RRCSetupComplete/RRCResumeComplete message.

RAN4 #106bis: <Agreement>: FFS : whether to introduce indication of availability and/or validity status of measurement results from UE to NW into the [existing] EMR reports.

Proposal 2: The non-EMR capable UE can report the indication of availability and validity status of measurement results during idle/inactive mode to the network in RRCSetupComplete or RRCResumeComplete message.

Proposal 3: The EMR capable UE can report the indication of validity status of EMR results to network in RRCSetupComplete or RRCResumeComplete message.

After receiving this availability and validity indication from the UE, the network may not be interested in all available and valid measurement results, or to limit the size of measurement results, the network can request the UE to report the measurement result in specific bands via UEInformationRequest/RRCResume message. Then, the UE sends measurement result via UEInformationResponse/RRCResumeComplete message.

Proposal 4: The UE can be configured by the network which bands should be reported for the measurement results during idle/inactive mode via UEInformationRequest/RRCResume message.

Proposal 5: The UE can send measurement results during idle/inactive mode via UEInformationResponse/RRCResumeComplete message.

Another question is if the reporting procedure on EMR and the reporting procedure for non-EMR capable UE need be decoupled. From the network perspective, for the EMR capable UE, in some cases, the network may not be interested in the measurement results during idle/inactive mode, the network can only request the UE to report the EMR results, which can give the flexibility to network.

Proposal 6: The reporting procedure on EMR and the reporting procedure for non-EMR capable UE can be decoupled.

  - Solution based on enhanced measurement

Starting point of the enhanced measurement

- UE is allowed to perform addition measurement starting from RRC connection setup/resume procedure - For MT originating call, UE starts to perform additional measurement after paging reception. - And for MO call, UE starts to perform additional measurement after first RACH preamble transmission, i.e. Msg1.

For solution based on enhanced measurement, RAN4 has agreed that UE can be allowed to perform additional measurement during RRC connection setup/resume procedure and the corresponding starting point has been defined. However, from the RAN2 understanding, for MO call, the first RACH preamble transmission is not the earliest starting point in RRC connection setup procedure. Therefore, for MO call, we think the starting point can be that the UE initiates the RRC connection setup procedure.

Proposal 7:  RAN2 to discuss whether starting point of the enhanced measurement can be that the UE initiates the RRC connection setup procedure for MO call.

Ending point of the enhanced measurement

- The ending point of enhanced measurement at the reception of the RRC CONNECTED mode measurement configuration (the 1st RRC_reconfiguration message)

Based on the RAN4’s agreement, the ending point of enhanced measurement is at the reception of the RRC CONNECTED mode measurement configuration. From the RAN2 perspective, when the UE enters the RRC_CONNECTED state, the reasonable network behavior is that the network can request UE to report the measurement result if it is available and valid. If there is no available and valid measurement result, the next step, the network configures the RRC_CONNECTED mode measurement configuration. Therefore, we think after the network requests the measurement result from UE and the UE reports the measurement result, the enhanced measurement can be stopped. If there is no available and valid measurement result from UE, the network can explicitly indicate to stop the enhanced measurement.

Proposal 8: RAN2 to discuss the ending point of the enhanced measurement. After the UE reports the enhanced measurement result to network or the network explicitly indicates to stop the enhanced measurement, the enhanced measurement can be stopped.

Measurement configuration

RAN4 mentioned in their LS:

<Agreement>: - Reduce the number of EMR carriers to be measured for improved measurement. - FFS: how to select the frequency layers for improved measurement, how many frequency layers to measure and which scenario that is applicable.

To limit the size of the number of EMR carriers to be measured for enhanced measurement, if the network knows which frequencies can be added as candidate frequencies of SCells exactly, it can indicate the frequencies of enhanced measurement via system information. Specifically, for each frequency, the specific configuration adopted in EMR can be taken as baseline, such as the cell list, measurement configuration of SSB etc.

Proposal 9: The frequencies of enhanced measurement can be configured by the network via system information.

Proposal 10: For the specific frequency configuration of enhanced measurement, per-frequency configuration adopted in EMR can be taken as the baseline.

The reporting procedure for enhanced measurement

According to the previous discussion in Chapter 2.1, the similar reporting procedure can also apply to the enhanced measurement. Therefore, we can give the similar proposals as below:

Proposal 11: The UE can report the indication of availability and validity status of enhanced measurement results to network.

Proposal 12: The UE report enhanced measurement results upon the reception the request from the network.

Proposal 13: The UE is configured by the network which bands should be reported in the enhanced measurement result.

- Conclusion

In this paper, the following proposals are given:

Proposal 1: The reporting procedure of EMR can be the baseline of the reporting procedure of the measurement results during idle/inactive mode for non-EMR capable UE.

Proposal 2: The non-EMR capable UE can report the indication of availability and validity status of measurement results during idle/inactive mode to the network in RRCSetupComplete or RRCResumeComplete message.

Proposal 3: The EMR capable UE can report the indication of validity status of EMR results to network in RRCSetupComplete or RRCResumeComplete message.

Proposal 4: The UE can be configured by the network which bands should be reported for the measurement results during idle/inactive mode via UEInformationRequest/RRCResume message.

Proposal 5: The UE can send measurement results during idle/inactive mode via UEInformationResponse/RRCResumeComplete message.

Proposal 6: The reporting procedure on EMR and the reporting procedure for non-EMR capable UE can be decoupled.

Proposal 7:  RAN2 to discuss whether starting point of the enhanced measurement can be that the UE initiates the RRC connection setup procedure for MO call.

Proposal 8: RAN2 to discuss the ending point of the enhanced measurement. After the UE reports the enhanced measurement result to network or the network explicitly indicates to stop the enhanced measurement, the enhanced measurement can be stopped.

Proposal 9: The frequencies of enhanced measurement can be configured by the network via system information.

Proposal 10: For the specific frequency configuration of enhanced measurement, per-frequency configuration adopted in EMR can be taken as the baseline.

Proposal 11: The UE can report the indication of availability and validity status of enhanced measurement results to network.

Proposal 12: The UE report enhanced measurement results upon the reception the request from the network.

Proposal 13: The UE is configured by the network which bands should be reported in the enhanced measurement result.

- References

[1] R4-2314466, LS on improvement on FR2 SCell/SCG setup delay, RAN4, LS in, Rel-18, NR_Mob_enh2, To:RAN2.