3GPP TSG-RAN WG2 Meeting #123bis									   R2-2310556

Xiamen, China, October 9th – 13th, 2023

Agenda item:		7.3.5

Title: 	The remaining issues on connected mode mobility

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In RAN#98, a new WID [1] of Network energy savings for NR has been agreed. And the objective about CHO enhancement is the following:

- Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2]

In this paper, we will further discuss the CHO enhancement for NES, and discuss the impact on RAN2 specification.

- Discussion

In RAN2#123 meeting, the agreements about mobility enhancements for NES are as below:

Agreements

- We will support the CHO triggers for the use case of turning off the cell.

- (At least for cell DTX/DRX) Time-based CHO is not to be considered in NES.

- Do not consider using an indication in SIB1 for triggering NES CHO execution condition

First of all, in the case of turning off the cell, UE should be handovered to other cell before the serving cell is turned off. Considering turning off the cell generally needs a long term evaluation, gNB could estimate the turning-off time and configure the triggering time of CHO configuration at advance. The time-based CHO combing with Event A3, A4, A5 with a lower threshold could trigger a handover just before the serving cell is turned off. Hence, time-based CHO is considered for the use case of turning off the cell. For the UEs without GNSS, due to the limited number, legacy handover procedure could be used.

Proposal 1: Time-based CHO is considered for the use case of turning off the cell.

Moreover, if the source cell enables cell DTX/ DRX, it means the source cell enters into NES mode. If the source cell enables cell DTX/ DRX, and the discontinuous transmission or reception would cause the UE performance deterioration, a hanover is preferred. From our understanding, the activated DRX/DTX pattern won’t cause the obvious system performance deterioration. The number of UE whose performance will deteriorate due to the activated DRX/DTX pattern is few, thus, the RRC reconfiguration procedure to trigger the handover won’t cause the heavy load of signaling. Specially, a RRC reconfiguration could be send to configure the Event A3, A4, A5 with a lower threshold to trigger a handover before the discontinuous transmission or reception takes effect. In final, the legacy handover procedure is considered for the use case of cell DTX/DRX

Proposal 2: Legacy handover procedure is considered for the use case of cell DTX/DRX.

For the selection of the target cell, considering UE doesn’t know the load and scheduling of the cell, it is unable to distinguish a cell that could allocate the most resource for it. But gNB could obtain the information of the neighbor cell, the information could include the load, NES mode and so on. From network perspective, if the neighbor cell enables NES mode, but it still could supply better service quality for UE, it could be the candidate cell. Hence, it is up to NW implementation to select some candidate cells and configure them to UE via CHO. If there are more than one triggered cells, it is up to UE to select one of the triggered cells as the selected cell for reconfiguration execution.

Proposal 3: It is up to NW implementation to configure the candidate cells for CHO and it is up to UE implementation to select one of the triggered cells if there are more than one, i.e. the existing conditional reconfiguration execution applies.

- Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are given:

Proposal 1: Time-based CHO is considered for the use case of turning off the cell.

Proposal 2: Legacy handover procedure is considered for the use case of cell DTX/DRX.

Proposal 3: It is up to NW implementation to configure the candidate cells for CHO and it is up to UE implementation to select one of the triggered cells if there are more than one, i.e. the existing conditional reconfiguration execution applies.

- References

[1] RP-223540 New WID proposal: Network energy savings for NR