3GPP TSG-RAN WG2 Meeting #123	R2-2310558

XIAMEN, CHINA 9th – 13th october, 2023

Source: 	Vodafone

Title: 	Discussion on Cell DTX and DRX activation

Agenda Item:	7.3.2

Document for:	Discussion and decision

# Introduction

## RAN WG1 took several agreements on the activation/deactivation procedure of Cell DTX/DRX and the signalling needed from the higher layers for this method to work.

At the same time, RAN WG2 agreed:

“As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released.”

The current running stage 2 includes this agreement as:

“Cell DTX/DRX can be activated/deactivated by RRC signalling or L1 group common signalling.”

It has to be highlighted that Vodafone sees RAN WG2 in charge to take overall decisions on activations procedures (well known that there might be opposite examples from the past). The reason for this view is that activation of the procedures should consider not only performance perspective such as delay of activation, but also such aspects like integrity protection, ciphering, user consent, overall need and system complexity and RAN WG2 is the owner of the most Radio protocols, where these aspects are considered.

# Discussion

According to the running stage 3, currently there is no statement that Cell DTX/Cell DRX is activated if the configuration is received. If the activation of the configuration is done by RRC signaling, it would be logical to assume that also de-activation is done by RRC signaling. Once activation of cell DTX/DRX was provided by RRC, the UE does not need to listen to group common L1 signaling using PDCCH/ DCI format 2_X for cell DTX/DRX activation and deactivation.

Proposal 1: If cell DTX/DRX is activated by RRC, it is also de-activated by RRC singling and the UE does not need to listen to group common L1 signaling using PDCCH/ DCI format 2_X once cell DTX DRX is activated by RRC signalling.

As per RAN WG1 agreement the DCI format for cell DTX/DRX activation/deactivation will be associated with a particular L1 related configuration (RP-231926):

Agreement:

The following high layer signalling are to be included to the RRC parameter list for new DCI format 2_X for activation and deactivation of cell DTX/DRX

- search space set configuration with new DCI format 2_X

- DCI size for new DCI format 2_X

Proposal 2: If RRC configuration includes L1 signaling, the activation of Cell DTX/DRX and also de-activation of Cell DTX/DRX is be done by group common L1 signaling.

Activation time:

Looking into agreements from RAN WG1:

It seems that even using the L1 common signaling, there is a delay between the receiving of L1 common signaling and the activation of the feature.

In case of RRC signaling, the activation of Cell DTX/DRX will be done once RRC connection reconfiguration will be received. According to the current specification the processing delay of RRC connection reconfiguration is between 10 ms and 16+(Nseg-1)*10 ms.

Taking these delay numbers into account, it is proposed to discuss what is the expected delay between the reception of RRC connection reconfiguration and the point of time the UE would apply provided configuration.

Observation 1: Both ways L1 and RRC introduce delay

Currently L1 is an optional feature and is subject to capabilities and therefore it cannot be expected that all UEs in the cell will support it. This will lead to the situation that gNB would need to send RRC Reconfiguration message to a part of UEs in the cell earlier to be sure they can activate the feature in time and L1 signaling later as it can activate cell DTX/DRX a few ms earlier.

Such operation will result in higher testing effort and increased costs on the operator site without any considerable benefit. RAN WG2 highlighted in their LS (R2-2304568) to benefits of having L1 signaling

- Reduced signaling overhead caused by multiple dedicated RRC messages (group common signaling)

- More dynamic changing than RRC signaling (however, RAN2 did not evaluate the network energy saving gain by reducing the latency of activation/deactivation with L1 signaling for more dynamic changing)

Reduced signaling overhead: This benefit is rather questionable as L1 signaling is intended to be optional and consequently not be supported by all UEs

More dynamic changing than RRC signaling: After it is clear that even L1 signaling is associated with a delay of 3ms, the benefit of using L1 becomes smaller compared to RRC signaling (10 ms delay).

Considering above it is proposed to discuss how these 2 ways of activation could co-exist and if both ways are really needed. If the opinion of the group is still that 2 ways are needed, it is crucial not to make network operation more complex and expensive (e.g. higher integration costs) and therefore, it is proposed to make the support of both ways of activation mandatory from the UE perspective, which would allow operator to rely on one way of activation in a particular deployment.

Proposal 3: If there is still no consensus to keep only one way of cell DTX/DRX activation, it is proposed to make the support of both ways of activation mandatory from the UE perspective.

# Summary

Proposal 1: If L1 signaling is not configured in RRC, cell DTX/DRX is activated/deactivated by RRC and the UE does not need to listen to group common L1 signaling using PDCCH/ DCI format 2_X

Proposal 2: If RRC configuration includes L1 signaling configuration, the activation of Cell DTX/DRX and also de-activation of Cell DTX/DRX shall be done by group common L1 signaling.

Observation 1: The delay between reception of the RRC connection reconfiguration and the activation is around 10 ms and RAN WG2 need to discuss when Cell DTX/DRX is activated once RRC connection reconfiguration is received.

Proposal 3: If there is still no consensus to keep only one way of cell DTX/DRX activation, it is proposed to make the support of both ways of activation mandatory from the UE perspective.