3GPP TSG-RAN WG2 Meeting #123bis										   R2-2310582

Xiamen, China, October 9th – 13th, 2023

Source: 	Xiaomi, vivo

Title:  	Clarification on srs-TxSwitch and MIMO-layer for MUSIM

Agenda Item:	7.17.3

Document for:	Discussion and Decision

# Introduction

According to the temporary capability restriction discussion for Rel-18 MUSIM in RAN2#123 meeting, RAN2 made the following agreements:

1: The UE can indicate that some frequencies (e.g. frequency ranges, bands or BCs) are impacted by NW B so that they are: - 1) forbidden because of collision - 2) having restricted (lower) capabilities (e.g. with lower MIMO layer). 4: The restrictions can apply to CA, DC and/or single CC. 5: The UL/DL MIMO layer and/or the UL/DL supported bandwidth restriction (if supported) shall only work for the restricted frequencies for the proactive case. 1 For dual active MUSIM, the UE can indicate the temporary maximum MIMO layers for specific serving cells for both UL and DL. 3 Maximum MIMO layers/bandwidth restriction is reported per CC ((FFS how we signal this). FFS whether we support indicating temporary capability restrictions on SRS Tx switching capability. FFS whether this could be already indicated by the MIMO layer restrictions.

In this contribution, we provide some clarification on the MUSIM UE implementations for srs-TxSwitch and MIMO-layer, and the necessity of reporting the temporary capability restrictions on SRS Tx switching capability.

# Discussion

## SRS Tx switching and MIMO layer

The detailed capability descriptions for srs-TxSwitch from 38.306 and 38.331 are quoted as follows:

Observation 1: The number of Tx antennas “x” for “supportedSRS-TxPortSwitch” is smaller than or equal to the number of Rx antennas “y” for “supportedSRS-TxPortSwitch”.

Observation 2: The “y” number of Rx antennas for “supportedSRS-TxPortSwitch” is smaller than or equal to the “a” number of UE receive antennas.

The details on how the capability of srs-TxSwitch is used can be found in the section 6.2.1.2 of 38.214. Some examples for SRS Tx switching function can be found in the Annex. Some signalling procedures for “UE sounding procedure for DL CSI acquisition” using the SRS Tx switching function are quoted from 38.214 as follows:

#### 6.2.1.2 UE sounding procedure for DL CSI acquisition When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitch ('t1r2' for 1T2R, 't1r1-t1r2' for 1T=1R/1T2R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't1r6' for 1T6R, 't1r8' for 1T8R, 't2r6' for 2T6R, 't2r8' for 2T8R, 't4r8' for 4T8R, 't1r1-t1r2-t1r4' for 1T=1R/1T2R/1T4R, 't1r4-t2r4' for 1T4R/2T4R, 't1r1-t1r2-t2r2-t2r4' for 1T=1R/1T2R/2T=2R/2T4R, 't1r1-t1r2-t2r2-t1r4-t2r4' for 1T=1R/1T2R/2T=2R/1T4R/2T4R, 't1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r1-t2r2' for 1T=1R/2T=2R, 't4r4' for 4T=4R, or 't1r1-t2r2-t4r4' for 1T=1R/2T=2R/4T=4R): …

The detailed capability descriptions for DL MIMO layer from 38.306 and 38.331 are quoted as follows:

According to the PHY layer processing procedure, the maximum number of (DL/UL) MIMO layers should be smaller than or equal to the number of UE (Rx/Tx) antennas.

Observation 3: The “b” number of “MIMO-LayersDL” is smaller than or equal to the “a” number of UE receive antennas.

Figure 1: Examples for the MUSIM UE implementation regarding SRS Tx switching and MIMO layer

According to the MUSIM UE implementation, when the UE switches from one active SIM-A to two active SIM-A and SIM-B, the UE capability for SIM-A can be reduced. As the examples illustrated in Figure 1, when the UE reduces the number of DL MIMO layers of SIM-A, the number of the Rx antenna for SRS Tx switching of SIM-A may/may not be reduced, as highlighted in brown. On the other hand, the UE changing from one active SIM-A to two active SIM-A and SIM-B could keep the number of DL MIMO layers unchanged for SIM-A, but may/may not reduce the number of the Rx antenna for SRS Tx switching of SIM-A, as highlighted in blue. Thus, the reporting of the maximum MIMO layers restriction cannot reflect the SRS Tx switching restriction.

Observation 4: When the UE reports the reduced number of DL MIMO-layer for SIM-A, the number of Rx antennas for “supportedSRS-TxPortSwitch” may/may not be reduced.

Observation 5: When the UE does not report the reduced number of DL MIMO-layer for SIM-A, the number of Rx antennas for “supportedSRS-TxPortSwitch” may be reduced.

## Reporting of the SRS Tx switching capability restriction

According to the analysis given above, the UE should be allowed to report the SRS Tx switching capability restriction to improve the downlink throughput.

According to the endorsed RRC running CR for Rel-18 MUSIM as quoted below, the UE can report its affected frequency and the affected capability for the affected frequency.

MUSIM-CapabilityRestrictedBandParameters ::= SEQUENCE { bandEntryIndex INTEGER (1..maxSimultaneousBands), musim-capabilityRestricted-r18 FFS }

According to 38.331 and 38.306, srs-TxSwitch capability is reported per band per band combination. Thus, we consider that the reporting of the SRS Tx switching capability restriction can be per band.

Proposal 1: The SRS Tx switching capability restriction is reported for the restricted band.

As provided in “Section 2.1 SRS Tx switching and MIMO layer”, the values for the SRS Tx switching capability (i.e. supportedSRS-TxPortSwitch) include: {t1r2, t1r4, t2r4, t1r4-t2r4, t1r1, t2r2, t4r4, notSupported, t1r1-t1r2, t1r1-t1r2-t1r4, t1r1-t1r2-t2r2-t2r4, t1r1-t1r2-t2r2-t1r4-t2r4, t1r1-t2r2, t1r1-t2r2-t4r4}. Since the SRS Tx switching capability of SIM-A can be changed due to the activation/deactivation of SIM-B, we think that the UE can indicate its supported supportedSRS-TxPortSwitch value for SIM-A due to the activation/deactivation of SIM-B.

Proposal 2: The values for SRS Tx switching capability restriction (i.e. supportedSRS-TxPortSwitch) include: {t1r2, t1r4, t2r4, t1r1, t2r2, t4r4, notSupported}.

# Conclusions

According to the analysis given above, we have the following observations and proposal:

Observation 1: The “x” number of Tx antennas for “supportedSRS-TxPortSwitch” is smaller than or equal to the “y” number of Rx antennas for “supportedSRS-TxPortSwitch”.

Observation 2: The “y” number of Rx antennas for “supportedSRS-TxPortSwitch” is smaller than or equal to the “a” number of UE receive antennas.

Observation 3: The “b” number of “MIMO-LayersDL” is smaller than or equal to the “a” number of UE receive antennas.

Observation 4: When the UE reports the reduced number of DL MIMO-layer for SIM-A, the number of Rx antennas for “supportedSRS-TxPortSwitch” may/may not be reduced.

Observation 5: When the UE does not report the reduced number of DL MIMO-layer for SIM-A, the number of Rx antennas for “supportedSRS-TxPortSwitch” may be reduced.

Proposal 1: The SRS Tx switching capability restriction is reported for the restricted band.

Proposal 2: The values for SRS Tx switching capability restriction (i.e. supportedSRS-TxPortSwitch) include: {t1r2, t1r4, t2r4, t1r1, t2r2, t4r4, notSupported}.

# Reference

[1] RP-220955, vivo, “Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR”

[2] 3GPP TS 38.306, “NR; User Equipment (UE) radio access capabilities”

[3] 3GPP TS 38.331, “NR; Radio Resource Control (RRC) protocol specification”

[4] 3GPP TS 38.214, “NR; Physical layer procedures for data”

[5] https://www.sharetechnote.com/html/5G/5G_SRS.html

[6] R2-2309307, vivo, “Running RRC CR for NR MUSIM enhancements”

# Annex

The followings are quoted from [5]: