3GPP TSG-RAN WG2 Meeting #123bis										   R2-2310584

Xiamen, China, October 9th – 13th, 2023

Source: 	Xiaomi

Title:  	Remaining issues of MUSIM gap

Agenda Item:	7.17.4

Document for:	Discussion and Decision

# Introduction

According to the “Running RRC CR for NR MUSIM enhancements” [1], the following FFS issues need to be discussed for the MUSIM gap priority.

- Editor note: FFS whether prohibit timer is needed for the signaling of MUSIM gap(s) priority preference. - Editor’s Note: Pending to RAN4 discussion on whether aperiodic gap can have a preferred gap priority. - Editor’s Note: FFS musim-GapPriorityToAddModList-r18 is for aperodic MUSIM gap.

According to the RAN4 LS [2] for the gap collision handling, RAN4 made the following agreements:

- Agreement (from RAN4#107): - Define two solutions for collision handling between different MUSIM gaps - 1) Priority based solution (i.e., network controls the MUSIM gaps priority) - 2) “Keep” solution (i.e., keep all collided MUSIM gaps) - FFS on the mechanism to select and/or switch between the solutions

- Agreement (in RAN4#108): - Introduce signalling to allow UE to request to use “keep solution” collision handling mechanism for requested aperiodic and periodic MUSIM gaps and network to grant UE the use of “keep solution”. The same request applies for all MUSIM gaps altogether (i.e. one bit indication). Signalling design is up to RAN2.

- Agreement (in RAN4#108) - Aperiodic MUSIM gap is always kept (not dropped) from UE perspective in case of collisions with other gaps (i.e. all gaps including MUSIM gaps, MGs, etc) - The gap priority level is not explicitly configured by the NW

When “keep solution” is used, the UE keeps all colliding MUSIM gaps irrespective of the priority of the MUSIM gaps.

In this contribution, we provide our understandings on the remaining issues for handling the MUSIM gaps.

# Discussion

## UE preferred gap priority

According to the RAN4 LS in [2] and the RAN4#106bis agreements as quoted below, it seems that RAN4’s discussion on the MUSIM aperiodic gap is focusing on the “keep solution” (i.e. Option 2), and it is still unclear whether the UE is allowed to report its preferred gap priority for the aperiodic gap of MUSIM or whether the aperiodic gap of MUSIM can be configured with a gap priority, when “keep solution” is not enabled. Thus, we think that RAN2 can send an LS to ask RAN4 to clarify the corresponding gap priority configuration/reporting of the aperiodic gap of MUSIM for “Priority based solution”.

RAN4#106bis meeting minutes: Issue 2-1-5: Priority setting for aperiodic MUSIM gaps Agreements from GTW - Option 1 - The priority level of aperiodic MUSIM gap can be configured by NW A - If the priority level is not configured by NW A then the aperiodic MUSIM gap by default has the highest priority level - The aperiodic MUSIM gap priority level can be optionally requested by UE from NW A - Option 2 - Aperiodic MUSIM gap by default has the highest priority level. - The gap priority level is not explicitly configured by the NW

Proposal 1: RAN2 is kindly asked to send an LS to RAN4 to clarify the follow issues for MUSIM aperiodic gap for “priority based solution”:

- Whether gap priority can be configured for MUSIM aperiodic gap

- Whether the UE is allowed to report its preferred gap priority for MUSIM aperiodic gap

## Prohibit timer for MUSIM gap(s) priority preference

According to 3GPP TS 38.331, the reporting of the Rel-17 UE assistance information for MUSIM is controlled by a prohibit timer T346h, as quoted below:

According to the running RRC CR [1] for Rel-18 MUSIM as quoted below, the MUSIM gap priority can only be reported when the Rel-17 musim-GapPreferenceList-r17 is also reported.

Thus, we think that the Rel-17 T346h should be reused for controlling the reporting of the Rel-18 MUSIM gap(s) priority preference.

Proposal 2: T346h is reused for prohibiting the MUSIM gap priority preference reporting.

## “keep solution” for MUSIM gap collision handling

Figure 1: Procedure of "keep solution"

According to the RAN4 LS [2], the “keep solution” for MUSIM gap would include the following steps (as also illustrated in Figure 1):

- Step 1: The gNB configures whether the UE is allowed to report the request of “keep solution”.

- Step 2: If the UE is allowed to report the request of “keep solution”, the UE may report the indication of requesting the “keep solution” for all MUSIM gaps.

- Step 3: If “keep solution” is enabled, the UE keeps all colliding MUSIM gaps irrespective of the priority of the MUSIM gaps.

Regarding the corresponding RAN2 specification impacts, we think that the 1 bit on/off configuration in otherConfig can be provided from the gNB to the UE in Step 1 to allow the UE reporting for the request of “keep solution”.

Proposal 3: The gNB can configure (i.e. 1 bit in otherConfig) whether the UE is allowed to report the request of “keep solution”.

In Step 2, we think that the UE should be allowed to request to enable or disable “keep solution”, since the UE’s preference on the MUSIM gap handling (i.e. preference on “Priority based solution” or ““Keep” solution”) could change when the ongoing service changes in the SIM-A. The UE requesting signalling for enabling or disabling “keep solution” can be included in UAI. For example, the reporting of keepMUSIM-Gap-Request can mean that the UE requests the configuration of “keep solution”. The absence of keepMUSIM-Gap-Request in UAI can mean that the UE does not request the configuration of “keep solution” any more.

Proposal 4: The UE can request “keep solution” for all MUSIM gaps in UAI.

In Step 3, the gNB can provide 1 bit configuration to enable the “keep solution” for all MUSIM gaps configured for the UE.

Proposal 3: The gNB can configure whether “keep solution” (i.e. 1 bit in RRCReconfiguration) is enabled for all MUSIM gaps.

Regarding the detailed UE behaviours on the MUSIM gap handling of the “Priority based solution” or the ““Keep” solution”, we think that those UE behaviours can be captured in the RAN4 specification of 3GPP TS 38.133 [4], as Rel-17.

# Conclusions

According to the analysis given above, we have the following proposals:

Proposal 1: RAN2 is kindly asked to send an LS to RAN4 to clarify the follow issues for MUSIM aperiodic gap for “priority based solution”:

- Whether gap priority can be configured for MUSIM aperiodic gap

- Whether the UE is allowed to report its preferred gap priority for MUSIM aperiodic gap

Proposal 2: T346h is reused for prohibiting the MUSIM gap priority preference reporting.

Proposal 3: The gNB can configure (i.e. 1 bit in otherConfig) whether the UE is allowed to report the request of “keep solution”.

Proposal 4: The UE can request “keep solution” for all MUSIM gaps in UAI.

Proposal 3: The gNB can configure whether “keep solution” (i.e. 1 bit in RRCReconfiguration) is enabled for all MUSIM gaps.

# Reference

[1] R2-2309307, vivo, “Running RRC CR for NR MUSIM enhancements”

[2] R2-2309461, R4-2314465 from RAN4 to RAN2, “LS on Dual Tx/Rx Multi-SIM”

[3] 3GPP TS 38.331, “NR; Radio Resource Control (RRC) protocol specification”.

[4] 3GPP TS 38.133, “NR; Requirements for support of radio resource management”.