3GPP TSG-RAN WG2 Meeting #123bis										   R2-2310587

Xiamen, China, October 9th – 13th, 2023

Source: 	Xiaomi

Title:  	Remaining issues for 2 TA handling of mTRP

Agenda Item:	7.20.2

Document for:	Discussion and Decision

# Introduction

According to the previous RAN2 discussion on the 2 TA management of mTRP, RAN2 achieved the following agreements:

RAN2#122 meeting agreements [4]: - Configure one TAT per TAG to support two TAs for a serving cell, i.e., in this case 2 TAGs are configured for the serving cell.

RAN2#123 meeting agreements [5]: Each joint/UL TCI state is associated with either TAG1 or TAG2 by RRC configuration. RAN2 do not assume any restriction on grouping serving cells/TRPs to TAGs unless RAN1 indication comes. RAN2 assumes the current 4 TAGs per cell group is sufficient to support Rel-18 mDCI mTRP with 2 TAs. At least when both TATs for a SpCell are expired, 1-8 are applied to all TRPs of all serving cells. At least when both TATs for a SCell are expired (assuming PTAG(s) of the cell group still running), 1-7 are applied to all TRPs associated to the TAG with the expired TAT (including both TRPs of the concerned SCell). At least when both TATs for a SpCell are expired, 1-8 are applied to all TRPs of all serving cells. At least when both TATs for a SCell are expired (assuming PTAG(s) of the cell group still running), 1-7 are applied to all TRPs associated to the TAG with the expired TAT (including both TRPs of the concerned SCell). Working assumption: We will use the 2-PTAG model, i.e., both TAGs of SpCell are PTAGs; - When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired, - when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired. 1. not perform any uplink transmission except the Random Access Preamble and MSGA transmission; 2. flush all HARQ buffers; 3. notify RRC to release PUCCH, if configured; 4. notify RRC to release SRS, if configured; 5. clear any configured downlink assignments and configured uplink grants; 6. clear any PUSCH resource for semi-persistent CSI reporting; 7. maintain NTA (defined in TS 38.211 [8]) of this TAG; 8. consider all running timeAlignmentTimers as expired. For inter-cell PDCCH order CFRA to the additionalPCI, - PDCCH order indicates which additionalPCI’s PRACH configuration to be used (according to RAN1 agreement), The following is taken as baseline (for intra-cell case): for CBRA, we reuse the mechanism agreed for CFRA case, i.e. use the RA RAR to indicate the TAG.

In this contribution, we discuss the remaining issues for handling the mTRP of 2 TAs.

# Discussion

## Modelling of PTAG or STAG

As indicated in the above RAN2 agreement, the 2-PTAG model as a working assumption needs to be confirmed by RAN2.

38.321: Timing Advance Group: A group of Serving Cells that is configured by RRC and that, for the cells with a UL configured, using the same timing reference cell and the same Timing Advance value. A Timing Advance Group containing the SpCell of a MAC entity is referred to as Primary Timing Advance Group (PTAG), whereas the term Secondary Timing Advance Group (STAG) refers to other TAGs.

As quoted above, the PTAG needs to include at least SpCell and zero or more SCell(s). The STAG includes one or more SCell(s). When an SpCell is configured with two TAGs, if one TAG of SpCell is defined as STAG, RAN2 needs to redefine the concept of STAG and differentiate the UE behaviours for Rel-18 STAG. One the other hand, according to the above UE behaviours following RAN1 agreements and RAN2 agreements, the UE behaviours related to the TAT expiry of 2 TAs of SpCell/SCell would need to be specified in the MAC specification, no matter whether RAN2 uses 2-PTAG mode or PTAG+STAG mode. Using 2-PTAG mode would require less specification clarification on changing the concept of STAG. Thus, we think that RAN2 should confirm that the 2-PTAG mode is used when SpCell is configured with two TAGs.

Proposal 1: RAN2 is kindly requested to confirm that the 2-PTAG model (i.e., both TAGs of SpCell are PTAGs) is used.

## TAT expiry for two TAGs of a serving cell

List of the intended UE behaviours for TAT expiry: 1. not perform any uplink transmission except the Random Access Preamble and MSGA transmission; 2. flush all HARQ buffers; 3. notify RRC to release PUCCH, if configured; 4. notify RRC to release SRS, if configured; 5. clear any configured downlink assignments and configured uplink grants; 6. clear any PUSCH resource for semi-persistent CSI reporting; 7. maintain NTA (defined in TS 38.211 [8]) of this TAG; 8. consider all running timeAlignmentTimers as expired.

Working assumption: We will use the 2-PTAG model, i.e., both TAGs of SpCell are PTAGs; - When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired, - when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired.

According to the RAN1 feedbacks, “when the TA timer associated to one TRP expires for a TAG associated with a TCI state, UL or DL operation associated to the another TRP is not impacted”.

As provided above, RAN2 intends to identify which of the above UE behaviours need to be perform at the TAT expiry. It is still FFS whether 2-6 are applied to the TRP with TAT expired.

For “2”, as the HARQ entity is per cell, the HARQ buffer of the serving cell should not be flushed when one TRP still has a running TAT.

For “3/4/5/6”, the RRC configured UL/DL resources (i.e. PUCCH/SRS/Configured DL/Configured UL/SP-CSI) can still be used when one TRP still has a running TAT, as RAN1 does not introduce per-TRP RRC configuration for the corresponding radio resources. If one TRP is not able to work due to the TAT expiry of the TRP, the gNB can indicate the UE to use another TRP with a new TCI-state, via MAC CE or DCI.

Proposal 2: When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), the UE does not perform the following behaviours for this serving cell:

- flush all HARQ buffers;

- notify RRC to release PUCCH, if configured;

- notify RRC to release SRS, if configured;

- clear any configured downlink assignments and configured uplink grants;

- clear any PUSCH resource for semi-persistent CSI reporting;

Proposal 3: when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), the UE does not perform the following behaviours for this serving cell:

- flush all HARQ buffers;

- notify RRC to release PUCCH, if configured;

- notify RRC to release SRS, if configured;

- clear any configured downlink assignments and configured uplink grants;

- clear any PUSCH resource for semi-persistent CSI reporting;

## Maximum uplink transmission timing difference between TAGs

38.321: When the MAC entity stops uplink transmissions for an SCell due to the fact that the maximum uplink transmission timing difference between TAGs of the MAC entity or the maximum uplink transmission timing difference between TAGs of any MAC entity of the UE is exceeded, the MAC entity considers the timeAlignmentTimer associated with the SCell as expired.

As quoted above from the 38.321, it is possible that when two TAGs are configured for a serving cell, the uplink transmission timing difference between one TAG of this serving cell and another TAG is more than the maximum uplink transmission timing difference between TAGs. Then the UE is not able to perform UL transmission for these two TAGs. From our understanding, the legacy requirements for the maximum uplink transmission timing difference between TAGs should still be applied, and the UE should stop the uplink transmission towards one TAG by considering the corresponding TAT as expired.

Proposal 4: When the maximum uplink transmission timing difference between TAGs is exceeded, the TAT associated with one TAG is considered as expired, as legacy.

To capture the above RAN2 agreements and our Proposals, we also provided a TP in Annex B, to reflects the specification impacts in MAC.

Observation: The TP capturing RAN2 agreements and the Proposals can be found in Annex B.

# Conclusion

According to the above analysis, we have the following observation and proposals.

Observation: The TP capturing RAN2 agreements and the Proposals can be found in Annex B.

Proposal 1: RAN2 is kindly requested to confirm that the 2-PTAG model (i.e., both TAGs of SpCell are PTAGs) is used.

Proposal 2: When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), the UE does not perform the following behaviours for this serving cell:

- flush all HARQ buffers;

- notify RRC to release PUCCH, if configured;

- notify RRC to release SRS, if configured;

- clear any configured downlink assignments and configured uplink grants;

- clear any PUSCH resource for semi-persistent CSI reporting;

Proposal 3: when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), the UE does not perform the following behaviours for this serving cell:

- flush all HARQ buffers;

- notify RRC to release PUCCH, if configured;

- notify RRC to release SRS, if configured;

- clear any configured downlink assignments and configured uplink grants;

- clear any PUSCH resource for semi-persistent CSI reporting;

Proposal 4: When the maximum uplink transmission timing difference between TAGs is exceeded, the TAT associated with one TAG is considered as expired, as legacy.

# Reference

[1] R1-2302226, LSin from RAN1, “LS to RAN2/4 on Agreements for Rel-18 MIMO”

[2] R1-2213004, LSin from RAN1, “LS on Multi-DCI Multi-TRP with two TAs”

[3] R1-2306249, LSin from RAN1, “Reply on LS 2TA for multi-DCI multi-TRP”

[4] RAN2#122 meeting minutes.

[5] RAN2#123 meeting minutes.

# Annex A

The followings are extracted from the RAN1 reply LS in [3]:

Question 1 on TAG grouping RAN2 discussed how the cells/TRPs configured for the UE, are to be grouped if UE is configured with two TA groups per serving cell. Currently, NR does not impose any requirements in configuring the association of serving cells and TAGs. Q1a: For the 2TA operation, are there any restrictions on the association of serving cells and/or TRPs to the TAGs? Answer: Apart from the agreements RAN1 has sent in LS R1-2302226 to RAN2 before, RAN1 has not agreed to any further restrictions on the association of serving cells and/or TRPs to the TAGs at this point. If RAN1 agrees to such restrictions, RAN1 will inform RAN2. Q1b: NR currently supports up to 4 TAGs per cell group. Are the 4 TAGs enough or does RAN1 see a need to increase the number of TAGs per cell group? Answer: RAN1 has not reached consensus to increase the current number of TAGs per cell group. Question 2 on operation Q2: When the time-alignment timer associated with one of the TRPs of a serving cell expires, are certain UL or DL operation only impacted towards that TRP while they are not impacted towards the another TRP? If so, which UL or DL operation? Answer: RAN1 confirms that when the TA timer associated to one TRP expires for a TAG associated with a TCI state, UL or DL operation associated to the another TRP is not impacted. This further depends on PTAG/STAG definition, which is up to RAN2 to decide. Which UL or DL operation is impacted have not been discussed in RAN1.

# Annex B

According to the above proposals, we have to following text proposal:

Text proposal for 2 TAs of a cell in 3GPP TS 38.321: 1> when a timeAlignmentTimer expires: 2> if the timeAlignmentTimer is associated with the PTAG, and the SpCell includes only one TAG; 2> else if the timeAlignmentTimer is associated with the PTAG, and the SpCell includes two TAG(s) for which another timeAlignmentTimer also expires or is not running: 3> flush all HARQ buffers for all Serving Cells; 3> notify RRC to release PUCCH for all Serving Cells, if configured; 3> notify RRC to release SRS for all Serving Cells, if configured; 3> clear any configured downlink assignments and configured uplink grants; 3> clear any PUSCH resource for semi-persistent CSI reporting; 3> consider all running timeAlignmentTimers as expired; 3> maintain NTA (defined in TS 38.211 [8]) of all TAGs. 2> else if the timeAlignmentTimer is associated with an STAG, then for all SCells belonging to this TAG when the SCell includes only one TAG; 2> else if the timeAlignmentTimer is associated with an STAG, and for all SCells belonging to this TAG when the Serving Cell includes two TAG(s) for which another timeAlignmentTimer(s) also expires or is not running: 3> flush all HARQ buffers; 3> notify RRC to release PUCCH, if configured; 3> notify RRC to release SRS, if configured; 3> clear any configured downlink assignments and configured uplink grants; 3> clear any PUSCH resource for semi-persistent CSI reporting; 3> maintain NTA (defined in TS 38.211 [8]) of this TAG. 2> if the timeAlignmentTimer is associated with the PTAG or an STAG, then for the Serving cells belong to this TAG when the Serving cell includes two TAG(s) for which another timeAlignmentTimer is running: 3> stop all UL transmission belonging to this TAG except PRACH and MsgA; 3> maintain NTA (defined in TS 38.211 [8]) of this TAG.

When the MAC entity stops uplink transmissions for an SCell or a TAG of a serving cell when two TAGs are configured for the serving cell due to the fact that the maximum uplink transmission timing difference between TAGs of the MAC entity or the maximum uplink transmission timing difference between TAGs of any MAC entity of the UE is exceeded, the MAC entity considers the timeAlignmentTimer associated with the SCell or a TAG of a serving cell when two TAGs are configured for the serving cell as expired. The MAC entity shall not perform any uplink transmission on a Serving Cell except the Random Access Preamble and MSGA transmission when any timeAlignmentTimer associated with the TAG to which this Serving Cell belongs is not running, CG-SDT procedure is not ongoing and SRS transmission in RRC_INACTIVE as in clause 5.26 is not on-going. Furthermore, when any timeAlignmentTimer associated with the SpCell is not running, CG-SDT procedure is not ongoing and SRS transmission in RRC_INACTIVE as in clause 5.26 is not ongoing, the MAC entity shall not perform any uplink transmission on any Serving Cell except the Random Access Preamble and MSGA transmission on the SpCell. The MAC entity shall not perform any uplink transmission except the Random Access Preamble and MSGA transmission when the cg-SDT-TimeAlignmentTimer is not running during the ongoing CG-SDT procedure as triggered in clause 5.27 and the inactivePosSRS-TimeAlignmentTimer is not running.