3GPP TSG-RAN WG2 Meeting #123bis											R2-2310609

Xiamen, China, October 9th – 13th, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	On flightpath reporting

Agenda item:		7.8.3

Document for:		Discussion and Decision

- Introduction

In RAN2#121-bis meeting, following agreements were reached.

Agreements:

- Flightpath update indication in UAI is configurable by the network

- Maximum number of waypoints is set to 20 same as in LTE and number of waypoints is configurable by network as in LTE

- Flightpath information should be forwarded from source gNB to target gNB during handover. Send LS to RAN3 to check for feasibility [LS to RAN3 over email 307]

- As a baseline, we can consider a simple network control mechanisms (e.g. a threshold(s)) that controls triggering the flightpath update indication in UAI. FFS if new threshold or the kind of threshold(s)

- As a baseline, single indication is used for both initial and updated flightpath available (i.e. same flag is used for initial and updated flightpath indication.  FFS if further differentiation is needed if we decide to have delta signaling

In RAN2#122[2], following agreements were reached:

Agreements

- The network can configure the UE to trigger a flightpath update notification based on a configured delta time (when timestamp is configured to be reported) or distance configuration.

In RAN2#123[3], following agreements were reached:

Flight path update/reporting

- No delta signaling for updated flight path will be supported

- Confirm that same indication is used for both initial and updated flightpath plan

- If any waypoint/timestamp has changed more than the configured threshold the UE triggers flightpath update indication.

- When both distance and time threshold are configured, the flightpath update can be triggered if either of those conditions are met.   No prohibit timer will be introduced.

This contribution provides further consideration on the remaining issues.

- Discussion

It was agreed the network can configure the UE to trigger flightpath update notification based on a configured delta time or distance threshold. These conditions are related to waypoints that are already reported to network. However, it is still unclear whether UE can trigger flightpath update notification due to adding new waypoints or removing reported waypoints. In our understanding, adding or removing of a waypoint is not covered by the time/distance threshold configuration. And we think UE should be allowed to trigger flightpath update notification when waypoints are added or removed.

Proposal 1: Flightpath update indication can be triggered due to adding or removing waypoints.

If proposal 1 is agreed, one related issue is whether outdated waypoints, i.e. the waypoints that are traveled through or whose timestamp elapsed, can be regarded as removal of waypoint and trigger flightpath update notification. In our understanding, the flightpath plan is well planed and it is meaningful to trigger update if the plan is changed. However, a waypoint is traveled through should not be regarded as change of flightpath plan.

If flightpath update indication can be triggered for this reason, there will be a lot of flightpath plan updates. And the network cannot distinguish the reason if only one indicator is defined for flightpath update notification. At least for the waypoints with timestamp, it is meaningless for the network to acquire the updated flightpath since the network can deduce it based on the timestamp. So we propose the flightpath update indication is not triggered if the flightpath update is just for removing outdated waypoints..

Proposal 1a: Flightpath update indication is not triggered if the flightpath update is just for removing outdated waypoints.

In the current running CR, it is captured that UE triggers flightpath update when timestamp was not previously provided but is now available. Considering timestamp is optionally provided for a waypoint, it may be added, removed for a waypoint. From network point of view, it is meaningful to be notified if timestamp is added for a waypoint. And it is also meaningful to be notified when a timestamp is removed. So we propose the flightpath update can be triggered when timestamp is added or removed for a waypoint.

Proposal 2: Flightpath update indication can be triggered due to adding or removing the timestamp for a waypoint.

In current RRC running CR[4], there is an FFS issue on the UE behavior when neither distance nor time threshold is configured:

Editor’s Note: FFS how to capture the UE behavior when neither distance nor time threshold is configured, e.g. leave upto UE with a NOTE like below or add network restriction to always configure at least one threshold “NOTE x:	If neither flightPathUpdateDistanceThr nor flightPathUpdateTimeThr is configured, it is up to UE implementation whether to initiate transmission of the UEAssistanceInformation message when updated flight path information is available.”

We can first determine the meaning of the absence of both distance and time threshold: whether it means the flightpath update notification is allowed or not allowed. We noticed that we already have introduced a new IE uav-FlightPathAvailabilityConfig in OtherConfig. The network can turn off flight path availability indication by not configuring this IE. In other words, we don’t need another mechanism with similar function.

Furthermore, we don’t think it is reasonable to add network restriction to always configure at least one threshold. It is unreasonable to do so if network don’t want to configure any distance or time threshold. On the contrary, we prefer to have some flexibility for the network configuration to be able to not configure any threshold.

Proposal 3: It can be up to UE implementation whether to initiate the UAI message with the following note when neither distance nor time threshold is configured:

NOTE x:	If neither flightPathUpdateDistanceThr nor flightPathUpdateTimeThr is configured, it is up to UE implementation whether to initiate transmission of the UEAssistanceInformation message when updated flight path information is available.

- Conclusion and proposals

Proposal 1: Flightpath update indication can be triggered due to adding or removing waypoints.

Proposal 1a: Flightpath update indication is not triggered if the flightpath update is just for removing outdated waypoints.

Proposal 2: Flightpath update indication can be triggered due to adding or removing the timestamp for a waypoint.

Proposal 3: It can be up to UE implementation whether to initiate the UAI message with the following note when neither distance nor time threshold is configured:

NOTE x:	If neither flightPathUpdateDistanceThr nor flightPathUpdateTimeThr is configured, it is up to UE implementation whether to initiate transmission of the UEAssistanceInformation message when updated flight path information is available.

- Referenced

- RAN2#121bis-e chairman notes

- RAN2#122 chairman notes

- RAN2#123 chairman notes

- R2-2309611  RRC Running CR (Introduction of Aerial Support)  Qualcomm Incorporated