3GPP TSG-RAN WG2 Meeting #123bis										    R2-2310620

Xiamen, China, October 9th – 13th, 2023

Source: 	Xiaomi

Title:  	Discussion on subsequent CPAC

Agenda Item:	7.4.3

Document for:	Discussion and Decision

Introduction

A WID on Further NR mobility enhancements [1] has been approved. In the WID, one of the objectives is to specify mechanism and procedures of NR-DC with selective activation of the cell groups, as described below:

2. To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements: - To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4] Note 5: A harmonized RRC modelling approach for objectives 1 and 2 could be considered to minimize the workload in RAN2.

In RAN2#122 meeting [2], RAN 2 agreed terminology is “Subsequent CPAC”. In this contribution, we provide our understandings for the remaining issues of selective activation of the cell groups (i.e. subsequent CPAC).

Discussion

  - Configuration of subsequent CPAC

1) SN initiated intra-SN subsequent CPC

In RAN2 #121 meeting [3], the following scenarios have been supported for subsequent CPAC:

- Case 1: SN initiated intra-SN subsequent CPAC

- Case 2: MN initiated inter-SN subsequent CPAC

- Case 3: SN initiated inter-SN subsequent CPAC

Whether to support the coexistence of case 1, case 2 and case 3 has been discussed. Similar to legacy CPAC, the coexistence should be supported.

Proposal 1: The coexistence of SN initiated intra-SN subsequent CPAC, MN initiated inter-SN subsequent CPAC and SN initiated inter-SN subsequent CPAC is supported.

For MN/SN initiated inter-SN subsequent CPAC, it is configured by MN-format and MN involvement is always needed. The conditional reconfigurations for MN/SN initiated inter-SN subsequent CPAC are stored in MCG VarConditionalConfig.

For the SN initiated intra-SN subsequent CPAC, similar to legacy SN initiated intra-SN CPC, it can be configured by SN via SRB3 or forwarded via SRB1 without MN involvement. All configurations for intra-SN subsequent CPAC are stored within the SCG VarConditionalConfig.

Proposal 2: The SN initiated intra-SN subsequent CPAC without MN involvement can be supported and it is configured by SN-format and stored in SCG VarCondReconfiguration.

For the MN-format configuration of the MN/SN initiated inter-SN subsequent CPAC, the candidate SN should be allowed to be the source SN and provide intra-SN candidate PSCells for the preparation of subsequent CPAC. For this case, intra-SN subsequent CPC is a specific case of the inter-SN subsequent CPAC.

Proposal 3: The MN/SN initiated inter-SN subsequent CPAC can be used to configure intra-SN candidate PSCells for the preparation of subsequent CPAC.

2) FFS on how to configure the subsequent execution conditions.

In previous meetings, RAN2 has agreed for SN/MN initiated subsequent CPAC, candidate SN generates execution conditions for subsequent CPC. But how to provide the conditions generated by candidate SN to UE via RRC signalling is FFS. There are two options can be considered for the configuration of execution conditions generated by candidate SN.

- Option 1: Using nested configuration to configure the execution conditions generated by candidate SN.

For option 1, the execution conditions can be included in the candidate cell configuration (i.e. the RRCReconfiguration associated with the candidate PSCell), and the execution conditions for subsequent CPC are contains in the conditionalReconfiguration included in the RRCReconfiguration.

- Option 2: Introducing a new field in CondReconfigToAddMod IE to include a list of execution conditions generated by candidate SN. Each condition included in the list is associated with one candidate cell.

For option 2, when UE executes CPAC and apply the candidate configuration in the CondReconfigToAddMod, these conditions can be used for subsequent CPC.

Because RAN2 has agreed “Will not spend specific efforts for supporting nested configurations for candidate cell configuration.” Hence, option 1 is de-prioritized and option 2 is a more straightforward method.

Proposal 4：The following method can be considered for the configuration of execution conditions generated by candidate SN for subsequent CPC.

- A new field can be introduced in CondReconfigToAddMod IE to include a list of execution conditions generated by candidate SN. Each condition included in the list is associated with one candidate cell.

3) Reference and delta configuration

To support delta configuration, RAN2 introduced a separate reference configuration for subsequent CPAC and LTM. There are still some remaining issues for the reference configuration in LTM and subsequent CPC. And the similar issues are discussing in LTM. For subsequent CPAC, we can wait more progress on LTM CR [4] and the ASN.1 structure and RRC procedure can follow LTM principle.

Proposal 5：For the reference configuration of subsequent CPAC, the ASN.1 structure and RRC procedure can follow LTM principle and wait more progress on LTM CR.

For subsequent CPAC, some specific issues for the reference and delta configuration can be discussed:

- Issue 1: whether to use two independent reference configurations for SN initiated intra-SN subsequent CPAC and MN/SN initiated inter-SN subsequent CPAC

- Issue 2: whether the reference configuration can include MCG configuration

- Issue 3: which node initially generates the reference configuration in subsequent CPAC Candidate SN or Source SN

Because the intra-SN and inter-SN subsequent CPAC configurations provided to the UE are in SN format and MN format separately, two independent reference configurations should be configured for SN initiated intra-SN subsequent CPAC and MN/SN initiated inter-SN subsequent CPAC.

Proposal 6: Two independent reference configurations can be configured for SN initiated intra-SN subsequent CPAC and MN/SN initiated inter-SN subsequent CPAC separately.

In Rel-17, the inter-SN CPAC candidate configuration can include MCG part, the candidate configuration for inter-SN subsequent CPAC provided in MN format can also include the MCG configuration. For intra-SN subsequent CPAC without MN involvement, only SCG configuration can be included in the candidate configuration. To align with candidate configurations of subsequent CPAC, the reference configurations should include the same part of candidate configurations.

The MCG configuration as a part of candidate configurations or the reference configuration should be generated by the MN. For the SCG configuration included in the candidate or reference configuration, it can be generated by the source or candidate SN.

Given the above, for SN initiated intra-SN subsequent CPAC without MN involvement, the MCG configuration cannot be included in the reference configuration, which is generated by the serving SN. For MN/SN initiated inter-SN subsequent CPAC, the MCG configuration can be included in the reference configuration. And the MCG part in the reference configuration is generated by the MN and the SCG part in the reference configuration is generated by the source or candidate SN(s).

Proposal 7: For SN initiated intra-SN subsequent CPAC without MN involvement, the reference configuration is generated by the serving SN, and the MCG configuration cannot be included in the reference configuration.

Proposal 8: For MN/SN initiated inter-SN subsequent CPAC, the MCG configuration can be included in the reference configuration. And the MCG part in the reference configuration is generated by the MN and the SCG part in the reference configuration is generated by the source or candidate SN(s).

4) Remove the configurations of intra-SN subsequent CPAC upon SCG release

According to the above analysis, in Rel-18, both inter-SN and intra-SN subsequent CPAC are supported and can be configured by MCG conditionalReconfiguration and SCG conditionalReconfiguration separately. The UE maintains two independent VarConditionalReconfig, one associated with each conditionalReconfiguration.

For intra-SN subsequent CPC, all configurations are generated by the serving SN. Hence, UE should remove all associated conditional configurations of the intra-SN subsequent CPC upon SCG release as in legacy. The legacy SCG release procedure can be reused and UE shall remove all the entries within the SCG VarConditionalReconfig upon SCG release, and there is no specification change is needed.

Proposal 9: UE should remove all conditional configurations for SN initiated intra-SN subsequent CPC upon SCG release, i.e. UE shall remove all the entries within the SCG VarConditionalReconfig upon SCG release.

  - CPA for subsequent CPAC

In previous meetings, RAN2 has agreed to support “CPA” selective activation of cell groups and a CPA conditional configuration can be used for CPC (but with different triggering conditions).

For CPA in Rel-18 subsequent CPAC, there are some issues can be discussed:

1) Whether to support subsequent CPA.

To support subsequent CPA, UE can perform PSCell addition without reconfiguration and re-initiation of CPA which can reduce signalling overhead (i.e. maintaining candidate PSCell configurations for CPA after SCG release.). For subsequent CPA, UE can keep subsequent CPA configurations when SCG release. If it is needed, the network can initiate the removal of subsequent CPAC configurations by the explicit RRC indication (i.e. condReconfigToRemoveList).

RAN2 has agreed to use different conditions for CPA and CPC. If the CPA condition is configured for the subsequent CPAC configuration, the maintained candidate configurations after SCG release can be used for subsequent CPA. If CPA condition is not provided, the associated maintained configuration can only be used for subsequent CPC after SCG addition.

Proposal 10: Subsequent CPA can be supported and the UE can use the maintained conditional configurations for subsequent CPA after SCG release, if the CPA condition is included in the subsequent CPAC configuration.

2) How to configure the CPA conditional configuration with different triggering conditions (for CPA and CPC)

According to the scenarios of subsequent CPAC, the following cases can be considered:

- Case 1: The CPA conditional configuration can be used for MN initiated subsequent CPA and SN initiated subsequent CPC.

- Case 2: The CPA conditional configuration can be used for MN initiated subsequent CPA and MN initiated subsequent CPC.

In the current ASN.1 structure for conditional reconfiguration, each conditional configuration can include one condition from MN or one condition from SN associated with the candidate.

For case 1, we need to achieve “the condition from MN and the condition from SN can be configured simultaneously in one conditional reconfiguration associated with the same candidate PSCell.” And the current ASN.1 structure of CondReconfigToAddMod without any change can achieve case 1.

However, for case 2, current fields in CondReconfigToAddMod cannot be used to configure different conditions generated by MN for CPC and CPA separately. Hence, a new field is needed. Similar to R16 condExecutionCond, the new field can also be used to configure a condition generated by MN.

To use a unified method for case 1 and case 2, a new field can be introduced in CondReconfigToAddMod to configure the CPA condition for CPA with different triggering conditions. And legacy fields can be used to configure CPC condition generated by MN or SN.

The following ASN.1 example can be considered:

CondReconfigToAddModList information element

-- ASN1START

-- TAG-CONDRECONFIGTOADDMODLIST-START

CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16

CondReconfigToAddMod-r16 ::=     SEQUENCE {

condReconfigId-r16               CondReconfigId-r16,

condExecutionCond-r16            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

condRRCReconfig-r16              OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Cond condReconfigAdd

...,

[[

condExecutionCondSCG-r17         OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL     -- Need M

]],

[[

condExecutionCondCPA-r18         SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- FFS

]]

}

CondReconfigExecCondSCG-r17 ::=  SEQUENCE (SIZE (1..2)) OF MeasId

-- TAG-CONDRECONFIGTOADDMODLIST-STOP

-- ASN1STOP

The condExecutionCondCPA can be introduced to configure the CPA condition for the case where one conditional configuration with two different conditions. The condExecutionCondCPA is generated by MN and the associated measId(s) refer to the measConfig associated with MCG.

To support one conditional configuration used for CPA and CPC with different trigger conditions, the legacy field (i.e. condExecutionCond and condExecutionCondSCG) is used to configure the CPC condition and the new field (i.e. condExecutionCondCPA) can be used to configure CPA conditions.

Proposal 11: A new field can be introduced in CondReconfigToAddMod IE to support one conditional configuration including one CPA condition and one CPC condition. The legacy field (i.e. condExecutionCond or condExecutionCondSCG) is used to configure the CPC condition and the new field is used to configure the CPA condition.

  - Security issues for subsequent CPAC

Security issues for subsequent CPAC have been discussed in previous meetings, and RAN2 aim to support selective activation of cell groups without RRC reconfiguration with respect to security. It is necessary to avoid key reuse in subsequent CPAC without RRC Reconfiguration.

Following is the solution details from security perspective:

1. The MN provides to the UE the SN Counter values for each SN, and the UE stores these values along with the CPC.

2. The UE derives the KSN using the KgNB together with an unused SN Counter value pre-provisioned by the MN. The UE changes the SN Counter upon every SN change.

SA3 also discussed the security issue to avoid key reuse in subsequent CPAC, and provided the solution in SA3 LS [5]:

Based on the solution provided by SA3, RAN2 has reached the following agreement to avoid key reuse [6]:

- Will support the SA3 solution, i.e. update of Sk-counter at inter-SN-mobility, based on pre-configured multiple Sk-counter. UE need to know when Sk counter need to change.

According to the above solution, multiple sk-Counter value lists can be included in ConditionalReconfiguration, and each sk-Counter value list is associated with one cell set ID, which has been captured in the running CR [6], as shown below:

SK-CounterConfiguration-r18      ::= SEQUENCE {

sk-CounterConfigToReleaseList-r18      SK-CounterConfigToReleaseList-r18                 OPTIONAL,    -- Need N

sk-CounterConfigToAddModList-r18       SK-CounterConfigToAddModList-r18                  OPTIONAL     -- Need N

}

SK-CounterConfigToAddModList-r18 ::= SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF SK-CounterConfigToAddMod-r18

SK-CounterConfigToAddMod-r18 ::= SEQUENCE {

securityCellSetID-r18          SecurityCellSetID-r18

sk-CounterList-r18             SK-CounterList-r18

}

SecurityCellSetID-r18 ::= INTEGER (1.. maxSecurityCellSet-r18)

SK-CounterList-r18    ::= SEQUENCE (SIZE (1..maxSK-Counter)) OF SK-Counter

sk-CounterConfigToRemoveList-r16 ::=     SEQUENCE (SIZE (1.. maxSecurityCellSet-r18)) OF  SecurityCellSetID-r18

Some details for the above solution also needed to be discussed:

Issue 1: FFS on how to determine the inter-SN mobility for security key update for subsequent CPAC

For issue 1, a method similar to the indicator for L2 reset in LTM [4] can be considered, and cell sets can be introduced and one or more candidate configurations are inside in the same set associated with the same candidate SN (i.e. associated with the same SN Counter value list), i.e. cell set IDs introduced in the above ASN.1 structure can be used to indicate different candidate SNs.

The cell set ID can be included in each conditional reconfiguration (i.e. CondReconfigToAddMod) associated with each candidate cell to indicate whether the UE should perform SN key update upon the execution of subsequent CPAC. If the configuration set ID for the candidate cell is the same as the configuration set ID for the serving cell, it means UE performs intra-SN mobility and the UE shall not perform SN key update, otherwise, it means UE performs inter-SN mobility and the UE shall derive a new KSN.

Proposal 12: Cell set IDs can be included in each conditional reconfiguration (i.e. CondReconfigToAddMod) to determine whether to perform security key update. Upon execution of subsequent CPAC, if the set ID for the candidate cell is the same as the set ID for the serving cell, the UE shall not perform SN key update, otherwise, UE shall derive a new KSN.

Issue 2: how UE select sk-Counter for key update for subsequent CPAC and how to handle the used sk-counter.

Because it has been agreed to provide multiple sk-Counters for one candidate SN and A sk-Counter list associated with one candidate SN can be pre-configured by the network. Naturally, the simple way is that UE always selects the first unused entry in the sk-Counter list that corresponds to the candidate SN.

In the post email discussion for subsequent CPAC security [8], there are two methods have been provided for how to handle the used Sk-counter to ensure the UE can always select the first unused SK-counter in SK-counter list.

- Method 1: The SK-counter is marked as used within the list after generation of SN key.

- Method 2: The UE can remove the selected sk-Counter from the sk-Counter list after generation of SN key.

Actually, we don’t need to specify how to handle the used sk-Counter by UE. And based on UE implementation, UE can always select the first unused sk-Counter in the sk-Counter list.

Proposal 13：Upon SN change, UE always selects the first unused entry in the sk-Counter list that corresponds to the candidate SN. It is up to UE implementation to ensure the UE can select the first unused sk-Counter in the sk-Counter list.

Issue 3: FFS on whether and how to inform the selected sk-Counter to MN/SN.

Because all sk-Counter lists and the association between sk-Counter lists and SCPAC configurations are always provided by the MN and MN can always know which candidates SN is selected by the UE via the RRCReconfigurationComplete message, the MN can always know which SK-counter has been used. Therefore, there is no mis-synchronization of the selected counter if the MN and the UE handle the selected SK-counter with the same behaviour, and the selected sk-Counter or SN key can be sent from the MN to candidate SN. Hence, there is no need to inform the selected SK counter value to the NW (MN or SN).

Proposal 14：There is no need to inform the selected sk-Counter to the NW (MN or SN).

Conclusions

According to the analysis given above, we have the following observations and proposals:

Configuration of subsequent CPAC

Proposal 1: The coexistence of SN initiated intra-SN subsequent CPAC, MN initiated inter-SN subsequent CPAC and SN initiated inter-SN subsequent CPAC is supported.

Proposal 2: The SN initiated intra-SN subsequent CPAC without MN involvement can be supported and it is configured by SN-format and stored in SCG VarCondReconfiguration.

Proposal 3: The MN/SN initiated inter-SN subsequent CPAC can be used to configure intra-SN candidate PSCells for the preparation of subsequent CPAC.

Proposal 4：The following method can be considered for the configuration of execution conditions generated by candidate SN for subsequent CPC.

- A new field can be introduced in CondReconfigToAddMod IE to include a list of execution conditions generated by candidate SN. Each condition included in the list is associated with one candidate cell.

Proposal 5：For the reference configuration of subsequent CPAC, the ASN.1 structure and RRC procedure can follow LTM principle and wait more progress on LTM CR.

Proposal 6: Two independent reference configurations can be configured for SN initiated intra-SN subsequent CPAC and MN/SN initiated inter-SN subsequent CPAC separately.

Proposal 7: For SN initiated intra-SN subsequent CPAC without MN involvement, the reference configuration is generated by the serving SN, and the MCG configuration cannot be included in the reference configuration.

Proposal 8: For MN/SN initiated inter-SN subsequent CPAC, the MCG configuration can be included in the reference configuration. And the MCG part in the reference configuration is generated by the MN and the SCG part in the reference configuration is generated by the source or candidate SN(s).

Proposal 9: UE should remove all conditional configurations for SN initiated intra-SN subsequent CPC upon SCG release, i.e. UE shall remove all the entries within the SCG VarConditionalReconfig upon SCG release.

CPA for subsequent CPAC

Proposal 10: Subsequent CPA can be supported and the UE can use the maintained conditional configurations for subsequent CPA after SCG release, if the CPA condition is included in the subsequent CPAC configuration.

Proposal 11: A new field can be introduced in CondReconfigToAddMod IE to support one conditional configuration including one CPA condition and one CPC condition. The legacy field (i.e. condExecutionCond or condExecutionCondSCG) is used to configure the CPC condition and the new field is used to configure the CPA condition.

Security issues for subsequent CPAC

Proposal 12: Cell set IDs can be included in each conditional reconfiguration (i.e. CondReconfigToAddMod) to determine whether to perform security key update. Upon execution of subsequent CPAC, if the set ID for the candidate cell is the same as the set ID for the serving cell, the UE shall not perform SN key update, otherwise, UE shall derive a new KSN.

Proposal 13：Upon SN change, UE always selects the first unused entry in the sk-Counter list that corresponds to the candidate SN. It is up to UE implementation to ensure the UE can select the first unused sk-Counter in the sk-Counter list.

Proposal 14：There is no need to inform the selected sk-Counter to the NW (MN or SN).

Reference

[1] RP-231475, MediaTek Inc., “Revised WID on Further NR mobility enhancements”

[2] R2-xxxxxxx, “Report of 3GPP TSG RAN WG2 meeting #122”

[3] R2-2302401, “Report of 3GPP TSG RAN WG2 meeting #121”

[4] R2-xxxxxxx, “RRC running CR for LTM”

[5] S3-233200, “LS on Security Solution for Selective SCG”

[6] R2-xxxxxxx, “Report of 3GPP TSG RAN WG2 meeting #123”

[7] R2-xxxxxxx, “Subsequent CPAC Running CR RRC”

[8] R2-xxxxxxx, “[Post123][046][feMob] subsequent CPAC security (Nokia)”