3GPP TSG-RAN WG2 Meeting #123bis                               R2-2310671

Xiamen, China, Oct 9-13, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Consideration on RRC signalling design for CE

Agenda item:		7.21.2

Document for: 	Discussion and decision

- Introduction

For PRACH enhancement in Rel-18 further NR coverage enhancements, how to configure the preambles for Msg1 repetition numbers has been discussed in RAN2 via [Post123][802][CE_enh] CP running CR and open issue discussion, but no conclusion has been made because it relates to framework discussion. Based on the progress of [Post123][801][CE_enh] UP running CR and open issue discussion, majority companies are supportive of Alt 2.3.

In this contribution, we discuss the possible RRC signalling structure under framework Alt 2.3 and share our views.

- Discussion

The Alt 2.3 framework has following attributions:

- All repetitions are treated as a single feature, but within the feature, different repetition numbers are treated as different RACH type

- Separate RO for different number is supported by configuring different repetition numbers in different partitions (i.e. featureCombinationPreambles);

- RACH resources of RACH partitions that configured with the same “featureCombination” are considered to be within the same set of RACH resources;

- Fallback from lower number to higher number is performed within the selected set of RACH resources.

So, in case of separateRO for different repetitions, the RACH resources can be configured in separate featureCombinationPreambles IEs. However, it is still undetermined whether a single featureCombinationPreambles IE can provide RACH resources for more than one repetition numbers in case of sharedRO.

(Note: In this document, “sharedRO” means the RO is shared for different numbers, “separateRO” means the RO is separate for different numbers.)

Observation 1 For shared RO with separate preambles in framework Alt 2.3, whether a single featureCombinationPreambles IE can provide RACH resources for multiple repetition numbers is still FFS.

There are two options on the table:

Option 1: In sharedRO case, RACH resources for different repetition numbers can only be provided by separate featureCombinationPreambles IEs.

For option 1, for a given feature or feature combination, different repetition numbers must be configured by separate featureCombinationPreambles IEs. Figure 1 shows an example of the configuration.

Figure 1 Example of Option 1 configuration

The possible ASN.1 change is shown below:

FeatureCombinationPreambles information element

-- ASN1START

-- TAG-FEATURECOMBINATIONPREAMBLES-START

FeatureCombinationPreambles-r17 ::=   SEQUENCE {

featureCombination-r17                FeatureCombination-r17,

startPreambleForThisPartition-r17     INTEGER (0..63),

numberOfPreamblesPerSSB-ForThisPartition-r17 INTEGER (1..64),

ssb-SharedRO-MaskIndex-r17            INTEGER (1..15)        OPTIONAL, -- Need S

groupBconfigured-r17                  SEQUENCE {

ra-SizeGroupA-r17                     ENUMERATED {b56, b144, b208, b256, b282, b480, b640,

b800, b1000, b72, spare6, spare5,spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB-r17          ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA-r17        INTEGER (1..64)

}                                                            OPTIONAL, -- Need R

separateMsgA-PUSCH-Config-r17         MsgA-PUSCH-Config-r16  OPTIONAL, -- Cond MsgAConfigCommon

msgA-RSRP-Threshold-r17               RSRP-Range             OPTIONAL, -- Need R

rsrp-ThresholdSSB-r17                 RSRP-Range             OPTIONAL, -- Need R

deltaPreamble-r17                     INTEGER (-1..6)        OPTIONAL, -- Need R

...,

[[

msg1-RepetitionNumber-r18               ENUMERATED {2, 4, 8}    OPTIONAL -- Need R

]]

}

-- TAG-FEATURECOMBINATIONPREAMBLES-STOP

-- ASN1STOP

Because separate featureCombinationPreambles IEs are used to configure different repetition numbers.

Option 2: In sharedRO case, RACH resources for different repetition numbers are provided within a single featureCombinationPreambles IE.

For option 2, for a given feature or feature combination, different repetition numbers are configured by one featureCombinationPreambles IE if RO is shared. Figure 2 shows an example of the configuration.

Figure 2 Example of Option 2 configuration

Regarding the ASN.1 change, in our view, we already have “startPreamble” and “numberOfPreambles” in one partition, and in sharedRO scenario, it is not necessary to configure noncontinuous ranges for different numbers, so, we can simply split the range into multiple spaces. An example of the division is shown in Figure 3:

Figure 3 Example of preamble division for Msg1 repetition in sharedRO case

If network configures sharedRO for two repetition numbers (e.g. Num2+Num4, Num2+Num8, Num4+Num8), then the network just needs to configure “numberOfGroupA-Preabmles-FirstRepetition-r18” and “numberOfGroupB-Preambles-FirstRepetition-r18” parameters.

To differentiate which numbers are configured in this partition, a separate bitmap can be introduced, if the first bit is set to “1”, it means this partition provides RACH resources for Num_2; if the second bit is set to “1”, it means this partition provides RACH resources for Num_4 and so on. The possible ASN.1 change is shown below:

FeatureCombinationPreambles information element

-- ASN1START

-- TAG-FEATURECOMBINATIONPREAMBLES-START

FeatureCombinationPreambles-r17 ::=   SEQUENCE {

featureCombination-r17                FeatureCombination-r17,

startPreambleForThisPartition-r17     INTEGER (0..63),

numberOfPreamblesPerSSB-ForThisPartition-r17 INTEGER (1..64),

ssb-SharedRO-MaskIndex-r17            INTEGER (1..15)        OPTIONAL, -- Need S

groupBconfigured-r17                  SEQUENCE {

ra-SizeGroupA-r17                     ENUMERATED {b56, b144, b208, b256, b282, b480, b640,

b800, b1000, b72, spare6, spare5,spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB-r17          ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA-r17        INTEGER (1..64)

}                                                            OPTIONAL, -- Need R

separateMsgA-PUSCH-Config-r17         MsgA-PUSCH-Config-r16  OPTIONAL, -- Cond MsgAConfigCommon

msgA-RSRP-Threshold-r17               RSRP-Range             OPTIONAL, -- Need R

rsrp-ThresholdSSB-r17                 RSRP-Range             OPTIONAL, -- Need R

deltaPreamble-r17                     INTEGER (-1..6)        OPTIONAL, -- Need R

...,

[[

numberOfGroupA-Preambles-FirstRepetition-r18       INTEGER (1..64)   OPTIONAL, -- Need R

numberOfGroupA-Preambles-SecondRepetition-r18      INTEGER (1..64)   OPTIONAL, -- Need R

numberOfGroupB-Preambles-FirstRepetition-r18       INTEGER (1..64)   OPTIONAL, -- Need R

numberOfGroupB-Preambles-SecondRepetition-r18      INTEGER (1..64)   OPTIONAL, -- Need R

msg1-RepetitionNumbers-r18               BIT STRING (SIZE (3))    OPTIONAL -- Need R

]]

}

-- TAG-FEATURECOMBINATIONPREAMBLES-STOP

-- ASN1STOP

As we can see, although Option 1 requires less ASN.1 change, the drawback of Option1 is that too many RACH partitions (i.e. featureCombinationPreambles) will be configured. Assuming without Msg1 repetition, the network has already configured 10 partitions (e.g. RedCap, Slice 1, Slice 2,…,RedCap +slice 1, RedCap +slice 2,…, RedCap+Msg3 repetition,…), when the network wants to enable Msg1 repetition function, the network has to configure 10*(3+1)=40 partitions. Based on current specification, the network can only configure up to 16 featureCombinationPreambles. On the other hand, several parameters are considered as “common” for repetition number 2, 4 and 8. In Option 1, those parameters need to be carried multiple times and this increases the signalling overhead.

However, for Option 2, although the signalling change looks more than Option 1, when network wants to enable Msg1 repetition function and intends to use sharedRO with separate preambles to configure multiple repetition numbers. For the above example, the network only needs to configure 10*(1+1) = 20 partitions, this occupies less RACH partitions and the signalling overhead can be reduced.

So, we suggest to support configuring RACH resources for multiple repetition numbers in one featureCombinationPreambles IE. Only in case of separate RO, the RACH resources for different repetition numbers are configured in separate featureCombinationPreambles IEs. Please note, this is only applicable when fallback from lower number to higher number is supported. Otherwise, Option 1 will be more suitable.

Proposal 1	If fallback from lower number to higher number is supported. In sharedRO case, the RACH resources for different repetition numbers are configured within one featureCombinationPreambles IE.

- Conclusion

In this contribution, proposals and observations are:

Observation 1 For shared RO with separate preambles in framework Alt 2.3, whether a single featureCombinationPreambles IE can provide RACH resources for multiple repetition numbers is still FFS.

Proposal 1	If fallback from lower number to higher number is supported. In sharedRO case, the RACH resources for different repetition numbers are configured within one featureCombinationPreambles IE.

- Reference

[1] R2-2310196 Report of [Post123][802][CE_enh] CP running CR and open issue discussion  Huawei;

[2] R2-2310669 Report of [Post123][801][CE_enh] UP running CR and open issue discussion  ZTE;