3GPP TSG-RAN2 Meeting #123bis	R2-2310676

Xiamen, China, October 9th – 13th, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on FR2 unknown SCell activation

Agenda item:	7.25.1

Document for: 	Discussion and Decision

- Introduction

In this meeting, RAN2 has received an LS from RAN4 to share the new agreements on the unknown FR2 SCell activation enhancement with L3 reporting. The content of the LS is as follows [1]:

Enhancement for multiple FR2 unknown SCells’ activation - Define requirements for multiple SCell activation (with and without PUCCH SCell) considering at least enhancement of L3 measurement reporting triggered by SCell activation command when all to-be-activated SCells are in the same band. - Note: If requirements are not finalized in RAN4 #109 then they will be deprioritized TCI activation command transmitted is based on L3 measurement results or L1 measurement results? - If the L3 report is triggered after SCell activation, the TCI activation command is assumed based on: - the L3 report, if no L1-RSRP report is received before transmitting the TCI activation command. - the L3 report or L1-RSRP report (i.e., up to network implementation), if L3 report and L1-RSRP report is received before transmitting the TCI activation command.

Besides, RAN2 has reached a consensus that a new report Type is used. However, there are still some important issues left. In this contribution, we’d like to share our views on these issues.

- Discussion

In the last meeting, we have agreed to take CR R2-2309254 as baseline for further process. However, there are still some FFS left, as follows:

Editor Note 1: FFS on whether the reporting configuration with the reportType set to reportOnScellActivation is associated with a CG or a SCell (servingCellMO). Editor Note 2: FFS on whether and how to specify the new report is only triggered upon receiving the SCell activation indication from lower layer.

For the first bullet, if the reporting configuration with reportType set to reportOnScellActivation is associated with a CG, only one measurement identity is used as shown in Fig.1a; while if the reporting configuration with reportType set to reportOnActivation is associated with an SCell, more than one measurement identities are used as shown in Fig.1b.

Fig. 1 The reportOnActivation is associated with a cell group (a) and an SCell (b), respectively.

In the existing specification, the amount of measurement identities is limited (the maximum number is 64). If the reporting configuration with reportOnActivation links with one SCell, a lot of measurement identities are required, which seriously impacts UE measurement capability in some cases. Besides, if multiple unknown SCells are activated via SCell activation command, more than one measurement identities may be triggered simultaneously, which may cause that the UE reports the measurement results repeatedly. For example,  SCell 1 and SCell 2 are activated via one SCell activation command, measId 1 related to SCell 1 and measId 2 related to SCell 2 may be triggered at the same time. The MeasurementReport message initiated by measId 1 carries the measurement results for not only SCell 1 but also SCell 2 and SCell 3. The MeasurementReport message initiated by measId 2 also carries these measurement results. This may lead to signaling wasted.

One may argue that if the first SCell is triggered to report measurement results to the network, other SCells shall be considered as known, which can not initiate the measurement reporting procedure again. However, this requires the UE to determine whether the SCell is unknown one by one. If the UE determines in parallel, the problem above can not be avoided.

Regarding which MO is linked to the reporting configuration with reportOnActivation, we can take height-based reporting in LTE as reference. In the height-based reporting, the height information is obtained from sensors or other positioning methods, no need to perform L3 measurement on the specific frequency. In this case, the height-based reporting configuration is linked with the MO for serving cell, no need to introduce new MO. Following the same logic, we think the reporting configuration with reportOnActivation can be linked to the MO for serving Cell.

Furthermore, in order to indicate which SCell(s) the new enhancement is applicable for, we can introduce a list of SCells in the MeasConfig, as follows:

MeasConfig ::=             SEQUENCE {

measObjectToRemoveList              MeasObjectToRemoveList                 OPTIONAL,   -- Need N

measObjectToAddModList              MeasObjectToAddModList                 OPTIONAL,   -- Need N

reportConfigToRemoveList            ReportConfigToRemoveList               OPTIONAL,   -- Need N

reportConfigToAddModList            ReportConfigToAddModList               OPTIONAL,   -- Need N

measIdToRemoveList                   MeasIdToRemoveList                       OPTIONAL,   -- Need N

measIdToAddModList                   MeasIdToAddModList                       OPTIONAL,   -- Need N

s-MeasureConfig                       CHOICE {

ssb-RSRP                            RSRP-Range,

csi-RSRP                            RSRP-Range

}                                                                                   OPTIONAL,   -- Need M

quantityConfig                      QuantityConfig                             OPTIONAL,   -- Need M

measGapConfig                       MeasGapConfig                               OPTIONAL,   -- Need M

measGapSharingConfig                MeasGapSharingConfig                     OPTIONAL   -- Need M

...,

[[

interFrequencyConfig-NoGap-r16      ENUMERATED {true}                         OPTIONAL    -- Need R

]],

[[

sCellListForFastUnknownSCellAct-r18  SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex  OPTIONAL -Need R

]]

}

Proposal 1: For fast FR2 unknown SCell activation, the reporting configuration with the reportType set to reportOnActivation is associated with a CG.

Proposal 2: The reporting configuration with reportOnActivation is linked to the MO for serving cell.

Proposal 3: Introduce an SCell list in the measConfig to indicate which SCells the fast unknown SCell activation feature is applicable for.

- Conclusion and proposals

Based on the above discussions, we have the following proposals:

Proposal 1: For fast FR2 unknown SCell activation, the reporting configuration with the reportType set to reportOnActivation is associated with a CG.

Proposal 2: The reporting configuration with reportOnActivation is linked to the MO for serving cell.

Proposal 3: Introduce an SCell list in the measConfig to indicate which SCells the fast unknown SCell activation feature is applicable for.

- Reference

- R2-2309450 LS on FR2 SCell activation enhancements (R4-2314338; contact: Apple) Rel-18 NR_RRM_enh3  From RAN4 To RAN2