3GPP TSG-RAN2 Meeting #123bis	R2-2310677

Xiamen, China, October 9th – 13th, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on Cross RRH TCI state switch

Agenda item:	7.25.1

Document for: 	Discussion and Decision

- Introduction

In the RAN2 #122 meeting, an LS was received from RAN4 to ask RAN2 to introduce MAC-CE 1bit indication to inform UE on the TCI state switch across non-collocated RRHs [1]. After the discussions, we replied an LS to RAN4 to get further information, including which MAC CE(s) is affected by the RAN4 LS and the intended UE behaviors [2]. In this meeting, we receive the feedback from RAN4, and their answers are as follows [3]:

Answer 1: Yes, RAN4 LS only affects the MAC CEs intended for indicating target TCI state for PDCCH in 6.1.3.15 in TS 38.321. Answer 2: Enhanced TCI state indication in 6.1.3.44 (i.e., the MAC CE indicates two target TCI states) and the unified TCI state indication in 6.1.3.47 (i.e., the MAC CE indicating a unified states for UL and DL) are not intended to be supported by enhanced MAC-CE indication. Answer 3: In HST FR2 scenario, when highSpeedMeasFlagFR2-r17 is configured the following HST PC 6 UE behavior is expected upon the reception of enhanced MAC CE: - MAC-CE indicates “1”: - Rel-17 TCI state switching delay requirements in HST FR2 scenarios from TS 38.133, Clause 8.10.3A are followed without changes. - When highSpeedLargeOneStepUL-TimingFR2-r17 is enabled for HST PC 6 UE supporting [largeOneStepUL-timingFR2-r17] capability, UE shall apply the one shot large timing adjustment specified in Clause 7.1.2.3 in TS 38.133 on the first UL transmission after TCI state switch without checking the DL timing difference threshold. - MAC-CE indicates “0”: - UE follows the Rel-15 TCI state switching requirements as described in TS 38.133 Clause 8.10.3 - Gradual timing adjustment requirements in Clause 7.1.2.1 in TS 38.133 apply to the first UL transmission after TCI state switch, i.e., one shot large UL timing adjustment is not applied.

In this contribution, we ‘d like to share our views on MAC-CE based indication for cross-RRH TCI state switch.

- Discussion

2.1 The MAC CE design

According to the LS [1], 1 bit MAC CE indication shall be added in the TCI State Indication for UE-specific PDCCH MAC CE to indicate TCI state switch across non-collocated RRHs, without considering the Enhanced TCI States Indication for UE-specific PDCCH MAC CE and Unified TCI States Activation/Deactivation MAC CE. The format for TCI State Indication for UE-specific PDCCH MAC CE is as follows:

Fig. 1 TCI State Indication for UE-specific PDCCH MAC CE

As we can see, there is no reserved bit left, thus we can not directly add 1 bit indication in the existing MAC CE. We need to introduce a new MAC CE. In order to identity the new MAC CE, we can use MAC subheader with new LCID or new eLCID. Although the length of LCID is one byte shorter than eLCID, considering only 12 LCID values left and the using scenario for the new MAC CE is not common enough, we prefer using new eLCID.

Regarding the format for the new MAC CE, there are the following two options:

Option 1:  Reuse the existing structure of TCI State Indication for UE-specific PDCCH

Option 2:  Extend the existing structure of TCI State Indication for UE-specific PDCCH to add 1 bit indication, as follows:

The general idea of option 1 is that if receiving MAC subheader with legacy LCID, the TCI state switch is considered as intra-RRH, while if receiving MAC subheader with new eLCID, the TCI state switch is considered as inter-RRH. In option 2, if cross RRH field is set to 1, the TCI state switch is inter-RRH; if the cross RRH field is set to 0, the TCI state switch is intra-RRH.

According to the LS and existing Ts 38.133 clause 7.1.2.3, if highSpeedLargeOneStepUL-TimingFR2-r17 is set to true, the expected UE behaviors upon receiving the different MAC CE are as follows:

| New MAC CE with the indication 1 | New MAC CE with the indication 0 | Legacy MAC |
| --- | --- | --- |
| The UE performs one shot large timing adjustment without checking the DL timing difference threshold. | The UE performs gradual timing adjustment. | If DL timing difference is larger than threshold, the UE performs one shot large timing adjustment; otherwise, the UE performs gradual timing adjustment. |

It is noted the expected UE behaviors in these scenarios are different. However, with option 1, the UE can not distinguish between the scenario receiving new MAC CE with the indication 0 and the scenario receiving legacy MAC CE. Thus, option 1 is not feasible. With option 2, the UE can identity these scenarios. Thus, we prefer option 2.

Proposal 1: New eLCID and new MAC CE format are introduced to indicate TCI state switch across non-collocated RRH, and new cross RRH field is used to indicate whether the TCI state switch is across RRH or not.

2.2 UE capability

UE capability is required to inform the network whether the UE supports cross-RRH TCI state switch indication or not. Generally there are the following two options:

Option 1: Reuse existing UE capability ue-OneShotUL-TimingAdj-r17；

Option 2: Introduce new UE capability.

In the Rel-17, ue-OneShotUL-TimingAdj-r17 is introduced to indicate the support of one shot large timing adjustment, as follows:

| ue-OneShotUL-TimingAdj-r17 Indicates whether the UE supports one shot large UL timing adjustment. UE indicating support of this feature shall indicate support of ue-PowerClass-v1700 set to 'pc6'. | Band | No | N/A | FR2 only |
| --- | --- | --- | --- | --- |

However, UE supporting one shot large timing adjustment doesn’t always support cross-RRH TCI state switch indication. For example, a Rel-17 UE can support one shot large timing adjustment, but can not understand the new MAC CE. Thus, the existing UE capability ue-OneShotUL-TimingAdj-r17 can not be reused, and we need to introduce a new UE capability for cross-RRH TCI state switch indication. Taking ue-OneShotUL-TimingAdj-r17 for reference, the new UE capability is also per band level. And the UE indicating support of this feature shall indicate the support of ue-OneShotUL-TimingAdj-r17 and ue-PowerClass-v1700 set to 'pc6'.

Proposal 2: Introduce a new per band UE capability for cross-RRH TCI state switch indication, and the UE supporting this feature shall also support ue-OneShotUL-TimingAdj-r17 and ue-PowerClass-v1700 set to 'pc6'.

- Conclusion and proposals

Based on the above discussions, we have the following proposals:

Proposal 1: New eLCID and new MAC CE format are introduced to indicate TCI state switch across non-collocated RRH, and new cross RRH field is used to indicate whether the TCI state switch is across RRH or not.

Proposal 2: Introduce a new per band UE capability for cross-RRH TCI state switch indication, and the UE supporting this feature shall also support ue-OneShotUL-TimingAdj-r17 and ue-PowerClass-v1700 set to 'pc6'.

- Reference

- R2-2304641	LS on MAC-CE Based Indication for Cross-RRH TCI State Switch (R4-2306399; contact: Nokia)	LS in	Rel-18	NR_HST_FR2_enh	From RAN4 To RAN2

- R2-2306865   Reply LS on MAC-CE Based Indication for Cross-RRH TCI State Switch  LS out Rel-18	NR_HST_FR2_enh	   From RAN2 To RAN4

- R2-2309449   Reply LS on MAC-CE Based Indication for Cross-RRH TCI State Switch (R4-2314299; contact: Nokia)	LS in	Rel-18	NR_HST_FR2_enh	From RAN4 To RAN2