3GPP TSG-RAN WG2 Meeting #123bis	R2-2310688

Xiamen, China, 09 – 13 October 2023

Agenda item:	7.5.4.3

Source:	Nokia, Nokia Shanghai Bell

Title:	CG enhancements for XR

WID/SID:	NR_XR_enh-core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

RAN1 has discussed and made some progress on the two objectives for CG enhancements:

Specify the enhancements related to capacity: - Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2);

During RAN1#114 meeting, it was agreed to adopt OptionB-b2 as the option for UTO-UCI indication:

Agreement​ - Configure the RRC parameter Nu (Nu is the size of bit-map)​ - FFS range value of Nu​ - UTO_offset is the offset value. ​ - Alt-1: UTO_Offset is provided by configuration.​ - FFS range value of UTO_offset ​ - Alt-2: UTO_Offset = 0​ - A transmitted CG PUSCH, carries UTO-UCI that is applicable to the Nu consecutive and valid CG PUSCH TOs, starting with UTO_offset from the end of the transmitted CG PUSCH.​ - FFS on whether/how to extend to multiple CG configurations​. Conclusion (RAN1#114)​ Extending the UTO_UCI indication by CG PUSCH(s) of a CG configuration to CG PUSCH(s) of other CG configuration(s) is not supported in Rel-18.​ ​ Agreement (RAN1#114)​ For a CG configuration with UTO-UCI indication enabled:​ - For the range value for the RRC parameter Nu (Nu is the size of bit-map): (3, …, 8)​ Conclusion (RAN1#114) There is no consensus to introduce RRC parameter UTO_offset. This over-rides earlier RAN1 agreements.

This contribution discusses RAN2 impact on determination with multiple CG occasions, UTO-UCI determination and potential impact of UTO-UCI on UL skipping.

# 2	Discussion

## 2.1	UTO-UCI determination

The RAN1/RAN2 XR WI has been focused on defining the necessary enhancements to provide capacity optimization of time-frequency resources. The main reason to use the UTO-UCI is the possibility to reuse resources and optimize the network’s capacity (e.g., increasing the spectral efficiency). Option B-b2 with sliding window for indicating the CG occasions was agreed in RAN1. It has several benefits, such as a fixed number of bits, no issues with potential mismatch with XR traffic arrival, and fewer parameters to configure. To realize the capacity benefits of using UTO-UCI to indicate the usage of the CG PUSCH occasions, there must be a common set of rules for the UE, to ensure consistent behaviour from different UEs for the UTO-UCI indication of ‘not unused’/’unused’ resources. These rules should result in that UEs shall indicate CG-PUSCH TOs ‘unused’ as soon as possible to enable network to (re-)schedule the resources to other UEs. Thus, in our understanding, it is required to specify behaviour that the UE should follow to change a CG PUSCH occasion, from ‘not unused’ to ‘unused’ in the UTO-UCI based on e.g., the UL buffer status, CG-PUSCH allocation size and timing and estimated/known traffic periodicity, end of burst etc. Otherwise, skipping transmissions will be the only option the UEs use, and no CG PUSCH occasion cancellation will occur since RAN1 agreements discarded the incorporation of a UTO_offset.

Proposal 1: RAN2 should specify rules the UE should follow to determine the bitmap content in the UTO-UCI to ensure that capacity gain can be realized.

Wrt. the detailed specified rules, we think at least the buffer status at the time of the CG occasion, whether the CG occasion would be available due to the CG timer running, as well as NW processing time should be taken into account. Since the UE is not aware of the NW process time, it could be NW configured parameter for which the NW could also consider the aspects such as the mix of UE PUSCH processing times, USS configurations, and UL-DL slot patterns, etc. which is up to NW implementation.

Proposal 2: The following aspects the UE shall consider when setting the UCI bits:

- The buffer status

  - Buffer status at the CG occasions to be indicated in the UTO-UCI, taking the data to be transmitted in the previous CG occasions, size of following CG occasions, as well as potential discard into account, since the CG occasions to be indicated could be some time away.

  - Potential new data arrival taking traffic periodicity into account could also be considered to avoid too early indication of the CG occasions for the next unarrived frame/burst.

- HARQ process availability of the CG occasion as the CG occasion would be unavailable for new transmission when the CG timer for the HARQ process is running.

- The NW processing time tolerance (NW configured parameter), which indicates the latest time UTO-UCI should be used to indicate CG PUSCH occasion as unused to be reusable.

## 2.2	Impact of UL skipping on UCI indication

Another aspect worth to clarify is the UE behavior when UL skipping is configured. RAN1 has made agreements on UCI transmission and PUSCH transmission:

Agreement (RAN1#112b-e) - Option 1: For a CG PUSCH configuration, the UTO-UCI is included in every CG PUSCH that is transmitted (that is Option 1 in corresponding agreement in RAN1#112) - FFS details. Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience. Agreement (RAN1#113) - When a CG PUSCH occasion is indicated as “unused”, the UE is not allowed to transmit CG PUSCH on that CG PUSCH occasion. - For any other CG PUSCH occasion that is NOT indicated as “unused”, the UE is allowed to transmit or not to transmit CG PUSCH on that CG PUSCH occasion as per legacy specification. - No RAN1 specification impact.

According to the current specification, in case there is UCI to be multiplexed on the CG PUSCH resource, MAC should generate a MAC PDU even when there is no data available for any LCG. If UTO-UCI is configured to be multiplexed onto CG PUSCH, MAC PDU should be generated even when there is no UL data which obviously does not make sense as it would be impossible to apply UL skipping.

For example, if multiple CG PUSCH occasions per period are configured to one UE, due to UL traffic jitter, it is possible that at the beginning of the first CG PUSCH occasion, no application data has arrived at the UE’s buffer yet, as shown in Figure 1 below. The open issue is that whether UL skipping should be applied or not if the UTO-UCI is configured to be multiplexed onto the CG PUSCH occasion.

Figure 1 Example of late arrival of XR PDU set

In our understanding, according to current RAN2 specification, as the UTO-UCI needs to be sent, MAC has to generate one empty MAC TB and deliver to PHY which can be used for multiplexing UTO-UCI. Clearly this is not necessary since the UE will assume that all the CG PUSCH occasions will be used (since otherwise if it is indicted as unused it would cause delay for the late arrived data) and therefore, the reported UTO-UCI does not contain any new information comparing to the default assumption at gNB side that the configured CG PUSCH resources will be occupied unless there is “unused” indication from UE side. At least for this scenario, it makes sense that UE skips UL transmission.

Proposal 3: When other criteria for UL skipping is fulfilled, UE skips UL CG PUSCH transmission in case the UTO-UCI, which would be multiplexed on the CG PUSCH, does not contain any new information, i.e., UL skipping still applies for this case and the UE does not generate empty TB.

Proposal 4: confirm the agreement “For any other CG PUSCH occasion that is NOT indicated as “unused”, the UE is allowed to transmit or not to transmit CG PUSCH on that CG PUSCH occasion as per legacy specification” is applicable to the CG occasions that have been indicated as not unused as well as the occasions that have not been indicated with any state yet.

# 3	Conclusion

This contribution discussed potential RAN2 impact on CG enhancements with the following proposals proposed:

Proposal 1: RAN2 should specify rules the UE should follow to determine the bitmap content in the UTO-UCI to ensure that capacity gain can be realized.

Proposal 2: The following aspects the UE shall consider when setting the UCI bits:

- The buffer status

  - Buffer status at the CG occasions to be indicated in the UTO-UCI, taking the data to be transmitted in the previous CG occasions, size of following CG occasions, as well as potential discard into account, since the CG occasions to be indicated could be some time away.

  - Potential new data arrival taking traffic periodicity into account could also be considered to avoid too early indication of the CG occasions for the next unarrived frame/burst.

- HARQ process availability of the CG occasion as the CG occasion would be unavailable for new transmission when the CG timer for the HARQ process is running.

- The NW processing time tolerance (NW configured parameter), which indicates the latest time UTO-UCI should be used to indicate CG PUSCH occasion as unused to be reusable.

Proposal 3: When other criteria for UL skipping is fulfilled, UE skips UL CG PUSCH transmission in case the UTO-UCI, which would be multiplexed on the CG PUSCH, does not contain any new information, i.e., UL skipping still applies for this case and the UE does not generate empty TB.

Proposal 4: confirm the agreement “For any other CG PUSCH occasion that is NOT indicated as “unused”, the UE is allowed to transmit or not to transmit CG PUSCH on that CG PUSCH occasion as per legacy specification” is applicable to the CG occasions that have been indicated as not unused as well as the occasions that have not been indicated with any state yet.