3GPP TSG-RAN WG2 Meeting #123bis	R2-2310703

Xiamen, China, 09 – 13 October 2023

Agenda item:	7.13.4

Source:	Nokia, Nokia Shanghai Bell

Title:	SPR configuration and reporting related issues

WID/SID:	NR_ENDC_SON_MDT_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

The following agreements have been made so far in RAN2 and RAN3 about SPR configuration and retrieval:

RAN2#119bis

B.SPR is fetched via UE Information Request/Response procedure

RAN2#120

For the MN-initiated PSCell Change/Addition, MN sends the SPR config to the UE.

For the SN-initiated PSCell Change, the source-SN sends the Successful PSCell Change Configuration within the container through MN.

T304 trigger needs to be configured by the target SN node

Only MN can retrieve the SPR from the UE.

RAN2#123

1 UE clears SPR configurations if one of the following conditions is met:

-	Initiate RRC connection re-establishment

-	Initiate RRC connection resume

-	Reception of SCG Release

2	Clearing of the SPR configurations for the following scenarios. FFS which configuration (e.g., MCG or SCG based on configuration) will be cleared.

-	Successful PSCellAddition or PSCellChange

-	SCG failure

-	Reconfiguration with synch on PCell

RAN3#117bis

T310 of SCG and T312 of SCG are not considered as SPCR triggers for classic PSCell addition or CPA (since there is no source SN undergoing RLF).

RAN3#118

For SN-initiated classic PScell change the source SN node decides the T310/T312 triggers (e.g timer threshold) and the target SN node decides the T304 triggers (e.g timer threshold).

For classic addition/CPA, FFS on which node decides the T304 triggers(e.g timer threshold) and performs root cause analysis.

RAN3#119

If the trigger is T312/310, the objective of SPR is to optimize lower layer issues of source PSCell and to optimize PSCell change configuration during mobility.

If the trigger is T312/310, for MN-initiated classic PSCell change/CPC, at least the MN who sent the SPR configuration to the UE is involved in SPR related optimizations.

RAN3#120

If SPR available indication via SN RRCReconfigComplete is received by SN, SN should inform MN that an SPR is available at the UE e.g.., a new IE can be added in S-NODE MODIFICATION REQUIRED message.

RAN3#121

In case of MN initiated PSCell change, MN will have the final say on the T310/T312 SPR thresholds.

Regarding to SPR configuration and reporting, we discuss two major remaining issues in this paper.

# 2	Discussion

## 2.1		SPR configuration

Based on the scenarios agreed by RAN3, SPR should cover both MN- and SN-initiated PSCell changes (classic or conditional). It has also been agreed that SPR will use the same triggers as SHR in Rel-17, e.g. T310/T312/T304 threshold values. Obviously, in dual connectivity, these values refer to the Source and Target SN.

Observation 1.1: If the same logic is to be applied for SPR as it was for SHR, SPR should be triggered based on T310/T312 thresholds corresponding to the source SN timer values.

RAN2 decided that in case of the SN-initiated SN change, the source SN provides the SPR configuration in the SCG configuration sent to the UE via MN. Therefore, involvement of MN is not needed in this case.

For the for MN-initiated SN change, on the other hand, the decisions taken so far is that the MN sends the SPR configuration to the UE and that MN has the final say in the threshold configuration. These agreements still do notc clarify how the MN can determine a meaningful SPR configuration including trigger threshold values. Current Xn information exchange between MN and SN, is not enough for proper configuration of SPR by MN. There are no means for MN to know about the absolute T310 timer configured for the secondary link, as T310 for SN is configured by SN itself. Thus, it is infeasible for MN to configure a proper T310 threshold – in percentage of the T310 value - for SPR. Even if the MN configures a T310 threshold, e.g., at 20%, this would not be a proper input for MRO algorithm as the MN would not understand SPR reported by the UE without knowing the absolute T310 value configured by the SN – MN would not know how much interruption occurred.

Observation 1.2: MN cannot configure a meaningful T310 threshold in SPR configuration, as MN is unaware of the T310 timer value configured by the SN during connection establishment.

Similarly, a T312 timer value can be configured by the source SN (theoretically differently for each measurement object configured by the source SN – to optimize per cell border). In this case, as well, the MN would not know the T312 value configured at the UE by SN and cannot configure an adequate threshold value for T312.

Observation 1.3: MN cannot configure a meaningful T312 threshold in SPR configuration, as MN is unaware of T312 timer value configured by SN along with different measurement objects.

Thus, in order for the MN to perform a proper and meaningful SPR configuration in case of MN initiated PSCell change, coordination with the source SN is needed. For example, the MN could fetch the currently used T310/T312 values from the MN via Xn procedures.

Proposal 1: Coordination between MN and source SN is needed for the MN-initiated SN change to allow the MN to properly configure SPR.

## 2.2	SPR reporting mechanism

According to RAN2#119bis-e agreement the UEInformationRequest/UEInformationResponse mechanism will be used to fetch the SPR from the UE. The UE shall send the SPR report if available in the UEInformationResponse only when triggered by the UEInformationRequest. The UEInformationRequest with respect to SPR is sent to the UE only if UE has previously indicated the availability of SPR to the network.

SPR availability could be indicated using legacy mechanism such in the case of SHR, where the report availability is sent to the network via the RRCReconfigurationComplete message. However, in the dual connectivity scenario (as depicted in Fig. 3) during a PSCell change the RRCReconfigurationComplete message is sent to MN right after receiving the RRCConnectionReconfiguration and before the PSCell is accessed (successfully or not). At that stage the UE cannot know in advance if any of the configured thresholds for SPR generation will be exceeded and thus cannot generate SPR.

Figure 1-1: SN change procedure – MN initiated (TS 37.340 Fig. 10.5.2-1)

Observation 2: SPR availability cannot be indicated to the network by using RRCReconfigurationComplete message as the latter is sent to MN too early.

Proposal 2.1: RAN2 to agree that a different mechanism (other than indicating it in RRCReconfigurationComplete message) to indicate SPR availability to the network is needed.

Proposal 2.2: RAN2 discusses a different mechanism (other than indicating it in RRCReconfigurationComplete message) to indicate SPR availability to the network.

# 3	Conclusion

This document has made the following observations and proposals for SPR configuration and reporting:

Observation 1.1: If the same logic is to be applied for SPR as it was for SHR, SPR should be triggered based on T310/T312 thresholds corresponding to the source SN timer values.

Observation 1.2: MN cannot configure a meaningful T310 threshold in SPR configuration, as MN is unaware of the T310 timer value configured by the SN during connection establishment.

Observation 1.3: MN cannot configure a meaningful T312 threshold in SPR configuration, as MN is unaware of T312 timer value configured by SN along with different measurement objects.

Proposal 1: Coordination between MN and source SN is needed for the MN-initiated SN change to allow the MN to properly configure SPR.

Observation 2: SPR availability cannot be indicated to the network by using RRCReconfigurationComplete message as the latter is sent to MN too early.

Proposal 2.1: RAN2 to agree that a different mechanism (other than indicating it in RRCReconfigurationComplete message) to indicate SPR availability to the network is needed.

Proposal 2.2: RAN2 discusses a different mechanism (other than indicating it in RRCReconfigurationComplete message) to indicate SPR availability to the network.