3GPP TSG-RAN WG2 Meeting #123-bis	R2-2310712

Xiamen, China, October 9th-13th, 2023

Agenda item:	7.11.2.1

Source:	Huawei, Hisilicon

Title:	CP issues for multicast reception for RRC_INACTIVE UE

Document for:	Discussion

- Introduction

The following agreements were made during the discussion in RAN2#123 meeting for CP issues for multicast reception in RRC_INACTIVE.

- For a UE receiving multicast in RRC_INACTIVE, the UE resumes the RRC connection when the measured RSRP or RSRQ based on the existing measurement requirements (whichever is configured by the NW) of the serving cell becomes lower than the threshold configured by network. FFS whether/how we need to address ping-pong issue - The threshold can be configured in PTM configuration per MBS session via RRCRelease or multicast MCCH message. - Unless issues are identified with using one of existing resume causes, no new resume causes are introduced for UEs receiving MC in INACTIVE when they resume due to bad quality or lack of SIBx/PTM configuration

In this contribution, we will further analyze the remaining issues based on the progress on multicast reception in RRC_INACTIVE and also some issues related to the Stage-3 RRC.

- Discussion

2.1 PTM configuration and mobility aspects

UE resume due to bad reception quality

It has been agreed that UE triggers RRC connection resumption if the signal quality is below a configured threshold. Regarding the ping-pong issue, we agree this is something that is likely to happen without some mechanism being used to prevent it. This will then lead to unnecessary signalling overhead and service interruptions which should be avoided in the already congested cell.

Observation 1: Ping-pong issue for the UE resume due to bad quality should be avoided.

To address this issue, instead of re-discussing the signaling design and how to use a new threshold and a new timer for this case, a simpler way is to reuse the similar signaling design and logic as the existing A2 event, for the MBS case here. Then much time for discussion and spec effort for capturing can be saved. For example, the following configuration (with reportOnLeave removed) can be used for this purpose and the UE initiates resume when this even is triggered.

eventA2                                     SEQUENCE {

a2-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger

},

Proposal 1a: Reuse the signaling structure and logic of eventA2 to configure the UE to resume due to bad signal quality.

If the UE triggers resume for a deactivated MBS multicast session due to bad signal quality, the gNB has to release the UE back to RRC_INACTIVE due to no data transmission, and the UE will trigger resume again due to bad quality. To avoid such deadlock, the UE shall not trigger resume for a deactivated MBS multicast session even though the reception quality of the signal is below a configured threshold.

Proposal 1b: The UE only triggers resume due to bad quality in case the corresponding multicast session is activated.

Congestion mitigation

If the congestion situation for a cell providing multicast to RRC_INACTIVE UEs has been mitigated, the gNB might need to invoke the UE back to RRC_CONNECTED mode for better service experience. Group paging without “inactiveReceptionAllowed” indication should be allowed to be used in such situation for an active MBS session.

UE n+1：RRC Resume Req

UE

gNB

UE

UE

UE 1：State transition to RRC_CONNECTED

UE n：State transition to RRC_CONNECTED

Decide not to admit more UEs

RRC Release (inactive reception)

Group paging

Once the congestion is mitigated, RAN may decide to move the UEs back to RRC CONNECTED state for further MBS multicast reception. The gNBs will trigger group paging, but if the number of accessing UEs will be higher than expected by the network, the congestion may occur once again and the network should be able to prevent some UEs from entering RRC_CONNECTED mode. A simple way to achieve this, is to use two steps release procedure to directly redirect UE to RRC_INACTIVE without state transition. However, if the anchor gNB for the UE is not the serving gNB, some coordination between gNBs might be needed and a LS to RAN3 is needed to complete this procedure.

Proposal 2a: After group paging, the gNB should be able to use two step release procedure to redirect UE to RRC_INACTIVE without state transition for continuing MBS multicast reception.

Proposal 2b: Send an LS to RAN3 to discuss RAN3 impact for this procedure.

Resume cause and access category

The following agreement was made in the last meeting:

- Unless issues are identified with using one of existing resume causes, no new resume causes are introduced for UEs receiving MC in INACTIVE when they resume due to bad quality or lack of SIBx/PTM configuration

The baseline is to reuse the current resume cause for the resume due to bad quality or lack of SIBx/PTM configuration. The existing resume causes are listed below as extracted from [38.331]:

ResumeCause ::=             ENUMERATED {emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, rna-Update, mps-PriorityAccess,

mcs-PriorityAccess, spare1, spare2, spare3, spare4, spare5 }

It seems the MO-data is the closest resume cause for MBS data reception due to bad quality or lack of SIBx/PTM configuration. However, the legacy MO-data is triggered by NAS while MBS resume is triggered by AS. The UE behavior after having been barred are different for AS triggered resume and NAS triggered resume, and thus MO-data triggered by NAS and MO-data triggered for MBS need to be separately handled for UAC procedure. For example, it is NAS layer to re-initiate the procedure when barring is alleviated for legacy MO-data case. However, for MO-data triggered for MBS, similar to the existing resume procedure, the UE’s AS layer should re-initiate the procedure when barring is alleviated, similar as RNA update procedure.

Proposal 3a: MO-data is used for MBS resume due to bad quality or lack of SIBx/PTM configuration.

Proposal 3b: MO-data triggered by NAS and MO-data triggered for MBS need to be separately handled for UAC procedure, e.g., for MO-data triggered for MBS, the UE’s AS layer should re-initiate the procedure when barring is alleviated.

NCL for multicast

It has been agreed that NCL is supported for multicast reception in RRC_INACTIVE similar as MBS broadcast NCL. However, the NCL can only indicate whether there is ongoing RRC_INACTIVE multicast service in neighbour cells. The UE still has to read SIB and multicast MCCH after mobility, as long as the service is ongoing in the target cell for RRC_INACTIVE. However, at least for intra-gNB case, it is very likely that the gNB will use exactly the same PTM configuration to provide the same multicast service in different cells. If this information is provided to the UE beforehand and the target cell is using the same PTM configuration as the source cell, the UE can save the interruption time to acquire SIB and multicast MCCH in the target cell during mobility and directly begin to receive service. Note that this comes with little cost as it does not mean to provide full PTM configuration of neighbor cells but just to indicate whether the configuration is the same or not, i.e., adding one bit per neighbor cell providing the same service in mtch-NeighbourCell

Proposal 4: NW should be able to provide the UE with the information on whether the same PTM configuration is being used in neighbour cells for multicast in RRC_INACTIVE by adding one bit per neighbor cell in the mtch-NeighbourCell.

2.2 Other Stage 3 RRC issues

SDT and MBS multicast reception in RRC_INACTIVE

In the current RRC running CR, there is an Editor’s note about simultaneous configuration of SDT and multicast reception in RRC_INACTIVE, as follows:

Editor’s Note: FFS whether SDT and MBS multicast reception in RRC_INACTIVE can be configured together. And if yes, whether UE configured for MBS multicast reception in RRC_INACTIVE monitors group paging during SDT.

In our view, it is beneficial to allow to configure SDT and MBS multicast reception in RRC_INACTIVE together. Considering the scenario for multicast reception in RRC_INACTIVE is mainly due to NW congestion, the UE is possibly not expected to enter RRC_CONNECTED for small data transmission, and is better to stay in RRC_INACTIVE and use SDT.

Observation 2: It is beneficial to allow SDT and MBS multicast reception in RRC_INACTIVE to be configured together.

In Rel-17, the UE does not need to monitor group paging for MBS during SDT procedure, because the UE can be resumed to RRC_CONNECTED directly by the gNB when there is MBS service requirement. However, in Rel-18, if the UE is configured with multicast reception in RRC_INACTIVE, the UE should be informed of session activation via group paging in RRC_INACTIVE, even during SDT procedure. Therefore, the UE should monitor group paging during SDT when multicast reception in RRC_INACTIVE is configured.

Proposal 5a: The MBS multicast reception in RRC_INACTIVE and SDT can be configured together.

Proposal 5b: If configured with MBS multicast reception in RRC_INACTIVE, the UE monitors group paging regardless of whether SDT procedure is ongoing.

Whether MRB ID needs to be configured

In the current RRC running CR, there is an Editor’s note about MRB ID, as following:

MBS-SessionInfoMulticast-r18 ::=   SEQUENCE {

mbs-SessionId-r18                  TMGI-r17,

g-RNTI-r18                         RNTI-Value,

mrb-ListMulticast-r18              MRB-ListBroadcast-r17,

mtch-SchedulingInfo-r18            DRX-ConfigPTM-Index-r17                      OPTIONAL, -- Need S

mtch-NeighbourCell-r18             BIT STRING (SIZE(maxNeighCellMBS-r17))       OPTIONAL, -- Need S

pdsch-ConfigIndex-r18              PDSCH-ConfigIndex-r17                        OPTIONAL, -- Need S

mtch-SSB-MappingWindowIndex-r18    MTCH-SSB-MappingWindowIndex-r17              OPTIONAL, -- Cond MTCH-Mapping

thresholdIndex-r18                 INTEGER (0..maxNrofThresholdMBS-r18)         OPTIONAL  -- Need R

}

Editor’s note: FFS whether MRB ID needs to be configured.

While the current structure for MRB configuration in the running CR is reused from broadcast, it can be enhanced if needed. We think it is useful to configure the MRB ID to UE in RRC_INACTIVE, which will help the UE to associate the MRB used in RRC_CONNECTED and MRB used in RRC_INACTIVE. If RAN2 agrees to configure the MRB ID to the UE in RRC_INACTIVE, we can introduce a list of MRB ID in MBS-SessionInfoMulticast-r18 IE, e.g., mrb-ID-List-r18, to be used together with mrb-ListMulticast-r18 by UE.

Proposal 6a: MRB ID is configured in the MBS-SessionInfoMulticast-r18 IE.

If the proposal 6a is agreed, there may be another issue for service continuity during cell reselection. In Rel-17, it was agreed to “Extend MRB ID space beyond current 32 limit and up to 512.”, which aims to support MRB ID being unified across different cells within the same gNB-DU. And then, if the MRB ID is configured to the UE in RRC_INACTIVE, it's possible that different cell allocates the different MRB IDs for the same QoS flow(s). So, the UE cannot determine which MRBs are transmitting the same traffic in the source cell and the target cell. To address this issue, one possible solution is to configure the QoS Flow ID(s) mapped to each MRB in the MBS-SessionInfoMulticast-r18 IE.

Proposal 6b: QoS Flow ID(s) mapped to each MRB is configured in the MBS-SessionInfoMulticast-r18 IE.

- Conclusion

Based on the above discussion we have the following observations and proposals:

UE resume due to bad reception quality

Observation 1: Ping-pong issue for the UE resume due to bad quality should be avoided.

Proposal 1a: Reuse the signaling structure and logic of eventA2 to configure the UE to resume due to bad signal quality.

Proposal 1b: The UE only triggers resume due to bad quality in case the corresponding multicast session is activated.

Congestion mitigation

Proposal 2a: After group paging, the gNB should be able to use two step release procedure to redirect UE to RRC_INACTIVE without state transition for continuing MBS multicast reception.

Proposal 2b: Send an LS to RAN3 to discuss RAN3 impact for this procedure.

Resume cause and access category

Proposal 3a: MO-data is used for MBS resume due to bad quality or lack of SIBx/PTM configuration.

Proposal 3b: MO-data triggered by NAS and MO-data triggered for MBS need to be separately handled for UAC procedure, e.g., for MO-data triggered for MBS, the UE’s AS layer should re-initiate the procedure when barring is alleviated.

NCL for multicast

Proposal 4: NW should be able to provide the UE with the information on whether the same PTM configuration is being used in neighbour cells for multicast in RRC_INACTIVE by adding one bit per neighbor cell in the mtch-NeighbourCell.

SDT and MBS multicast reception in RRC_INACTIVE

Observation 2: It is beneficial to allow SDT and MBS multicast reception in RRC_INACTIVE to be configured together.

Proposal 5a: The MBS multicast reception in RRC_INACTIVE and SDT can be configured together.

Proposal 5b: If configured with MBS multicast reception in RRC_INACTIVE, the UE monitors group paging regardless of whether SDT procedure is ongoing.

Whether MRB ID needs to be configured

Proposal 6a: MRB ID is configured in the MBS-SessionInfoMulticast-r18 IE.

Proposal 6b: QoS Flow ID(s) mapped to each MRB is configured in the MBS-SessionInfoMulticast-r18 IE.