3GPP TSG-RAN WG2 Meeting #123-bis 	R2-2310717

Xiamen, China, 9th – 13th October, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0716 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 2 corrections on NCR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NetConRepeater |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The latest endorsed Stage 2 running CR states that “When the NCR-MT transitions from RRC_CONNECTED state to RRC_INACTIVE state, the NCR-Fwd may continue to amplify-and-forward RF signals in accordance with the last side control information received from the gNB.” However, the description is not accurate based on the following agreement made in RAN#121bis: - When NCR-MT is released to RRC_INACTIVE state (NCR-Fwd is forwarding), the periodic beam indication configuration (if configured and not removed) shall be applied. - Not to use aperiodic and semi-persistent beam indication configuration in RRC_INACTIVE. The current wording "last side control information" may cause misunderstanding as it includes all kinds of side control information. - The latest endorsed Stage 2 running CR states that “An NCR-MT in RRC_INACTIVE state that determines degradation of the NCR-Fwd backhaul link beam should cease amplifying-and-forwarding RF signals, and attempt to resume its RRC connection (with cause value ‘mo-Signalling’). The criteria to evaluate backhaul beam degradation are left to the NCR- node implementation.” The description is somehow not so clear. From the first sentence, it is not clear what it means by “NCR-MT…that determines degradation…should cease amplifying-and-forwarding… and attempt to resume… ”. One may think once, the beam degrades, UE cease amplifying-and-forwarding… and attempt to resume. It is understandable that the second sentence is intended as the supplementary description to the first sentence. But it uses “evaluate… degradation” rather than “determine… degradation”. It is not clear NCR-MT ceases amplifying-and-forwarding and attempts to resume according to the same criteria. In fact, the agreement made in RAN#122 was as follows, which is clearer: - Add a note in stage-2: when backhaul beam degrades in RRC_INACTIVE, based on implementation criteria, NCR should stop forwarding and should attempt to resume. So it is better to follow the agreement on this. - Some editoria corrections |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the “last side control information received from the gNB” to “last periodic beam indication received from the gNB”. - Change the description related to beam degredation into a NOTE according to the agreement. - Some editoria corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some descriptions in the spec will cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.X.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is based on the endorsed running CR in R2-2309053. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

4.X.3	Signalling procedures

NR RRC signalling is utilized to configure the NCR-MT to receive side control information from a gNB, which is used by the NCR-Fwd to determine whether and how to amplify-and-forward RF signals. If the side control configuration is removed, the NCR-Fwd ceases its amplifying-and-forwarding function.

When the NCR-MT is in RRC_CONNECTED state, the NCR-Fwd may amplify-and-forward RF signals based on the side control information received from the gNB. The NCR-MT does not support RRM measurements in RRC_CONNECTED.

When the NCR-MT transitions from RRC_CONNECTED state to RRC_INACTIVE state, the NCR-Fwd may continue to amplify-and-forward RF signals in accordance with the last periodic beam indication received from the gNB. When the NCR-MT is in RRC_INACTIVE state, the NCR-Fwd ceases amplifying-and-forwarding RF signals if no suitable cell is detected, or if the NCR-MT selects a different cell than the last serving cell on which side control configuration was received.

When the NCR-MT transitions from RRC_CONNECTED state to RRC_IDLE, the NCR-Fwd ceases any amplifying-and-forwarding of RF signals. How an NCR-MT transitions back from RRC_IDLE state to RRC_CONNECTED state is left to NCR-node or network implementation.

An NCR-MT can detect Radio Link Failure (RLF) on the control link as specified in TS 38.331 clause 5.3.10 [12]. When RLF is detected, the NCR-MT performs the RRC re-establishment procedure as specified in TS 38.331 [12]. During the RRC re-establishment procedure, the NCR-Fwd ceases amplifying-and-forwarding RF signals.

After successfully performing the RRC re-establishment procedure, the NCR-MT waits for the new side control configuration for the NCR-Fwd to resume the amplifying-and-forwarding of RF signals.

An NCR-MT can also perform Beam Failure Detection (BFD) and Beam Failure Recovery (BFR) as described in clause 9.2.8. Once the NCR-MT detects beam failure in the control link, the NCR-Fwd should cease amplifying-and-forwarding RF signals until BFR is completed.

NOTE: When the NCR-MT in the RRC_INACTIVE state determines degradation of the NCR-Fwd backhaul link beam based on the criteria by implementation, it should cease amplifying-and-forwarding RF signals and attempt to resume its RRC connection (with cause value ‘mo-Signalling’).

END OF CHANGES