3GPP TSG-RAN WG2 Meeting #123-bis	R2-2310719

Xiamen, China, October 9th-13th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4343 | rev | - | Current version: | 17.5.0 |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RedCap CFR configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  |  |  | Date: |  |  |  | 2023-09-28 |  |
| Category: | F |  |  |  |  |  |  |  |  | Release: |  |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |  |
| Reason for change: | In the current specification, the Redcap-specific initial BWP may not include CORESET#0 or CD-SSB. It is not clear for Redcap UEs that what is the bandwidth of CFR if sameAsSib1ConfiguredLocationAndBW is indicated, i.e., the initial BWP configured by initialDownlinkBWP or Redcap-specified initial BWP configured by initialDownlinkBWP-RedCap. And also, when network indicate value locationAndBandwidth that larger than initial BWP, it is also not clear which initial BWP is considered. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: | In 6.3.6, clarify that RedCap UE considers CFR configured via sameAsSib1ConfiguredLocationAndBW to be RedCap initial BWP bandwidth if it fully includes the CORESET#0 and CD-SSB. In 6.3.6, clarify that the CFR configured via locationAndBandwidth should be larger than RedCap initial BWP bandwidth if it fully includes the CORESET#0 and CD-SSB. Impact analysis Impacted 5G architecture options: NR SA, NE-DC, NR-DC Impacted functionality: MBS Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there is no inter-operability issue. 2. If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: | - It is not clear which initial BWP should be referred to when the Redcap UE determines the CFR, in case RedCap-specific initial BWP is configured. - It is not clear which initial BWP should be considered when using locationAndBandwidth to configure CFR for Redcap UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.6 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  | N |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  |  | x | Other core specifications |  |  |  |  | TS/TR ... CR .. |  |  |  |
| affected: |  |  |  |  | x | Test specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  |  | x | O&M Specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

### 6.3.6	MBS information elements

–	CFR-ConfigMCCH-MTCH

The IE CFR-ConfigMCCH-MTCH is used to configure the common frequency resource used for MCCH and MTCH reception.

CFR-ConfigMCCH-MTCH information element

-- ASN1START

-- TAG-CFR-CONFIGMCCH-MTCH-START

CFR-ConfigMCCH-MTCH-r17 ::= SEQUENCE {

locationAndBandwidthBroadcast-r17          LocationAndBandwidthBroadcast-r17  OPTIONAL,  -- Need S

pdsch-ConfigMCCH-r17                       PDSCH-ConfigBroadcast-r17          OPTIONAL,  -- Need S

commonControlResourceSetExt-r17            ControlResourceSet                 OPTIONAL   -- Cond NotSIB1CommonControlResource

}

LocationAndBandwidthBroadcast-r17 ::= CHOICE {

sameAsSib1ConfiguredLocationAndBW          NULL,

locationAndBandwidth                       INTEGER (0..37949)

}

-- TAG-CFR-CONFIGMCCH-MTCH-STOP

-- ASN1STOP

CFR-ConfigMCCH-MTCH field descriptions

commonControlResourceSetExt An additional common control resource set which may be configured and used for searchSpaceMCCH/searchSpaceMTCH or UE-specific search space in the BWP where searchSpaceMCCH is configured. It is contained in the bandwidth of the CFR for broadcast and larger than CORESET#0.

locationAndBandwidthBroadcast Indicates starting PRB and the number of PRBs of CFR used for MCCH and MTCH reception. Value sameAsSib1ConfiguredLocationAndBW means the CFR for broadcast has the same location and size as the locationAndBandwidth for initial BWP (for RedCap UEs initialDownlinkBWP-RedCap if configured and it includes CD-SSB and the entire CORESET#0) configured in SIB1. Value locationAndBandwidth is used to configure CFR with bandwidth that is larger than and fully contains the bandwidth for the initial DL BWP configured (for RedCap UEs initialDownlinkBWP-RedCap if configured and it includes CD-SSB and the entire CORESET#0) in SIB1 and CORESET#0. If the field is absent, the CFR for broadcast has the same location and size as CORESET#0.

pdsch-ConfigMCCH Indicates PDSCH parameters used for MCCH transmission. If the field is absent, PDSCH parameters used for MCCH are the same as those of PDSCH configuration provided in initialDownlinkBWP in SIB1.

| Conditional Presence | Explanation |
| --- | --- |
| NotSIB1CommonControlResource | The field is optional present in case commonControlResourceSet is not configured in SIB1, Need R, otherwise it is absent. |

END OF CHANGE