3GPP TSG-RAN WG2 Meeting #123	 R2-2310739

Xiamen, China, October 9-13, 2023

Source:	KDDI Corporation

Title:	Discussions on TA acquisition and indication for RACH-less LTM

Tdoc Type:	Discussion

Agenda Item:	7.4.2	L1L2 Triggered Mobility

Document for:  Discussion

Release:       Rel-18

1	Introduction

In this contribution, we will discuss and share our views on TA acquisition and indication for RACH-less LTM.

2	Discussion

RAN2#119bis-e (Online, August 2022), agreement[1]

L1L2 based mobility supports the following CA scenarios: - PCell change without SCell change - PCell change with SCell change Support NR-DC scenario in L1L2 based mobility, at least for the PSCell change without MN involvement case, i.e. intra-SN.

RAN2#122 (Incheon, Korea, May 2023), agreement[2]

- For early TA acquisition for candidate Cells For PDCCH ordered early TA acquisition without RAR, there is no need for UE to maintain the TA timer for candidate cell (i.e. it is NW implementation to determine the TA validity), TA is given in the cell switch MAC CE (when available in the network). RAN2 doesn’t see a need for a solution with RAR in for Rel-18. Observation: Without RAR (without UE maintaining TA), the UE will need to do RACH for link recovery and/or conditional (if supported), which is acceptable in Rel-18 The UE determines to trigger RACH-less cell switch in MAC layer, if the LTM cell switch MAC CE provides the TA value (no RAR is assumed). We send LS to R1

RAN2#123 (Toulouse, France, August 2023) agreement[3]

The UE will do RACH-less when: - - TA value is provided in the cell switch MAC CE (already agreed, TA=0 is assumed to be covered by this) - - When the UE shall apply the same TA value as the source (already agreed) FFS how the UE knows this.

In RAN2#122 (Korea 2023), RAN2 agreed that NW obtains TA through PDCCH ordered Rach procedure. PDCCH ordered Rach is a procedure to acquire the TA of candidate cell, but there are no clear discussions whether the TA of the candidate cell which NW obtains is only PCell or the cell that also includes SCell. Therefore, We would like to confirm that PDCCH ordered Rach procedure aims to obtain the TA of only Candidate PCell in Rel-18. Furthermore, in this case, we mention the MCG scenario but it is also applied to the scenarios such as PSCell change without MN involvement case in intra-SN.For that reason, we also would like to confirm that PDCCH ordered Rach procedure aims to obtain the TA of only Candidate PSCell in Rel-18.

Proposal1a: RAN2 agree that for MCG LTM case, NW obtains the TA of only Candidate PCell by PDCCH ordered Rach procedure before the Cell switch command in Rel-18.

Proposal1b: RAN2 agree that for SCG LTM case, NW obtains the TA of only Candidate PSCell by PDCCH ordered Rach procedure before the Cell switch command in Rel-18.

In addition, RAN2 has agreed to indicate the UE in the Cell switch command including the candidate cell TA but There are no clear discussions on how many TAs to be included in the Cell switch command. For example, if the NW acquires the TA of Candidate PCell and SCell using PDCCH ordered Rach, and then the NW instructs multiple TAs (one for PCell and others for SCell) in the Cell switch command. With indicating multiple TAs, after LTM execution, the TA acquisition procedure of SCell after Cell switch completion can be omitted, and It becomes possible to lead to low latency communication. However, it will be difficult to complete the work for multiple TAs in the remaining Rel-18 meetings. Even though it is effective to newly instruct multiple TAs using the Cell switch command. Therefore, in Rel-18, we would like to propose to indicate only the TA of Candidate PCell using Cell switch command.

Furthermore, since only the TA of PCell is indicated in the Cell switch command, it is necessary to obtain other TAs after LTM execution. In addition, the same scenarios about TA indication and other TAs acquisition procedure also applie to PSCell change scenarios such as instra-SN, so I would like to make the same proposals.

Proposal2a: RAN2 agree that for  MCG LTM case, Cell switch command indicates only the TA of candidate PCell in this release, and optimization of multiple TAs indications is postponed to the next release.

Proposal2b: RAN2 agree that for SCG LTM case, Cell switch command indicates only the TA of candidate PSCell in this release, and optimization of multiple TAs indications is postponed to the next release.

Proposal3a: RAN2 agree that for MCG LTM case, TA other than candidate PCell are acquired using the conventional TA acquisition procedure on the candidate cell side after LTM exection.

Proposal3b: RAN2 agree that for SCG LTM case,TA other than candidate PSCell are acquired using the conventional TA acquisition procedure on the candidate cell side after LTM exection.

Figure : Rach less LTM in MCG scenario including SCell

3	Conclusion

We make the following observations and proposals:

Proposal1a: RAN2 agree that for MCG LTM case, NW obtains the TA of only Candidate PCell by PDCCH ordered Rach procedure before the Cell switch command in Rel-18.

Proposal1b: RAN2 agree that for SCG LTM case, NW obtains the TA of only Candidate PSCell by PDCCH ordered Rach procedure before the Cell switch command in Rel-18.

Proposal2a: RAN2 agree that for MCG LTM case, Cell switch command indicates only the TA of candidate PCell in this release, and optimization of multiple TAs indications is postponed to the next release.

Proposal2b: RAN2 agree that for SCG LTM case, Cell switch command indicates only the TA of candidate PSCell in this release, and optimization of multiple TAs indications is postponed to the next release.

Proposal3a: RAN2 agree that for MCG LTM case, TA other than candidate PCell are acquired using the conventional TA acquisition procedure on the candidate cell side after LTM exection.

Proposal3b: RAN2 agree that for SCG LTM case,TA other than candidate PSCell are acquired using the conventional TA acquisition procedure on the candidate cell side after LTM exection.

3	Reference

[1]　 RAN2 Chair’s Notes      3GPP TSG RAN WG2 #119bis-e Online, August 2022

[2]  RAN2 Chair’s Notes      3GPP TSG RAN WG2 #122 Incheon, Korea, May 2023

[3]　RAN2 Chair’s Notes      3GPP TSG RAN WG2 #123 Toulouse, France, August 2023