3GPP TSG-RAN WG2 #123bis	R2-2310754

Xiamen, China, 9th - 13th October 2023

Agenda Item:	7.14.5 UE capabilities and other topics

Source:	Ericsson

Title:	Measurement status issue in conditional handovers and UE capabilities for QoE

Document for:	Discussion

# 1	Introduction

In release 18, the WID [1] on Enhancement on NR QoE management and optimizations for diverse services was agreed. In this contribution, an outstanding issue for QoE is discussed as well as UE capabilities.

# 2	Discussion

## 2.1	Outstanding issue related to session start/stop indication

In release 17, a session start and stop indication was defined in RRC. The UE sends a session start indication to the network when a QoE measurement session in the application layer starts and a session stop indication when a QoE measurement session in the application layer stops. The network uses the session status information for alignment with MDT and for area handling. At handover, the session status information is sent to the target node, so that the target node gets information of the current status of the session and can take over the responsibility of the measurements.

At conditional handover, the Handover Request message which contains the session status indication is sent prior to the actual execution of the handover. The target node prepares the target configuration for the UE in advance, but the waits with UE applying the target configuration until a certain event is fulfilled. There is normally a time in-between when the target configuration is prepared by the target node and when the configuration is applied by the UE. During this time the session status in the UE may change, and if it changes, the target node will have incorrect information regarding the session status when the conditional handover is executed. Incorrect measurement status information in the target node may lead to measurements being released when they should not and vice versa. It may also lead to incorrect alignment with MDT measurements. It is therefore proposed that RAN2 discusses how to resolve this issue.

The issue may occur also at regular handover even if the likelihood is smaller. It has been seen with UEAssistanceInformation, that if the UE transmits the message too close to the handover, the target node will not get the latest information, as there is a short time between the preparation of the handover and the execution of the handover. In the UEAssistanceInformation procedure, the problem is solved by the UE transmitting the message again to the target node, if a handover occurred shortly after the UE transmitted the UEAssistanceInformation message.

In existing procedures, a target node may end up having incorrect QoE measurement status information at conditional handover.

Discuss how to resolve the issue that a target node may not have correct measurement status information when a conditional handover is executed.

## 2.2	UE capabilities

One objective of the work item is to support QoE measurements in RRC_IDLE/RRC_INACTIVE.

Address this FFS:

1: Introduce an UE capability indicating whether UE can perform MBS QoE in RRC_IDLE and RRC_INACTIVE. FFS whether the same capability can be used for MBS QoE in RRC_CONNECTED.

The third objective of the work item is support for QoE measurements in NR-DC. This includes performing measurements according to a configuration received from a SN and support for transmission of reports related to the SN, possibly via the MN. As the reports can be transmitted either directly to the SN using SRB5, or via the MN, the UE capabilities could reflect these options. It is therefore proposed to have a separate UE capability for the support of SRB5. Hence, one capability can be defined for the support of QoE in NR-DC and one capability can be defined for the support of SRB5. If the UE only indicates support for QoE in NR-DC, it only supports transmission of QoE reports related to the SN via the MN. If the UE indicates support of both capabilities, all different transmission options for NR-DC are supported.

Define a new UE capability bit for support of QoE measurements in NR-DC.

There are more objectives in the work item, such as support of triggering events for RVQoE and support of IRAT handover. As these discussions are still more open, it is proposed to wait with the capability discussion until the scope is clearer.

# Conclusion

RAN2 is kindly asked to discuss the following observations and proposals:

Observation 1	In existing procedures, a target node may end up having incorrect QoE measurement status information at conditional handover.

Proposal 1	Discuss how to resolve the issue that a target node may not have correct measurement status information when a conditional handover is executed.

Proposal 2	Define a new UE capability bit for support of QoE measurements in NR-DC.

# References

RP-221803, Enhancement on NR QoE management and optimizations for diverse services, China Unicom

R2-2307967, QoE measurements in RRC_INACTIVE and RRC_IDLE, Ericsson