3GPP TSG-RAN WG2 Meeting #123bis	                             R2-2310767

Xiamen, China, October 9 - 13, 2023

Agenda Item: 	7.5.4.3

Source: 	Sony

Title: 	Configured Grant enhancements for XR

Document for: 	Discussion / Decision

# Introduction

RAN1 has made a significant progress for specifying multiple CG PUSCH transmission occasions in a period of a single CG configuration as well as dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE, as described in the WID [1-2].

In this contribution, we discuss some open issues for multiple CG PUSCH transmission occasions in a period of a single CG configuration in RAN2.

# Discussion

Configuration for multiple CG PUSCH transmission occasions: From RRC configuration perspective, the configuration for multiple CG PUSCH transmission occasions in a period of a single CG configuration could be done in such a way that there is a main CG occasion and one or more supplementary occasions within a period of a single CG configuration. The number of supplementary occasions should be semi-static and RRC configured regardless of whether they are activated (used) or not activated (unused) and all could have an equal offset/gap between them, starting from the main occasion, to simplify the specification. The offset/gap could be zero (i.e., contiguous).

However, it seems RAN1 agreed [3-4] that the number of supplementary occasions to be contiguous based on the following agreement (the part highlighted with yellow colour):

RAN1#113 Agreement:

For time domain resource allocation for multi-PUSCH CGs, support

- For TDRA determination (based on NR-U framework)

  - For Type-1, follow the rules for DCI format 0_0 on UE specific search space, as defined in Clause 6.1.2.1.1 of TS 38.214.

    - Note: To determine the configuration of TDRA, PUSCH repetition type A is assumed according to description in 6.1.2.3 in 38.214 for Type-1.

      - It is still an open issue whether repetition is supported. If it is decided repetition is not supported, it implies the corresponding repetition factor for is one.

  - For Type-2, the TDRA table is determined by the TDRA table associated with activation DCI, as defined in Clause 6.1.2.1 of TS 38.214.

    - Note: The DCI format for activation DCI with pusch-RepTypeA is applicable.

      - It is still an open issue whether repetition is supported. If it is decided repetition is not supported, it implies the corresponding repetition factor for is one.

- N is configured by higher layers

- A single SLIV is determined from TDRA.

  - The SLIV used for 1st PUSCH per CG period.

- PUSCH is used in each of N consecutive slots per CG period

- Note: N is configured independently from cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively. N configuration is independent from cgRetransmissionTimer configuration.

- To determine corresponding slots for CG PUSCHs in a period of a multi-PUSCH CG configuration:

  - For the first PUSCH in the period, follow the legacy procedures.

  - For remaining PUSCHs in the period

    - ForType-1 and Type-2, reuse the corresponding procedures for NR-U by applying the RRC parameters N and M, instead of cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively.

RAN1#114 Agreement:

For a multi-PUSCH CG configuration, the range value of the higher layer parameter indicating number of consecutive slots (N in previous agreements) is:

- Max value=16 or 32

  - Up to UE capability

- Min value=2

RAN1#114 Agreement:

For a CG configuration with UTO-UCI indication enabled:

- For the range value for the RRC parameter Nu (Nu is the size of bit-map): (3, …, 8)

From the above RAN1 agreements, it stated that to determine corresponding slots for CG PUSCHs in a period of a multi-PUSCH CG configuration, the first PUSCH in the period follows the legacy procedures and for the remaining PUSCH occasions (i.e., the number of consecutive supplementary occasions) in the period reuse the corresponding procedures for NR-U by applying the RRC parameters N instead of cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively.

Proposal 1: RAN2 to confirm that the configuration of the number of consecutive supplementary occasions within a period of a single CG configuration is defined by the RRC configured parameter N, whilst the first PUSCH occasion in the period follows the legacy procedures/configuration, as agreed by RAN1.

Interaction of MAC and PHY for UTO-UCI indication: MAC determines whether there is data for one or more CG occasions in a CG configuration. Obviously if there is no data for some of the occasions then MAC must deliver some information to the physical layer (PHY). Then the PHY can include the information in the UTO-UCI indication so that gNB is aware of the unused occasions. However, we do not think the interaction of MAC and PHY for UTO-UCI indication needs to be specified, it can be left for UE internal implementation.

Proposal 2: The interaction of MAC and PHY for the indication of unused occasions (UTO-UCI) does not need to be specified, it can be left for UE internal implementation.

Timeline for indicating unused CG occasions: The benefits of indicating unused CG occasions is to reuse the resources for other UEs at the gNB, as well as reducing the interference. However, gNB can only schedule these resources to other UEs if it gets the information/indication of unused CG occasions early from the UE. Therefore, in order to help gNB scheduler, we think RAN2/RAN1 should specify a timeline for a UE to indicate unused CG occasions in a period of a single CG configuration (i.e., minimum indication time for unused CG occasion).

Based on RAN1 agreement, if an occasion is indicated as unused, it cannot be changed later to used occasion, the reason is that gNB may have already allocated for other UEs. Therefore, it is reasonable to specify a timeline, i.e., the latest time that the UE can inform the unused CG occasions to the gNB. In practice, this timeline must be set based on scheduler implementation, i.e., not too long/late nor too short/early. Note that for some reasons, if the UE cannot decide at this point whether the occasion is “unused”, it can always set to “used”. Then the UE can update and indicate “unused” at some point later, although a gNB may not be able to assign the resource to other UEs.

Proposal 3: Specify a timeline for a UE to indicate unused CG occasions in a period of a single CG configuration.

No UL data available for a CG occasion: RAN1 made agreement that UTO-UCI will be defined to provide information about unused CG PUSCH transmission occasions in a single CG configuration, and this UCI will be carried in every transmitted CG PUSCH (i.e., UCI + data). However, in some cases, data may not always be available for one or more CG PUSCH(s) where a UE has earlier indicated to be “used” occasions, for example if data is discarded recently or due to timeline (as described above). In that case, some companies proposed that the legacy UL skipping can be applied. But it may be too late for a UE to change its mind from used to unused, and in any case gNB cannot make use of the UL resource anymore. In addition, the gNB will blindly decode PUSCH if it is present or not. So, it is preferable that the UE still indicates the UTO-UCI despite transmitting nothing in the data part, i.e., PUSCH with-only UTO-UCI should be transmitted.

Proposal 4: If a UE has earlier indicated for an occasion to be used, but there is no data/PUSCH transmission, then the UE should still indicate the UTO-UCI despite transmitting nothing in the data part, i.e., PUSCH with-only UTO-UCI should be transmitted.

# Conclusion

In this contribution, we have discussed some open issues for multiple CG PUSCH transmission occasions in a period of a single CG configuration in RAN2. We have the following proposals:

Proposal 1: RAN2 to confirm that the configuration of the number of consecutive supplementary occasions within a period of a single CG configuration is defined by the RRC configured parameter N, whilst the first PUSCH occasion in the period follows the legacy procedures/configuration, as agreed by RAN1.

Proposal 2: The interaction of MAC and PHY for the indication of unused occasions (UTO-UCI) does not need to be specified, it can be left for UE internal implementation.

Proposal 3: Specify a timeline for a UE to indicate unused CG occasions in a period of a single CG configuration.

Proposal 4: If a UE has earlier indicated for an occasion to be used, but there is no data/PUSCH transmission, then the UE should still indicate the UTO-UCI despite transmitting nothing in the data part, i.e., PUSCH with-only UTO-UCI should be transmitted.

# References

- RP-223502, “New WID on XR Enhancements for NR”, RANP #98e, December 2022

- RP-223187, “Study on XR enhancements for NR (TR 38.835 V1.0.0)”, December 2022

- Chair's notes RAN1#113 eom

- Chair's notes RAN1#114 eom2