3GPP TSG-RAN WG2 Meeting #123bis  	                            R2-2310775

Xiamen, China, October 9-13th, 2023

Agenda Item:	7.16.2.2

Source: 	Sony

Title:	Summary of impacts due to data collection

Document for:	Discussion

# Introduction

Proposals from [POST 123] email discussion #059 on data collection are:

Proposal 1 For training of NW-side models, both gNB- and OAM-centric data collection are considered in the study. Proposal 2 For training of NW-side models, the gNB-centric data collection implies that the gNB configures the UE to initiate the data collection procedure and the collected data are terminated at the gNB. To further study the details of the data collection configuration. Proposal 3 Related to gNB-centric data collection for NW-side model training, RAN2 studies the potential impact on L3 signalling for the reporting of collected data, taking into account RAN1 further inputs/progress. Proposal 4 Related to gNB-centric data collection for NW-side model training, the following principles can be considered for the L3 signalling reporting framework, if used: a. The L3 signalling reporting framework for NW-side model training should allow the UE to store sets of measurements and then report them to the gNB in multiple RRC messages (e.g. similar to the logged MDT). b. The L3 signalling reporting framework for NW-side model training should allow the UE to store multiple measurements taken at different points in time and report them in a single RRC report. c. The L3 signalling reporting framework for NW-side model training implies that the UE may be configured to report measurements periodically or upon fulfilling certain events. d. The UE memory/processing power/energy consumption/signalling overhead should be taken into account. Note: The above principles, for the L3 signalling reporting framework, can be revised depending on RAN1 progress/requirements Proposal 5 Related to gNB-centric data collection for NW-side model training, further principles for the L3 signalling reporting framework may be considered in RAN2, also taking into account RAN1 progress/requirements. Proposal 6 For training of NW-side models, an OAM-centric data collection implies that the OAM provides the configuration (via the gNB) needed for the UE to initiate the data collection procedure and the collected data are terminated at the OAM entity. MDT framework can be considered as baseline. Proposal 7 Related to OAM-centric data collection for NW-side model training, RAN2 studies the potential impact on the immediate MDT, taking into account RAN1 further inputs/progress. Proposal 8 Related to OAM-centric data collection for NW-side model training, RAN2 to further discuss motivations of the suitability of the logged MDT framework or other frameworks. Proposal 9 Related to OAM-centric data collection for NW-side model training, the following principles can be considered for the immediate MDT framework, if used: a. The Immediate MDT framework for NW-side model training should allow the UE to store sets of measurements and then report them in multiple RRC messages (e.g. similar to the logged MDT). b. The Immediate MDT framework for NW-side model training should allow the UE to store multiple measurements taken at different points in time and report them in a single RRC report. c. The Immediate MDT framework for NW-side model training should allow the network to configure the UE to report measurements periodically or upon fulfilling certain events. d. The UE memory/processing power/energy consumption/signalling overhead should be taken into account. Note: The above principles, for the immediate MDT framework, can be revised depending on RAN1 progress/requirements Proposal 10 Related to OAM-centric data collection for NW-side model training, further principles for the immediate MDT may be considered in RAN2, also taking into account RAN1 progress/requirements. Proposal 11 Related to OAM-centric data collection for NW-side model training, the principles of the logged MDT framework will be discussed if its suitability is assessed. Proposal 12 Related to performance monitoring of NW-side models, impact (if any) in RAN2 protocols needs to be further evaluated, depending on RAN1 progress/inputs. Proposal 13 If the UE-side OTT server is responsible for the UE-side model training, the way the OTT server collects data should not be studied in RAN2. Proposal 14 Related to UE-side model training, RAN2 to study impacts in existing RAN2 protocols to enable RAN-awareness of UE-side model training (e.g. to initiate the data collection for UE-side model training, to provide the necessary configuration for data collection for the UE-side training, etc.), taking into account RAN1 progress/requirements. Proposal 15 Related to performance monitoring of UE-side models, impact (if any) in RAN2 protocols needs to be further evaluated, depending on RAN1 progress/inputs.

In this paper we share our views on the summary of RAN2 impacts due to data collection.

# Discussion

The email discussion focussed on gNB centric data collection for NW side, OAM centric data collection for NW side and UE centric data collection for model training, inference and monitoring.

gNB centric data collection for NW side (model training, inference, and monitoring)

The email discussion focused on enhancing L3 signalling and some companies proposed to specifically enhance RRM measurement framework. Some other companies expressed the need for L1 measurements for inference and in the past UE capability signalling and its change due to applicability conditions was discussed. We think the current use cases are mainly for RRC connected mode UEs.

Observation 1: Summary of possible RAN2 impacts for gNB centric data collection for NW side for model training/inference and monitoring:

- Enhance L3 measurement framework.

- Enhance UE capability signalling for e.g. change in applicability conditions leading to change in capability

- Enhance L1 measurement framework.

- Focus on RRC Connected state.

For OAM centric data collection for NW side (model training, inference, and monitoring)

Immediate MDT should be the starting point where data can be logged instead of one shot reporting. Immediate MDT is suitable for model monitoring as latency is an issue with logged MDT.

Observation 2: RAN2 impacts are foreseen on enhancing immediate MDT.

If L3 measurements are enhanced for gNB centric data collection and immediate MDT is enhanced for OAM centric data collection. Then except for the trigger, changes for RAN2 might be overlapping with slight exceptions like if one RRC message is used or multiple RRC messages are used.

Observation 3: RAN2 scope for gNB centric and OAM centric might have overlaps for RAN2.

UE side OTT server

Following options along with their main characteristics were discussed in the past:

- EVEX

  - No RAN involvement in data collection

  - Non standardised solution

  - RRC connected state

  - User consent available

- MDT

  - RAN involvement in data collection

  - Standardised data collection

  - RRC connected state

  - User consent available

- UAI

  - RAN involvement in data collection

  - Standardised data collection

  - RRC connected state

  - No user consent yet

However, proposal 13 above proposes RAN2 should not be involved for data collection. In our understanding, data to be collected will be proprietary but a generic mechanism to transfer the data should be discussed.

Observation 4:  data transfer from UE to OTT server for model training may have RAN2 impacts including the need for RAN awareness.

# Conclusion

We propose RAN2 to discuss and agree on the following observations and proposal:

Observation 1: Summary of possible RAN2 impacts for gNB centric data collection for NW side for model training/inference and monitoring:

- Enhance L3 measurement framework.

- Enhance UE capability signalling for e.g. change in applicability conditions leading to change in capability

- Enhance L1 measurement framework.

- Focus on RRC Connected state.

Observation 2: RAN2 impacts are foreseen on enhancing immediate MDT.

Observation 3: RAN2 scope for gNB centric and OAM centric might have overlaps for RAN2.

Observation 4:  data transfer from UE to OTT server for model training may have RAN2 impacts including the need for RAN awareness.

# References

[1]	RP-213599, Qualcomm (Moderator), “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” RAN #94e, Dec. 2021

[2] R2-230xxxx: Summary of [POST 123] email discussion #059

[4] SP-230394: Alignment of activities on UE data collection reporting and event exposure, SA